3GPP TSG-RAN WG4 Meeting #94-e                                                             R4-2002049

Online, 24th Feb 2020 - 6th Mar 2020

Source:	Google Inc.

Title:	TP for TR 37.716-11-11: DC_(n)48 and DC_48_n48

Agenda item:			9.3.2

Document for:	Approval

# 1. Introduction

In RAN#86 meeting, the revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band +1 NR band) has been approved in [1]. This contribution is a text proposal for TR 37.716-11-11 to include DC_(n)48 and DC_48_n48 combinations with only supporting single switch UL mode.

# 2. Reference

- RP-192742, Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band), CHTTL.

# 3. Text Proposal

<Start of Text Proposal>

# 1	Scope

The present document is a technical report for Dual Connectivity (EN-DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (EN-DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL)for the Rel-16 band combinations in Table 1-1 and Table 1-2. The co-existence analysis and RF front end requirements such as Delta RIB,C and TIB,C are described based on the band combination basis since such information have no difference between the EN-DC configulations consisting with the same E-UTRA band and the same NR band.

Table 1-1: Release 16 EN-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) within FR1

| DC combination | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n3A | DC_1A_n3A |
| DC_1C_n3A | DC_1C_n3A |
| DC_1A_n5A | DC_1A_n5A |
| DC_1A_n7A | DC_1A_n7A |
| DC_1A_n8A | DC_1A_n8A |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A |
| DC_1A_n41A | DC_1A_n41A |
| DC_1A_n50A | DC_1A_n50A |
| DC_1A_n77(2A) | DC_1A_n77A |
| DC_2A-2A_n5A | DC_2A_n5A |
| DC_2A_n7A DC_2A_n7(2A) | DC_2A_n7A |
| DC_2A_n38A | DC_2A_n38A |
| DC_2A_n41A | DC_2A_n41A |
| DC_2C_n41A | DC_2A_n41A |
| DC_2A_n48A | DC_2A_n48A |
| DC_3A_n1A | DC_3A_n1A |
| DC_3C_n1A | DC_3A_n1A DC_3C_n1A |
| DC_3A-3A_n1A | DC_3A_n1A |
| DC_3A_n3A | DC_3A_n3A |
| DC_3A_n5A | DC_3A_n5A |
| DC_3C_n5A | DC_3A_n5A DC_3C_n5A |
| DC_3A_n20A | DC_3A_n20A |
| DC_3A_n34A | DC_3A_n34A |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A |
| DC_3A_n41A | DC_3A_n41A |
| DC_3A_n50A | DC_3A_n50A |
| DC_3A_n77(2A) | DC_3A_n77A |
| DC_3A-3A_n77A | DC_3A_n77A |
| DC_3A-3A_n78A | DC_3A_n78A |
| DC_4A_n38A | DC_4A_n38A |
| DC_4A_n41A | DC_4A_n41A |
| DC_4A_n78A | DC_4A_n78A |
| DC_4A_n78(2A) | DC_4A_n78A |
| DC_5A_n2A | DC_5A_n2A |
| DC_5A_n7A DC_5A_n7(2A) | DC_5A_n7A |
| DC_5A_n48A | DC_5A_n48A |
| DC_5A_n71A | DC_5A_n71A |
| DC_5A_n79A | DC_5A_n79A |
| DC_7A_n1A | DC_7A_n1A |
| DC_7C_n1A | DC_7A_n1A DC_7C_n1A |
| DC_7A-7A_n1A | DC_7A_n1A |
| DC_7A_n3A | DC_7A_n3A |
| DC_7A_n5A | DC_7A_n5A |
| DC_7A-7A_n5A | DC_7A_n5A |
| DC_7C_n5A | DC_7A_n5A DC_7C_n5A |
| DC_7A_n7A | DC_7A_n7A1 |
| DC_7A_n66A | DC_7A_n66A |
| DC_7A-7A_n66A | DC_7A_n66A |
| DC_7C_n66A | DC_7A_n66A |
| DC_7A_n71A | DC_7A_n71A |
| DC_7A_n77A | DC_7A_n77A |
| DC_7A-7A_n77A | DC_7A_n77A |
| DC_8A_n1A | DC_8A_n1A |
| DC_8A_n3A | DC_8A_n3A |
| DC_8A_n28A | DC_8A_n28A |
| DC_8A_n34A | DC_8A_n34A |
| DC_8A_n39A | DC_8A_n39A |
| DC_8A_n41A | DC_8A_n41A |
| DC_8A_n41C | DC_8A_n41A DC_8A_n41C |
| DC_8A_n41(2A) | DC_8A_n41A |
| DC_12A_n2A | DC_12A_n2A |
| DC_12A_n7A DC_12A_n7(2A) | DC_12A_n7A |
| DC_12A_n71A | DC_12A_n71A |
| DC_12A_n78A DC_12A_n78(2A) | DC_12A_n78A |
| DC_13A_n5A | DC_13A_n5A |
| DC_13A_n48A | DC_13A_n48A |
| DC_13A_n66A | DC_13A_n66A |
| DC_13A_n71A | DC_13A_n71A |
| DC_18A_n3A | DC_18A_n3A |
| DC_20A_n1A | DC_20A_n1A |
| DC_20A_n3A | DC_20A_n3A |
| DC_20A_n7A | DC_20A_n7A |
| DC_20A_n38A | DC_20A_n38A |
| DC_20A_n41A | DC_20A_n41A |
| DC_20A_n50A | DC_20A_n50A |
| DC_25A-25A_n41A | DC_25A_n41A |
| DC_26A_n25A | DC_26A_n25A |
| DC_28A_n3A | DC_28A_n3A |
| DC_28A_n5A1 | DC_28A_n5A |
| DC_28A_n7A DC_28A_n7B | DC_28A_n7A DC_28A_n7B |
| DC_28A_n8A | DC_28A_n8A |
| DC_28A_n41A | DC_28A_n41A |
| DC_28A_n50A | DC_28A_n50 |
| DC_30A_n2A | DC_30A_n2A |
| DC_39A_n40A | DC_39A_n40A |
| DC_39A_n41A | DC_39A_n41A |
| DC_40A_n1A | DC_40A_n1A |
| DC_40A_n41A | DC_40A_n41A |
| DC_40A_n78A | DC_40A_n78A |
| DC_40C_n78A | DC_40C_n78A |
| DC_40A_n79A | DC_40A_n79A |
| DC_41A_n77(2A) | DC_41A_n77A |
| DC_41C_n77(2A) | DC_41A_n77A DC_41C_n77A |
| DC_(n)48AA | DC_(n)48AA1 |
| DC_(n)48CA | DC_(n)48AA1 DC_48A_n48A1 |
| DC_(n)48DA | DC_(n)48AA1 DC_48A_n48A1 |
| DC_48A_n48A | DC_48A_n48A1 |
| DC_48A_(n)48AA | DC_(n)48AA1 DC_48A_n48A1 |
| DC_48A-48A_n48A | DC_48A_n48A1 |
| DC_48C_n48A | DC_48A_n48A1 |
| DC_48D_n48A | DC_48A_n48A1 |
| DC_66A_n2A | DC_66A_n2A |
| DC_66A-66A_n5A | DC_66A_n5A |
| DC_66A-66A-66A_n5A | DC_66A_n5A |
| DC_66A_n7A DC_66A-66A_n7A DC_66A-66A_n7(2A) | DC_66A_n7A |
| DC_66A_n25A | DC_66A_n25A |
| DC_66A_n41A | DC_66A_n41A |
| DC_66A_n48A | DC_66A_n48A |
| DC_66A_n66A | DC_66A_n66A1 |
| DC_71A_n5A | DC_71A_n5A |
| NOTE 1: Only single switched UL is supported. NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the D |  |

<Unchanged text omitted>

## 6.3.x	DC(n)_48

#### 6.3.x.1	Channel bandwidths per operating band for DC

Table 6.3.x.1-1: Band combinations for intra-band contiguous EN-DC

| EN-DC Band Uplink Combination | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC(n)_48 | 48 | n48 | Yes |

Table 6.3.x.1-2: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for LTE carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for LTE carrier (MHz) |  |  |
| DC_(n)48AA2 | DC_(n)48AA1 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)48CA2 | DC_(n)48AA1 DC_48A_n48A1 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48DA2 | DC_(n)48AA1 DC_48A_n48A1 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| NOTE1: Only single switched UL is supported NOTE2: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |

#### 6.3.x.2	Configuration for DC

Table 6.3.x.2-1:  Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_(n)48AA2 | DC_(n)48AA1 | 48A | n48A |
| DC_(n)48CA2 | DC_(n)48AA1 DC_48A_n48A1 | CA_48C | n48A |
| DC_(n)48DA2 | DC_(n)48AA1 DC_48A_n48A1 | CA_48D | n48A |
| NOTE1: Only single switched UL is supported NOTE2: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |

#### 6.3.x.3	Maximum output power for DC

Table 6.3.x.3-1: Maximum output power for  Intra-band contiguous EN-DC

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)48AA1 |  |  | 23 | +2/-3 |
| NOTE 1: Only single switched UL is supported |  |  |  |  |

#### 6.3.x.4	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for these combinations. No spurious emission band UE co-existence requirement is needed

#### 6.3.x.5	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for these combinations. No MSD requirement is needed

#### 6.3.x.6	∆RIBNC values

Note that the minimum requirements for these combinations only apply for non-simultaneous Tx/Rx between all carriers. No ∆RIBNC values is needed

#### 6.3.x.7	MPR, AMPR requirements

Note that only Single Tx Switched UL mode is supported for these combinations. No MPR or A-MPR requirement is needed

## 6.4.x	DC_48_n48

#### 6.4.x.1	Channel bandwidths per operating band for DC

Table 6.4.x.1-1: Band combinations for intra-band non-contiguous EN-DC

| EN-DC Band Uplink Combination | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_48_n48 | 48 | n48 | Yes |

Table 6.4.x.1-2: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum ggregate bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for LTE carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for LTE carrier (MHz) |  |  |
| DC_48A_n48A2 | DC_48A_n48A1 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_48A_(n)48AA2 | DC_(n)48AA1 DC_48A_n48A1 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| DC_48A-48A_n48A2 | DC_48A_n48A1 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| DC_48C_n48A2 | DC_48A_n48A1 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_48D_n48A2 | DC_48A_n48A1 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| NOTE1: Only single switched UL is supported NOTE2: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |  |

#### 6.4.x.2	Configuration for DC

Table 6.4.x.2-1:  Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_48A_n48A2 | DC_48A_n48A1 | 48A | n48A |
| DC_48A_(n)48AA2 | DC_(n)48AA1 DC_48A_n48A1 | CA_48A-48A | n48A |
| DC_48A-48A_n48A2 | DC_48A_n48A1 | CA_48A-48A | n48A |
| DC_48C_n48A2 | DC_48A_n48A1 | CA_48C | n48A |
| DC_48D_n48A2 | DC_48A_n48A1 | CA_48D | n48A |
| NOTE1: Only single switched UL is supported NOTE2: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |

#### 6.4.x.3	Maximum output power for DC

Table 6.4.x.3-1: Maximum output power for  Intra-band non-contiguous EN-DC

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_48A_n48A1 |  |  | 23 | +2/-3 |
| NOTE 1: Only single switched UL is supported |  |  |  |  |

#### 6.4.x.4	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for these combinations. No spurious emission band UE co-existence requirement is needed.

#### 6.4.x.5	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for these combinations. No MSD requirement is needed.

#### 6.4.x.6	∆RIBNC values

Note that the minimum requirements for these combinations only apply for non-simultaneous Tx/Rx between all carriers. No ∆RIBNC values is needed.

#### 6.4.x.7	MPR, AMPR requirements

Note that only Single Tx Switched UL mode is supported for these combinations. No MPR or A-MPR requirement is needed.

<End of Text Proposal>