3GPP TSG-RAN WG4 Meeting #94	R4-2002122

E-meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	8.11.2.3

Source: 	Qualcomm Incorporated

Title: 	DL/UL beam indication with reduced latency and overhead

Document for:	Discussion

# Introduction

During the last meeting [1], RAN4 agreed to the following requirement:

Agreements - Simultaneous TCI States Activation/Selection across Multiple CCs/BWPs: - No RAN4 impact on Rel-15 active TCI state switching delay requirement has been identified due to newly introduced simultaneous TCI states activation /selection across multiple CCs/BWPs. - FFS new Rel-16 requirement on simultaneous TCI states activation/selection across multiple CCs/BWPs. - MAC-CE based spatial relation update for aperiodic SRS: - FFS RAN4 RRM requirement impact is identified from the mechanism of spatial relation updates for AP-SRS via MAC-CE

In this contribution, we discuss the above two unresolved topics.

# Simultaneous TCI states activation/selection across multiple CCs/BWPs

RAN1 made several agreements regarding simultaneous TCI states activation/selection across multiple CCs/BWPs.

Agreement made in RAN1#98 For latency/overhead reduction across multiple CCs/BWPs, support single MAC-CE to activate at least the same set of PDSCH TCI state IDs for multiple CCs/BWPs - Example 1: Reuse Rel-15 MAC-CE to activate same set of TCI state IDs for all active BWPs in same band or cell group(s) on FR2 - Support of this mode can be indicated by UE capability - To operate in this mode, UE may expect the same QCL-TypeD RS is configured for same TCI state ID for all BWPs in each band or cell group(s) - For activation MAC-CE received on any active BWP in a band or cell group(s), indicated activated TCI state IDs will be applied to every active BWP in that band or cell group(s) - Example 2: Reuse Rel-15 MAC-CE to activate one set of TCI state IDs (including both QCL Type-A and Type-D RSs) for an active BWP of the CC indicated by the MAC-CE to be applied to all active BWPs in same band or cell group(s) on FR2 - Note: The QCL Type A RS(s) applied to each CC/BWP is that corresponding to the same resource ID(s) indicated by the TCI state IDs - FFS: operation/signaling details including the possibility to activate different sets of PDSCH TCI state IDs for multiple CCs/BWPs - Note: QCL type-A comes from the BWP where the TCI state is applied Agreement made in RAN1#98Bis When a set of TCI-state IDs for PDSCH are activated by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the same set of TCI-state IDs are applied for the all BWPs in the indicated CCs. - Further signaling details are up to RAN2. - Whether to support the inter-band CA for this feature will be decided in RAN1#99. - Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same set of PDSCH TCI state IDs for multiple CCs/BWPs is up to capability discussion. - FFS on the UE capability signaling details - Note: This at least applies to single TRP case. - FFS: How many combinations of CCs can be configured by RRC and relevant UE capability

Activating TCI states of multiple CCs/BWPs through a single MAC-CE command reduces overhead and latency. This is an important feature. RAN4 should define requirements for this.

Proposal 1: RAN4 defines requirements for activating TCI states simultaneously across multiple CCs/BWPs with a single MAC-CE command.

# MAC-CE based Spatial Relationship Update for AP-SRS

This topic is also getting discussed in NR RRM core requirement enhancement section (agenda item 8.15.1 for this meeting). RRM eMIMO session should wait until the RRM core requirement enhancement section spends further time on this topic.

Proposal 2:  The decision whether to define MAC-CE based spatial relationship update for AP-SRS or not should be taken in ‘RRM core requirement enhancement’ section (Agenda Item: 8.15.1).

We have already submitted a contribution regarding this topic to AI 8.15.1. Still, we mention the relevant proposals in this contribution.

## 3.1. Details of MAC-CE based Spatial Relationship Update for AP-SRS

Proposal 3: RAN4 to define requirements of MAC-CE based spatial relation update for AP-SRS.

In terms of the reference signals based on which the requirements need to be defined, it is our view that RAN4 should first define requirements for spatial relation that are QCL’d to a reference signal for which the UE can provide an L1-RSRP measurement, namely SSB and CSI-RS. This would allow us to highly leverage the requirements as defined for TCI state switching. Requirements for spatial relation that are QCL’d to SRS can be defined in the second stage. Note that in the above case, if the QCL chain for a spatial relation includes SSB or CSI-RS we can define the requirements in the first stage.

Proposal 4: RAN4 to prioritize defining requirements for the case where the spatial relation is QCL’d to (or the QCL chain contains) SSB or CSI-RS.

For the case where the QCL chain for spatial relation contains SSB/CSI-RS, we propose to re-use the known state definition as was defined for the case of TCI state switching. We will need to extend the definition to say that the measurement corresponding to the first DL RS in the QCL chain has been provided within the last 1280 ms and that the SNR of that RS remains above -3 db.

Proposal 5: Re-use the known state definition for TCI state for known spatial relation.

# Conclusion

Proposal 1: RAN4 defines requirements for activating TCI states simultaneously across multiple CCs/BWPs with a single MAC-CE command.

Proposal 2:  The decision whether to define MAC-CE based spatial relationship update for AP-SRS or not should be taken in ‘RRM core requirement enhancement’ section (Agenda Item: 8.15.1).

Proposal 3: RAN4 to define requirements of MAC-CE based spatial relation update for AP-SRS.

Proposal 4: RAN4 to prioritize defining requirements for the case where the spatial relation is QCL’d to (or the QCL chain contains) SSB or CSI-RS.

Proposal 5: Re-use the known state definition for TCI state for known spatial relation.

# Reference

- Chairman’s notes, 3GPP TSG RAN WG4 Meeting #93

- Chairman’s notes, 3GPP TSG RAN WG1 Meeting #98.

- Chairman’s notes, 3GPP TSG RAN WG1 Meeting #98bis.