3GPP TSG-RAN WG4 Meeting #94-e                                                                 R4-2002141

Electronic Meeting, 24th February – 6th March, 2020

Title:	Draft LS on EN-DC power class

Response to:

Release:	Release 16

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

#### Name:	Liu Ye

E-mail Address:	leo.liuye@huawei.com

Attachments:	None

1. Overall Description:

RAN4 recently had identified potential inconsistency on EN-DC power class between RAN4 implementation and RAN2 signalling design.

For an EN-DC band combination, the power class for NR band which is inside the EN-DC combination is not indicated separately from ue-PowerClass  for BandNR. The power class capability can only be assumed identical for the NR band in both SA and NSA mode according to current signalling design.  However, the power class capability in these two scenarios could be different. For instance, if a UE in TDD NR band supports PC2 UL MIMO, 2Tx of 23dBm+23dBm is assumed based on RAN4 agreement, while for EN-DC combination, this NR band can only support 1Tx of 23dBm together with 1Tx from LTE, which is limited by current UE implementation capability.

RAN4 discussed how to solve the inconsistency issue and in the end made some clarification in RAN4 specification to allow the power class capability difference for NR band in SA and NSA mode in Rel-15. In order to avoid the ambiguity and misalignment between RAN4 and RAN2 specifications, RAN4 thinks that it is necessary to introduce new signalling from Rel-16.

2. Actions:

To RAN2

ACTION: 	RAN4 kindly asks RAN2 to introduce new signalling to distinguish the potential difference for NR SA and NSA power classes in Rel-16.

3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN4 Meeting #94bis	20th – 24th April 2020		 Canada

TSG RAN4 Meeting #95		25th – 29th May 2019	              Athens, Greece