3GPP TSG-RAN WG4 Meeting #94-e		R4-2002156
Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	10.2.3.1

Source:	Keysight Technologies

Title:	Reference Spatial Correlation Curves for Different NR FR1 MIMO OTA Test Frequencies

Document for:	Approval

# Introduction

This contribution presents the spatial correlation reference curves for NR MIMO OTA models at different test frequencies.

# Discussion

The radio channel models of NR MIMO OTA have been defined in [1]. The specified test frequencies for channel model and Quiet Zone validation are specified in [1] as well. A previous contribution on potential channel model validation procedures [2] outlined various test measurement and analysis procedure for NR FR1 MIMO OTA channel model validation. Subsequently, option 3 of spatial sampling for the correlation validation measurement [2] was agreed and incorporated in [1], which is defined on the circumference of the quiet zone, as illustrated in Fig 1. The test zone is a circle with 20 cm diameter in the horizontal plane. The reference point demarcated at AoA 270 is shown by a red marker. The mean AoAs of the CDL-A and CDL-C models are slightly different, but the underlying geometry for the CDL model indicates that the mean AoA (or assumed LoS direction) of the model is 180°. The reference point orientation of the validation measurement was agreed to have a 90° offset to the channel model reference AoA to enable accurate sampling of the main lobe of the spatial correlation curve. The reference point orientation must be defined in the channel model coordinate system instead of the chamber/probe coordinate system to enable optimization of OTA model implementation to achieve better alignment with the cluster AoAs and probe directions. In order to have spatial samples that entail reasonable measurement times and adequately capture the main lobe of the correlation curve, a non-uniform sampling is desired as proposed in [2] where the first quadrant i.e., 270-180, is sampled with dense sampling (λ/10) compared to the rest of the circle (λ/2). ​

Figure 1: Spatial sampling for spatial correlation validation

Figures 2 and 3 illustrate the reference curves plotted for CDL-A UMi and CDL-C UMa NR MIMO OTA models at different test frequencies agreed in [1] and tabulated in Table 1 in a vertically polarized 16 probe OTA setup. The reference curves have been evaluated by employing a numerically optimized OTA probe weighting method in the simulations. The line plots are generated with dense fixed spatial samples on the circumference of the quite zone independent of frequency. These line plots are overlaid with spatial sampling in wavelength, shown as markers, according to the option 3 in [2]. It is interesting to note that as the test frequency decreases and the wavelength increases, the number of spatial samples also decreases due to which it becomes difficult to accurately sample the actual correlation curve. It can be observed in Figure 2 and 3 that only four spatial samples are available at the lowest test frequency. To overcome this problem, two different sampling criteria can be defined for the test frequencies below and above 1800 MHz.

Figure 2: Spatial correlation reference curves for CDL-A UMi model in a vertically polarized 16 probe OTA setup at different NR test frequencies.

Figure 3: Spatial correlation reference curves for CDC-A UMa model in a vertically polarized 16 probe OTA setup at different NR test frequencies.

Table 1: Test frequencies for channel model and Quiet Zone validation

| NR FR1 Bands​ | Range​ | Test frequency (MHz)​ |
| --- | --- | --- |
| n71​ | Low​ | 617MHz ​ |
| n12, n17, n29, n14, n28, [n29]​ |  | 722MHz​ |
| n5, n8, n18, n20​ |  | 836.5MHz​ |
| n50, n51, n74​ | Mid​ | 1575.42MHz​ |
| n3, n2, n25, n39​ |  | 1880MHz​ |
| n1, n34, n65​ |  | 2132.5MHz​ |
| n7, n30, n41, n40, n38, [n90]​ |  | 2450MHz​ |
| n77,n78​ | High​ | 3600MHz​ |
| n79​ |  | [4700MHz] ​ |

Observation 1: The number of spatial samples defined in wavelength becomes small as the test frequency decreases.

Proposal 1: For the lowest four test frequencies, i.e., 617, 722, and 836.5 and 1575.42 MHz, adopt a spatial as λ/15 and λ/4 for first the quadrant of test zone circumference (270◦-180◦) and the remaining three quadrants, respectively.

Proposal 2: For the highest five test frequencies, i.e., 1800, 2132.50, 2450, 3600 and 4700 MHz, adopt the spatial sampling as λ/10 and λ/2 for first quadrant of test zone circumference (270◦-180◦) and the remaining three quadrants, respectively, i.e., same as proposed in [2]

Figure 3. Proposed spatial sampling for validation measurement for test frequencies below and above 1800 MHz: 617 MHz case (left) and 4700 MHz case (right).

If these proposals are agreeable, a revision of this contribution with text proposals for [1] could be prepared with the reference curves plotted and tabulated similar to the tables attached to this contribution.

# Conclusion

We present the spatial correlation reference curves for NR MIMO OTA models at different test frequencies. We reserve the following observation and propose a remedy for the small number of spatial samples at low test frequencies.

Observation 1: The number of spatial samples defined in wavelength becomes small as the test frequency decreases.

Proposal 1: For the lowest four test frequencies, i.e., 617, 722, and 836.5 and 1575.42 MHz, adopt a spatial as λ/15 and λ/4 for first the quadrant of test zone circumference (270◦-180◦) and the remaining three quadrants, respectively.

Proposal 2: For the highest five test frequencies, i.e., 1800, 2132.50, 2450, 3600 and 4700 MHz, adopt the spatial sampling as λ/10 and λ/2 for first quadrant of test zone circumference (270◦-180◦) and the remaining three quadrants, respectively, i.e., same as proposed in [2]

# References

- 3GPP TR 38.827 “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)” V1.0.0, December 2019.

- R4-1915077, Channel Model Validation for FR1 MIMO OTA, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #93, November 2019.