3GPP TSG-RAN WG4 Meeting #94-e	R4-2002182

Electronic Meeting, Feb.24th – Mar.6th 2020

Agenda item:		8.11.2

Source:		Samsung

Title:	WF on NR eMIMO RRM requirements

Document for:		Approval

# 1	Background

Rel-16 NR eMIMO WI (i.e., Enhancements on MIMO for NR) is a RAN1 leading WI with below major enhancement in RAN1 area which has potential RRM requirement impact in RAN4:

- Enhancements on multi-beam operation

  - DL/UL beam indication with reduced latency and overhead

  - Beam failure recovery for SCell

  - L1-SINR measurement

This WF is used to capture further agreements from RAN#94-e.

# 2	Agreement from RAN4#94-E Chairman Notes after 1st Round

Topic #1: L1-SINR Measurement Issue 1-1-1: Applicable condition(s) for one-shot L1-SINR measurement report for CMR only scenario: Agreement: Applicable condition(s) for one-shot L1-SINR measurement report for CMR only scenario: - M=1 shall be applied if - aperiodic CSI-RS resource is configured for channel measurement, or - periodic or semi-persistent CSI-RS resource is configured for channel measurement and higher layer parameter timeRestrictionForChannelMeasurements is configured. Issue 1-3-1: L1-SINR measurement side condition for Es/Iot for CMR+ZP-IMR - Agreement: Es/Iot on CMR: - Option 1: SSB or CSI-RS Es/Iot >= -3dB and <=25dB CRs/TPs CR/TP number; Decision. R4-2000997; Endorsed. Topic #2: SCell Beam Failure Recovery Issue 2-1-4: RS within a deactivated SCC is implicitly configured as the BFD-RS for another activated SCell - Agreement: UE is not required to perform BFD on RS within a deactivated SCC which is implicitly configured as the BFD-RS for another activated SCell Issue 2-1-5: RS within an activated SCC is implicitly configured as the BFD-RS for another deactivated SCell - Agreement: UE is not required to perform BFD on RS within an activated SCC which is implicitly configured as the BFD-RS for another deactivated SCell. Issue 2-1-7: RAN1 specification and RAN4 agreement mismatch for SSB-based BFD on SCell - Agreement: RAN4 revert previous agreement by only allowing CSI-RS based BFD for SCell,to align with RAN1 specification. Topic #3: DL/UL beam indication with reduced latency and overhead Issue 3-2: RAN4 RRM requirement impact due to MAC-CE based spatial relation update for aperiodic SRS Conclusion: The decision on whether to define MAC-CE based spatial relationship update for AP-SRS should be made in ‘NR RRM core requirement enhancement WI’ scope. Postpone discussion in the scope of NR eMIMO WI scope and wait for respective conclusions.

# 3	Agreements for Way Forward

3.1  L1-SINR Measurement

##### 3.1.1 Measurement period for L1-SINR measurement:

Applicable condition(s) for one-shot L1-SINR measurement report for CMR+IMR scenario:

  - M=1 shall be applied if

    - aperiodic CSI-RS resource is configured for interference measurement (for SSB-based CMR or aperiodic CSI-RS based CMR), or

    - periodic or semi-persistent CSI-RS resource is configured for channel and/or interference measurement and higher layer parameter timeRestrictionForChannelMeasurements or timeRestrictionForInterferenceMeasurements is configured.

Measurement period for SSB-based CMR+IMR scenario:

  - By following L1-RSRP measurement requirement and restricting CMR and IMR’s 1-to-1 mapping, measurement period requirements for FR1 and FR2 shall be defined as:

Table x: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity |  |

Table y: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity |  |

    - FFS how to define the value of P, and whether or not a single P value for both CMR and IMR.

    - FFS the scaling factor N under different conditions of “repetition” field of IMR.

Measurement period for CSI-RS-based CMR+IMR scenario:

  - By following L1-RSRP measurement requirement and restricting CMR and IMR’s 1-to-1 mapping, measurement period requirements for FR1 and FR2 shall be defined as:

Table x: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

Table y: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

    - FFS how to define the value of P, and whether or not a single P value for both CMR and IMR.

    - FFS the scaling factor N under different conditions of “repetition” field of CMR and IMR.

Side condition for measurement accuracy:

  - Following previous meeting’s simulation assumption, i.e., -3dB for ideal SINR

    - For NZP-IMR: 	Side condition (SNR) on CMR and Side condition (INR) on IMR are both 0dB

    - For ZP-IMR: 	Side condition (SNR) on CMR are -3dB.

Number of Samples for L1-SINR Measurement:

  - M = [3] if single shot measurement is not applicable for CMR-only, SSB+NZP-IMR, SSB+ZP-IMR, CSI-RS+NZP-IMR and CSI-RS+ZP-IMR.

Number of Scaling for FR2 (value of N)

  - For the cases of CMR only,

    - the variable N can be defined as same as L1-RSRP.

  - For the case of CSI-RS+NZP-IMR, SSB+IMR, CSI-RS+ZP-IMR

    - FFS the variable N.

##### 3.1.2 Measurement Restriction and Scheduling Availability

- NZP-IMR: QCL indication

  - For L1-SINR reporting with CMR + NZP-IMR configured in one CSI report, L1-SINR measurement requirements apply only if the NZP-IMR is not configured in another CSI report whose CMR is not Type-D QCLed; otherwise, longer measurement requirement is expected.

- NZP-IMR: “repetition = on”:

  - For CSI-RS configured as NZP-IMR in FR2, whether “repetition = on” is configurable:

    - Option-1: NZP-IMR can be configured with “repetition = off” or “repetition = on”:

      - The configuration of “repetition = on/off” is present only if its CSI-RS resource set is used for the other purpose than this L1-SINR measurement report;

      - RX beam for NZP-IMR for L1-SINR measurement shall always follow CMR, i.e., same RX filter shall be used.

    - Option-2: NZP-IMR can be configured with “repetition = off” or “repetition = on”:

      - CMR and NZP-IMR should be configured with the same repetition pattern.

      - CMR and NZP-IMR should not overlap in time domain if they are configured with “repetition = on”.

    - Option-3: Depending on scenarios (i.e., configuration restriction):

      - SSB based CMR and NZP-IMR with “repetition = ON” is error configuration;

      - SSB based CMR and NZP-IMR with “repetition = OFF” is correct configuration;

      - NZP CSI-RS based CMR with “repetition = ON” and NZP-IMR with “repetition = ON”  is correct configuration;

      - NZP CSI-RS based CMR with “repetition = ON” and NZP-IMR with “repetition = OFF” is correct configuration;

      - NZP CSI-RS based CMR with “repetition = OFF” and NZP-IMR with “repetition = ON” is error configuration;

      - NZP CSI-RS based CMR with “repetition = OFF” and NZP-IMR with “repetition = OFF” is correct configuration.

- ZP-IMR: “repetition = on”:

  - For CSI-RS configured as ZP-IMR in FR2, whether “repetition = on” is configurable:

    - Option-1: “Repetition” field should not be present for ZP-IMR’s CSI-RS resource set configuration.

    - Option-2: ZP-IMR CSI-RS shall only be configured with “repetition = off” in FR2.

- Measurement restriction between L1-SINR and L1-RSRP

  - For CMR+IMR scenario, FFS when the measurement restrictions need to be applied between L1-SINR measurement and L1-RSRP measurements.

##### 3.1.3 Side condition and Others

- L1-SINR measurement side condition for Es/Iot for CMR+NZP-IMR

  - Es/Iot on CMR and NZP-IMR:

    - Option-1: Es/Iot >= -3dB and <=25dB

    - Option-2: Es/Iot >=0dB and <=25dB, and the resultant L1-SINR should also lie between -3 dB and 25 dB.

3.2  SCell Beam failure recovery

##### 3.2.1 BFD on SCell

- Necessity of BFD procedure on multiple serving cell on the same FR2 band

  - No restriction introduced in RAN4. No RAN4 performance requirement will be defined for performing BFD measurement on more than 1 serving cell.

Sharing factor for BFD Time Period for FR1 and FR2

  - No BFD evaluation period will be specified for performing BFD measurement on more than 1 serving cell per band:

    - Sharing factor for BFD evaluation period due to BFD over multiple Sells will not be introduced for FR2 intra-band CA

    - No RAN4 performance requirement will be defined for more than 1 serving cell per band perform BFD measurement

    - FFS Sharing factor for BFD evaluation period for FR1 CA, FR2 inter-band CA and FR1-FR2 CA.

When more than 2 BFD-RSs are transmitted on a CC for current SCell and (implicitly configured for) other SCell

  - When more than 2 BFD-RSs are transmitted on a CC for current SCell and (implicitly configured for) other SCell, it is up to UE implementation to select two BFD-RSs in active BWP in current CC to perform BFD (either for current SCell or for other Sell(s)).

##### 3.2.2 CBD on SCell

- Sharing factor for CBD Time Period

  - No CBD evaluation period will be specified for performing CBD measurement on more than 1 serving cell per band:

    - Sharing factor for CBD evaluation period (due to CBD over multiple SCells) will not be introduced for FR2 intra-band CA.

    - No RAN4 performance requirement will be defined for more than 1 serving cell perform CBD measurement.

    - FFS Sharing factor for CBD evaluation period for FR1 CA, FR2 inter-band CA and FR1-FR2 CA.

##### 3.2.3 SCell Beam Failure Recovery ReQuest (BFRQ) Mechanism

- RAN4 requirement defined for two step BFRQ mechanism

  - RAN4 FFS whether and how the requirement could be defined for step 1 of BFR, where UE reports beam failure event through a dedicated SR like PUCCH resources:

    - No RRM core requirement is defined for UE conveying new beam information and failed CC indices via MAC-CE.

    - Applicable scenario:

      - Option-1: SCells with DL only and SCells with DL and UL

      - Option-2: SCells with DL only

FFS BFRQ requirement details, if the requirement of step-1 of BFR is agreed to be introduced:

  - Option-1: After detecting beam failure in an Scell and determining that the L1-RSRP of one candidate beam in SCell is greater than the configured threshold, UE is required to transmit scheduling request in the PSCell within a period T

    - Where T is equal to the periodicity of PUCCH that has been configured with schedulingRequestForBFR.

  - Other options are not precluded.

##### 3.2.4 UE Capability and Others

- UE Capability of Number of SCells for BFR:

  - RAN4 requirement should not block the possibility of configuring BFR on multiple SCells, and no RAN4 performance requirement will be defined for more than 1 serving cell per band perform BFD/CBD measurement.

- When CBD-RS not configured, UE’s expected behavior:

  - FFS UE is not required to perform BFD and CBD for a SCell which is not configured with CBD-RS resources.

  - FFS UE is required to perform BFD and CBD for a SCell if some RS resources are configured in SPCell but if they are implicitly configured as CBD-RS resources for SCell.

3.3 DL/UL beam indication with reduced latency and overhead

##### 3.3.1 Simultaneous TCI States Activation/Selection across Multiple CCs/BWPs

- Simultaneous TCI States Activation/Selection across Multiple CCs/BWPs:

  - No new requirement is introduced for the simultaneous TCI states activation/selection.

    - Rel-15 active TCI state switching delay requirements shall still be followed if simultaneous TCI states activation or selection across multiple CCs/BWPs is performed.

# 4	Reference

| T-doc Number | Title | Source | Type |
| --- | --- | --- | --- |
| R4-2000285 | On the Remaining Issues for L1-SINR Measurement Requirement | Samsung | discussion |
| R4-2000286 | Simulation Results for L1-SINR Measurement Accuracy | Samsung | discussion |
| R4-2000287 | Simulation Results Summary for L1-SINR Measurement Accuracy | Samsung | discussion |
| R4-2000288 | CR to TS38.133 on L1-SINR Measurement Requirement (Section 3.3 and 9) | Samsung | CR |
| R4-2000289 | On the Remaining Issues for SCell Beam Failure Recovery RRM Requirement | Samsung | discussion |
| R4-2000290 | CR to TS38.133 on SCell BFD and CBD (Section 8.5) | Samsung | CR |
| R4-2000291 | CR to TS38.133 on SCell BFRQ Procedure (Section 8.5) | Samsung | CR |
| R4-2000292 | On the Remaining Issues for Enhancement on UL/DL Transmit Beam Selection with Reduced Latency and Overhead | Samsung | discussion |
| R4-2000384 | Discussion about L1-SINR measurement requirements | Intel Corporation | discussion |
| R4-2000635 | Simulation results on L1-SINR | CMCC | discussion |
| R4-2000935 | Discussion on RRM requirements for L1-SINR | MediaTek inc. | discussion |
| R4-2000936 | Discussion on L1-SINR delay requirement | MediaTek inc. | discussion |
| R4-2000937 | Discussion on L1-SINR accuracy requirement | MediaTek inc. | discussion |
| R4-2000938 | Discussion on RRM requirements for BFR on Scell | MediaTek inc. | discussion |
| R4-2000997 | CR on SS-SINR and CSI-SINR measurement report mapping (section 10.1.16.1) | OPPO | draftCR |
| R4-2001362 | L1-SINR measurement period | Ericsson | discussion |
| R4-2001578 | Discussion on L1-SINR measurement requirements for NR eMIMO | Huawei, HiSilicon | discussion |
| R4-2001579 | Discussion on measurement restrictions for L1-SINR measurement | Huawei, HiSilicon | discussion |
| R4-2001580 | discussion on SCell BFR requiremetns for NR eMIMO | Huawei, HiSilicon | discussion |
| R4-2002120 | RRM requirements for L1-SINR estimation | Qualcomm | discussion |
| R4-2002121 | RRM requirements for SCell BFD, CBD and BFR | Qualcomm | discussion |
| R4-2002122 | DL/UL beam indication with reduced latency and overhead | Qualcomm | discussion |