3GPP TSG-RAN WG4 Meeting #94-e	R4-2002838

- Electronic meeting, 24 February – 6 March 2020

Source: 	Ericsson

Title: 	TP to TR 38.887: BS RF requirement for band n259

Agenda Item:	9.16.2

Document for:	Approval

# Introduction

The WI on introduction of band n259 should be finalized in this meeting. This contribution provides text proposal to TR 38.887 on BS RF requirements based on contributions [1] and [2].

# Reference

[1] R4-1912184, BS specific RF requirements for band n259, Ericsson, RAN4#92bis

[2] R4-1914084, BS RF requirements for band n259, NTT DOCOMO, INC., RAN4#93

## <Start of Text Proposal>

## 8.2	BS specific

Based on current BS specification, at least the following requirements needed to be considered for introduction of Band n259.

Table 8.2-1: Required changes in TS 38.104 for n259

| Section | Requirement | Required changes in TS 38.104 |
| --- | --- | --- |
| 5.2 | Operating bands | Operating band of n259 shall be added in to Table Table 5.2-2. |
| 5.3.5 | BS channel bandwidth per operating band | Channel bandwidths and corresponding SCS for n259 shall be added in Table 5.3.5-2. |
| 5.4.2.3 | Channel raster entries for each operating band | Applicable NR-ARFCN for n259 shall be added into Table 5.4.2.3-2 once the lower frequency limit for n259 is determined. |
| 5.4.3.3 | Synchronization raster entries for each operating band | Applicable SS raster entries for n259 shall be added into Table 5.4.3.3-2 once the lower frequency limit for n259 is determined. |

The BS RF requirements summarized in Table 8.2-2 is band agnostic RF requirements for FR2 which are applicable for band n259 as well.

Table 8.2-2: Summary on band agnostic of BS RF requirements for FR2

| BS TX side capture in TS 38.104 | BS RX side capture in TS 38.104 |
| --- | --- |
| 9.2 Radiated transmit power | 10.3 OTA reference sensitivity level |
| 9.3 OTA Base station output power | 10.5 OTA In-band selectivity and blocking |
| 9.4 OTA Output power dynamics | 10.7 OTA Receiver spurious emissions |
| 9.5 OTA Transmit ON/OFF power | 10.9 OTA In-channel selectivity |
| 9.6 OTA Transmitted signal quality |  |
| 9.7.2 OTA Occupied bandwidth |  |
| 9.7.5 OTA Transmitter spurious emissions |  |

## In the following sections BS specific requirements for band n259 are presented.

### 8.2.1 Radiated transmitter characteristics

#### 8.2.1.1   Adjacent Channel Leakage Ratio (ACLR)

The BS OTA ACLR limit for spectrum range 37 – 52.6 GHz has been defined in TS 38.104. This is also applicable for Band n259.

#### 8.2.1.2	OTA operating band unwanted emissions

The BS OTA operating band unwanted emission for spectrum range 37 – 52.6 GHz has been specified in TS 38.104, section 9.7. Those limits are applicable for Band n259.

#### 8.2.1.3  Definition of ΔfOBUE and ΔfOOB

In TS 38.104, the definition of ΔfOBUE and ΔfOOB applies to 3250 MHz operating bandwidth while for band n259 the operating bandwidth is FDL,high – FDL,low == 4000 MHz.   For ΔfOBUE and ΔfOOB to be applicable for band n259 the following tables in TS 38.104 should be modified as follows:

Table 9.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics | ΔfOBUE (MHz) |
| --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 MHz | 10 |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 MHz | 40 |
| BS type 2-O | FDL,high – FDL,low ≤ 4000 MHz | 1500 |

The same modification should be performed to Table 6.7.1-1: Maximum offset ΔfOBUE outside the downlink operating band in TS 38.141-2.

#### 8.2.1.4  Step frequencies for Tx spurious emission

It is also needed to add band n259 to Table 9.7.5.3.2.3-2 in TS 38.104 thus Table 9.7.5.3.2.3-2 should be modified as below to include band n259.

Table 9.7.5.3.2.3-2: Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 [GHz] | Fstep,2 [GHz] | Fstep,3 [GHz] (Note 2) | Fstep,4 [GHz] (Note 2) | Fstep,5 [GHz] | Fstep,6 [GHz] |
| --- | --- | --- | --- | --- | --- | --- |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23,5 | 35,5 | 38 | 45 | 47,5 | 59,5 |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [19], Annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in Table 9.7.1-1. |  |  |  |  |  |  |

### 8.2.2	Radiated receiver characteristic

The BS OTA Receiver characteristics have been specified in TS 38.104. The requirements are either band agnostic or are defined for spectrum range 37 – 52.6 GHz. Those limits are applicable for Band n259.

#### 8.2.2.1  Definition of ΔfOBUE and ΔfOOB

In TS 38.104, the definition of ΔfOBUE and ΔfOOB applies to 3250 MHz operating bandwidth while for band n259 the operating bandwidth is FDL,high – FDL,low == 4000 MHz.   For ΔfOBUE and ΔfOOB to be applicable for band n259 the following tables in TS 38.104 should be modified as follows:

Table 10.5.2.3-0: ΔfOOB offset for NR operating bands in FR2

| BS type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| BS type 2-O | FUL_high – FUL_low ≤ 4000 MHz | 1500 |

The same modification should be performed inTable 7.5.2.5.3-0: ΔfOOB offset for NR operating bands in FR2 in TS 38.141-2.

- 8.2.2.2  Step frequencies for Rx spurious emission

It is also needed to add band n259 to Table 10.7.3-2in TS 38.104 thus Table 10.7.3-2should be modified as below to include band n259.

Table 10.7.3-2: Step frequencies for defining the radiated Rx spurious emission limits for BS type 2-O

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) | Fstep,4 (GHz) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23,5 | 35,5 | 38 | 45 | 47,5 | 59,5 |
| n260 | 25 | 34 | 35.5 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26.0 | 29.85 | 30.35 | 38.35 |

## <End of Text Proposal>