3GPP TSG-RAN WG4 Meeting #94-e	R4-2002303

Electronic Meeting, Feb.24th – Mar.6th 2020

Agenda item:			8.2

Source:	Moderator (LG Electronics)

Title:	Email discussion summary for RAN4#94e_#49_NR_CLI_RIM_RRM

Document for:	Information

Introduction

This email discussion is for Rel-16 CLI RRM core maintenance and performance in Agenda 8.2.2 and 8.2.3. List of candidate target of email discussion for 1st round as follows:

- 1st round:

  - Topic#1 : CLI core requirement maintenance

    - Sub-topic 1-1 : Scheduling restriction

  - Topci#2 : CLI performance requirements

    - Sub-topic 2-1 : CLI measurement accuracy

    - Sub-topic 2-2 : CLI performance test case

    - Sub-topic 2-3 : Minimum SRS RP

Based on chairman’s report for 1st round, list candidate target of e-mail discussion for 2nd round as follows:

- 2nd round :

  - Topic#1 : CLI core requirement maintenance

    - Focus LS(R4-2001622) : e-mail discussion using sub-thread

    - Review the CR(R4-2001623)

  - Topic#2: CLI performance requirements

    - Focus WF : e-mail discussion using sub-thread

    - Review the CR(R4-2001624 and R4-2002255)

    - Work split for CRs of CLI performance part

Topic #1: CLI core requirement maintenance

Rel-16 CLI core was closed in last RAN4 meeting. This section will treat CLI core maintenance. Issues are listed in the following sub-section.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000653 | Nokia, Nokia Shanghai Bell | Proposal1: The RAN4 scheduling restriction on the PDCCH/PDSCH reception is not valid and shall be updated to align with RAN2 agreements. Proposal2: RAN4 have following questions clarified to properly update the RAN4 spec on scheduling restriction: 1. Does the UE keep monitoring the PDCCH/PDSCH on the CLI measurement resources? 2. Does the UE measure CLI on the SRS RSRP resources after DL reception if being scheduled, in case DL signal/channel and SRS RSRP resources are FDMed and the UE does not support FDMed reception? 3. Does the UE prioritize DL transmission on the 1 or 2 OFDM symbols before the OFDM symbol used for CLI measurements? Proposal3: It is proposed to send LS to RAN2 asking the above questions and confirm the RAN4 understanding if there is any consensus. |
| R4-2000960 | LG Electronics Inc. | Proposal: Update scheduling availability for CLI measurement based on CLI capabilities. |
| R4-2001621 | Huawei, HiSilicon | Proposal 3: RAN4 should inform RAN2 about the RAN4 agreement to apply scheduling restriction for CLI measurement, and suggest RAN2 to revisit their agreements as indicated in [3]. |
| R4-2001622 | Huawei, HiSilicon | Reply LS on CLI measurement capability |

Open issues summary

Sub-topic 1-1 : Scheduling restriction

There are conflicting agreement between RAN2 and RAN4 for UE behavior when DL signal/channel and CLI measurement resources are FDMed. RAN4 needs to decide whether to keep RAN4 agreement for scheduling restriction.

RAN2 agreed UE capabilities for FDMed between DL signal/channel and CLI measurement resources. Based on these UE capabilities, RAN4 needs to update scheduling restriction part in RAN4 specification, provided that it will decide after Issue 1-1 is clear.

Issue 1-1-1: Conflicting UE behaviour between RAN2 and RAN4

- Proposals

  - Option 1: Align with RAN2 agreements and clarify UE behaviour for FDMed scenarios

  - Option 2: keep RAN4 agreements and send LS to RAN2

- Recommended WF

  - TBA

Issue 1-1-2: Scheduling restriction based on UE capabilities

- Proposals

  - Option 1: Update scheduling restriction based on UE capabilities for FDMed between DL signal/channel and CLI measurement resource

- Recommended WF

  - TBA

Issue 1-1-3: handling LS

- Proposals

  - Moderator : depending on conclusion of Issue 1-1-1.

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Issue 1-1-1 : Conflicting UE behaviour between RAN2 and RAN4

| Company | Comments |
| --- | --- |
| LG | Support option 2. UE behavior for measurements should be considered in RAN4. So, RAN4 needs to send LS with RAN4 agreements to RAN2. |
| Huawei, HiSilicon | Support option 2. If option 1 is adopted, CLI measurement would have no guaranteed performance due to dynamic scheduling in the serving cell. Also the UE behavior would be complex as UE needs to determine if there is DL signals/channels overlapping with the CLI measurement resource early enough. |
| QC | We support option 2 also. In general, in RAN4 we have been prioritizing measurements over DL/UL. We should keep the same here and keep the scheduling restrictions in place. |
| Nokia, Nokia Shanghai Bell | RAN4 should follow RAN1/RAN2 agreements and then adapts the RAN4 spec. As the details of the scheduling principles are not clear, we need send LS to RAN1/RAN2 for the clarification. |
| LG | For Nokia comments, In general, RAN4 follows RAN1/RAN2 agreements, but UE behavior related measurement should be decided in RAN4 due to guarantee measurement performance. Additionally, RAN2 agreements for this issue are not captured in RAN2 specification, and if it is critical issue in RAN2, RAN2 would have informed this decision to RAN4. So, Option 2 should be considered. |
| NEC | We also support Option 2. Same view as QC. |
| DCM | Support option 2. |

Issue 1-1-2 : Scheduling restriction based on UE capabilities

| Company | Comments |
| --- | --- |
| LG | According to RAN2 UE capability for FDMed between DL signal/channel and CLI measurement resource, RAN4 needs to capture it in scheduling restriction part. |
| Huawei, HiSilicon | Support option 1. If it is agreeable, we assume the proponent (LGE) will prepare CR during the meeting to capture it. |
| QC | Updating with actual capability names makes sense |
| Nokia, Nokia Shanghai Bell | The UE capabilities need to be updated in accordance with RAN2 definition. But it can be done after issue 1-1-1 is solved. |
| NEC | Since there is conflicting UE behavior between RAN2 and RAN4, we also feel that it can be done after issue 1-1-1 is resolved. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2001623 | LG : Following sentence in this CR is better to move into chapter 9.7.1 with revised For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET. |
|  | Huawei, HiSilicon: we are fine with above comments from LGE, and will take it in the revision. |
|  | Nokia, Nokia Shanghai Bell: It needs further discussion. UE behavior shall be firstly clarified by RAN1/2. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | The tentative agreement for Issue 1-1-1 ~ 1-1-3 are based on majority companies’ view Issue 1-1-1: Conflicting UE behaviour between RAN2 and RAN4 Tentative agreements: Option 2 (keep RAN4 agreements and send LS to RAN2) Recommendations for 2nd round: N/A Issue 1-1-2: Scheduling restriction based on UE capabilities Tentative agreements: Update scheduling restriction based on UE capabilities for FDMed between DL signal/channel and CLI measurement resource Recommendations for 2nd round: recommend capturing the contents of R4-2000960 to Huawei CR (R4-2001623) in 2nd round Issue 1-1-3: handling LS Tentative agreements: send LS based on R4-2001622 (Huawei) Recommendations for 2nd round: Companies are encouraged to check the reply LS |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2001623 | To be revised. (revised T-doc number is needed) |

Discussion on 2nd round (if applicable)

Companies are encouraged to review the following LS and CR.

Based on session chairman’s decision, revised T-doc number for LS and CR are assigned.

- LS : R4-2001622 Reply LS on CLI measurement capability, Huawei

Revised T-doc number is R4-2002221.  Huawei will trigger e-mail discussion using sub-thread

From chairman report Issue 1-1-1: Conflicting UE behaviour between RAN2 and RAN4 Agreement: Option 2 (keep RAN4 agreements and send LS to RAN2)

- CR : R4-2001623 CR on CLI measurement requirements, Huawei

  - Revised T-doc number is R4-2002220.

Companies views’ collection for 2nd round

| Company | Comments for the CR |
| --- | --- |
| Nokia, Nokia Shanghai Bell | - In 9.7.1, the constant offset value should be a length of a time period, which should be “Tc*NTA_Offset” instead of NTA_Offset? - 9.7.7.2, we don’t think the SRS configuration should be added here. Maybe can be moved to accuracy section? - 9.7.2.5, do we use 1.5x for DRX < 320ms? - For the measurement resource overlapping with L1 measurement, have we discussed it before? Should this be RAN1 issue? - 9.7.4, it should be “For the UE which neither support cli-SRS-RSRP-FDM_DL nor cli-RSSI-FDM-DL”? |
| LG | For 9.7.4, two UE capabilities for CLI measurement are defined, and a UE can support either one or both. So, if the UE does not support only cli-SRS-RSRP-FDM-DL, only SRS-RSRP measurement restriction should be applied. I think that it is better to change “For a CLI measurement which a UE does not support cli-SRS-RSRP-FDM-DL or cli-RSSI-FDM-DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs the CLI measurement, and on 1 data symbol before an OFDM symbol used for the CLI measurement for 15 kHz and 30 kHz subcarrier spacing.” |
| Huawei, HiSilicon | To Nokia: - Agree, will update in the revision. - The question is whether UE is supposed to meet the core requirements if the SRS configuration is different than what is listed in 9.7.7.2. Our understanding is ‘no’, so we think it should be captured here. We are open to more opinions. - The 1.5 relaxation for DRX is same as for other L3 measurement. For example, the same is applied for intra- and inter-frequency measurement in 9.2 and 9.3. - In the current spec, the requirements for CLI measurement do not apply if the measurement resource is partially or fully overlapping with SMTC window or measurement gaps. The reason is that CLI measurement is performed with different timing than L3 measurement. In our understanding, the same problem exists for L1 measurement also, so the same rule should apply. What is your technical view on this? - Agree, the scheduling restriction should be defined separately for SRS-RSRP and CLI-RSSI, and each should be based on the corresponding capability. Will update in the revision. To LGE: Same as 5 for Nokia. |
| Nokia, Nokia Shanghai Bell | These two issues are separate from UE capability. We would like to have more study on them and come back next meeting. - As agreed in earlier meeting, the SRS-RSRP requirements are defined for limited SRS configurations, so the applicable SRS configurations should be defined. - The measurement resource overlapping with L1 measurement should also be considered. |
| Huawei, HiSilicon | We are fine to come back on the 2 issues you pointed out in next meeting. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2002221 | Agreeable (Reply LS on CLI measurement capability, Huawei) RAN2 has decided to follow RAN4’s agreement during in this E-meeting, so RAN4 agreements are captured in this LS. |
| R4-2002220 | Agreeable (CR on CLI measurement requirements, Huawei) All companies’ comments are captured. |

Topic #2: CLI performance requirements

Rel-16 CLI performance part start from RAN4#94-e. This section discuss CLI performance test cases and configurations for SRS-RSRP and CLI-RSSI. Issues are listed in the following sub-section.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000654 | Nokia, Nokia Shanghai Bell | Proposal1: It is proposed to adopt the CLI accuracy requirements values as defined in Table 1. |
| R4-2001621 | Huawei, HiSilicon | Proposal 1: The SRS-RSRP accuracy requirements at normal condition and normal Io are - FR1: 3.5dB for 15kHz, 4dB for 30kHz, 5.5dB for 60kHz - FR2: 6.5dB for 60kHz, 10dB for 120kHz Proposal 2: Re-use LAA RSSI accuracy requirements for CLI-RSSI for FR1, and allow 1.5dB relaxation for CLI-RSSI for FR2. |
| R4-2000962 | LG Electronics Inc. | Proposal: Use SRS-RSRP measurement accuracy of Table 2-1. |
| R4-2000958 | LG Electronics Inc. | Proposal: Use Table 2-1 and Table 2-2 for minimum SRS RP level for FR1 and FR2 |
| R4-2000961 | LG Electronics Inc. | Proposal 1: Select one option for SRS-RSRP measurement accuracy tests depending on SCS configuration. Proposal 2: Use timing error as TC × NTA_offset + 4.67us for FR1 and TC × NTA_offset + 3.67us for FR2 Proposal 3: Use SRS configuration for SRS-RSRP measurement accuracy test in Table 2-2. Proposal 4: Use AoA steup#1 for CLI measurement tests in FR2. |
| R4-2001625 | Huawei, HiSilicon | Proposal: Agree on the test case list as in Table 1-4. |

Open issues summary

Sub-topic 2-1 : CLI measurement accuracy

Collecting simulation results for SRS-RSRP based on last meeting agreements to define CLI measurement accuracy.

Issue 2-1-1: Define measurement accuracy for SRS-RSRP

- Proposals

  - Companies’ proposals (normal condition) are summarized in following table

| FR | SCS | Accuracy Requirement ( ±[ ]dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Huawei | Nokia | LG |
| FR1 | 15kHz | 3.5 | 2.9 | 3 |
|  | 30kHz | 4 | 3.5 | 4 |
|  | 60kHz | 5.5 | 5 | 5.5 |
| FR2 | 60kHz | 6.5 | 5.8 | 6.5 |
|  | 120kHz | 10 | 8.4 | 8.5 |

- Recommended WF

  - TBA

Issue 2-1-2: Define measurement accuracy for CLI-RSSI

- Proposals

  - Option 1: Re-use LAA RSSI accuracy requirements for CLI-RSSI for FR1, and allow 1.5dB relaxation for CLI-RSSI for FR2.

- Recommended WF

  - TBA

Issue 2-1-3: Measurement accuracy for extreme condition and high Io

- Proposals

  - Option 1:

    - Extreme condition with Io=-70dBm/BW is derived by adding 4.5/3dB for FR1/FR2 to the absolute L1-RSRP accuracy in normal condition.

    - Normal condition with Io=-50dBm/BW is derived by adding 3.5/3dB for FR1/FR2to the absolute L1-RSRP accuracy in normal condition with Io=-70dBm/BW.

    - Extreme condition with Io=-50dBm/BW is derived by adding 2/2dB for FR1/FR2 to the absolute L1-RSRP accuracy in extreme condition with Io=-70dBm/BW.

- Recommended WF

  - TBA

Sub-topic 2-2 : CLI performance test case

This sub-section discusses test case lists and test configurations for SRS-RSRP and CLI-RSSI.

Issue 2-2-1: Test case list

- Proposals

  - Option 1: (R4-2001625)

    - Event triggered reporting test cases for FR1/FR2 EN-DC/SA

      - With non-DRX / with DRX

    - Measurement accuracy test cases for FR1/FR2 EN-DC/SA

- Recommended WF

  - TBA

Issue 2-2-2: Test configuration: sub-tests for measurement accuracy

- Proposals

  - Option 1:

    - SRS-RSRP for FR1: 3 sub-tests with different SRS-RSRP and Io levels (similar to SS-RSRP)

    - SRS-RSRP for FR2: 2 sub-tests with different SRS-RSRP and Io levels (similar to SS-RSRP)

    - CLI-RSSI : 1 sub-test (similar to LAA RSSI)

- Recommended WF

  - TBA

Issue 2-2-3: Test configuration: SCS & BW

- Proposals

  - Option 1: 15kHz+10MHz, 30kHz+40MHz, 120k+100MHz

- Recommended WF

  - TBA

Issue 2-2-4: Test configuration: timing difference between DL and CLI measurement resource

- Proposals

  - Option 1: TC × NTA_offset + 4.67us for FR1 and TC × NTA_offset + 3.67us for FR2

- Recommended WF

  - TBA

Issue 2-2-5: Test configuration: SRS configuration for SRS-RSRP

- Proposals

  - Option 1: reuse the same format SRSConf.1 of Table A.4.4.1.1.1-3 in TS38.133

    - freqHopping c-SRS :12

    - SRS is configured on the last symbol in the S-slot

- Recommended WF

  - TBA

Issue 2-2-6: Test configuration: AoA setup

- Proposals

  - Option 1: AoA setup#1 for FR2 test cases

- Recommended WF

  - TBA

Sub-topic 2-3 : Minimum SRS RP

This sub-section discusses minimum SRS_RP value for FR1 and FR2

Issue 2-3-1: Minimum SRS_RP

- Proposals

  - Option 1:

    - For FR1

| Parameter | NR operating band groups Note1 | Minimum SRS_RP |  |  | SRS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSSRS |  |  | dB |
|  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |
| Conditions | NR_TDD_FR1_A | -120 | -117 | -114 | 1 |
|  | NR_TDD_FR1_C | -119 | -116 | -113 |  |
|  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 |  |
|  | NR_TDD_FR1_E | -118 | -115 | -112 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

    - For FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SRS_RP Note 2, Note 3 |  |  |  |  | SRS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSRS |  |  |  |  | dB |
|  |  |  | SCSSRS = 60 kHz |  |  |  | SCSSRS = 120 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -124.5 | -119.0 | -115.3 | -124.0 | (Value for SCSSRS = 60 kHz) +3dB | ≥1 |
|  |  | n258 | -124.5 | -119.0 | -115.3 | -124.0 |  |  |
|  |  | n260 | -121.5 |  | -112.7 | -122.0 |  |  |
|  |  | n261 | -124.5 | -119.0 | -115.3 | -124.0 |  |  |
|  | Spherical coverage Note 1 | n257 | -116.5 | -108.0 | -104.4 | -115.0 | (Value for SCSSRS = 60 kHz) +3dB | ≥1 |
|  |  | n258 | -116.5 | -108.0 | -104.4 | -115.0 |  |  |
|  |  | n260 | -113.5 |  | -100.1 | -110.0 |  |  |
|  |  | n261 | -116.5 | -108.0 | -104.4 | -115.0 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SRS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Sub-topic 2-1 : CLI measurement accuracy

Issue 2-1-1: Define measurement accuracy for SRS-RSRP

| Company | Comments |
| --- | --- |
| LG | we suggest following values for SRS-RSRP measurement accuracy (majority values) FR; SCS; Accuracy Requirement ( ±[ ]dB). FR1; 15kHz; 3. 30kHz; 4. 60kHz; 5.5. FR2; 60kHz; 6.5. 120kHz; 9. |
| Huawei, HiSilicon | For most cases results from 3 companies (Nokia, LGE and Huawei) are aligned with the difference < 0.5dB. The exception case is the 120kHz for which the results are listed below. It seems LGE’s results are clearly better than Nokia’s and ours. We suggest to keep requirements for this case FFS. Channel; Max (5%, 95%) in the results. Huawei; Nokia; LG. AWGN; 4.3; 4.4; 3.8. TDL-A; 5.6; 4.8; 4.3. TDL-C; 5.2; 5.2; 4.3. We would like to also note that proposed requirements in Nokia paper R4-2000654 are based on AWGN results. It should be based on worst case among all channels instead. |
| QC | Question for clarification, do the numbers proposed above contain the RF margin or do we add margin on top? |
| Nokia, Nokia Shanghai Bell | The performance requirement shall be defined under AWGN channel. The averaged value can be considered under the same simulation assumption. |
| LG | For QC’s comments The proposed values have already captured certain RF margin. For Nokia’s comments I have the same understanding with Huawei. It should be based on worst case among all channels For all, I think that following measurement accuracy is agreeable except 120kHz in FR2. FR; SCS; Accuracy Requirement ( ±[ ]dB). FR1; 15kHz; 3. 30kHz; 4. 60kHz; 5.5. FR2; 60kHz; 6.5. 120kHz; FFS. For 120kHz in FR2, we have two possible approaches Alt1: use average value among companies’ results (9dB) Alt2: keep FFS, and discuss it in the next F2F meeting |

Issue 2-1-2: Define measurement accuracy for CLI-RSSI

| Company | Comments |
| --- | --- |
| LG | We are fine to reuse LAA RSSI measurement requirements for CLI-RSSI. |
| Huawei, HiSilicon | Option 1. It should be noted that in option 1 there is a 1.5dB relaxation for FR2 considering the difference in RF margin. |
| QC | We would be ok to consider re-using the LAA RSSI measurements as baseline but postpone the decision to next meeting. Can HW also elaborate why do we only consider 15 dB as relaxation. |
| Nokia, Nokia Shanghai Bell | Agree to the proposal. Some relaxed margin can be considered for FR2. |

Issue 2-1-3: Measurement accuracy for extreme condition and high Io

| Company | Comments |
| --- | --- |
| LG | We are fine for option 1. |
| QC | We are fine with the first two bullet points which are aligned with intra-freq RSRP measurements . However, for extreme condition with Io=-50dBm/BW we would also like to keep the same alignment and add 3/3/dB for FR1/FR2 |
| Nokia, Nokia Shanghai Bell | Agree to the proposal. |

Sub-topic 2-2 : CLI performance test case

Issue 2-2-1 and Issue 2-2-2: Test case list and sub-tests for measurement accuracy

| Company | Comments |
| --- | --- |
| LG | Define event triggered reporting test cases and measurement accuracy test cases For sub-test, we can consider option 1 as starting point. |
| QC | I am trying to understand why we do we need two test cases for delay in CLI. Only non-DRx should be enough here. |
| Nokia, Nokia Shanghai Bell | Agree to define the test cases in SA FR1 and SA FR2. More discussion is needed for EN-DC. |
| LG | For QC’s comment I think that most use cases of CLI measurements will be non-DRX, but CLI measurement on DRX could be performed from the specification perspective. For Nokia’s comment EN-DC is not preclude in CLI feature. |

Issue 2-2-2 ~ Issue 2-2-6: Test configuration

| Company | Comments |
| --- | --- |
| LG | We prefer to use each option for Issue 2-2-2~2-2-5. For Issue 2-2-6, we prefer AoA Setup#1. In this online meeting, we need to make agreement as many test parameters for CLI performance test cases as possible to prepare initial draft CRs in the next meeting. |
| Huawei, HiSilicon | Support option 1 in each of Issue 2-2-3 ~ 2-2-6. |
| QC | Issue 2-2-2- Issue 2-2-4: We are fine with option 1 Issue 2-2-5: Need more time Issue 2-2-6: We would like to do these tests with 2 AoA since that represents a more realistic scenario as would be seen in the field. |
| Nokia, Nokia Shanghai Bell | We can reuse existing configuration if available. For the new conditions/configuration values, more discussion is needed. |
| LG | It seems to be agreeable For Issue 2-2-2 ~ 2-2-4. For AoA setup, UE does not need to sweep Rx beam for CLI measurement, so AoA setup#1 is enough to verify performance test. |

Sub-topic 2-3 : Minimum SRS RP

Issue 2-3-1: Minimum SRS_RP

| Company | Comments |
| --- | --- |
| LG | If these values are agreeable, we will prepare CR in this online meeting. |
| Huawei, HiSilicon | Support option 1. |
| QC | We are fine with option 1 too. |
| Nokia, Nokia Shanghai Bell | Agree. |
| LG | Thanks, I’ll share draft version. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2001624 | LG : pending until Issue 2-1-1~2-1-3 are clear. |
|  | Nokia, Nokia Shanghai Bell: It can be done after we agree with the accuracy values. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 ~ 2-3 | Issue 2-1-1: Define measurement accuracy for SRS-RSRP Tentative agreements: FR; SCS; Accuracy Requirement ( ±[ ]dB). FR1; 15kHz; 3. 30kHz; 4. 60kHz; 5.5. FR2; 60kHz; 6.5. 120kHz; FFS. Candidate options: Option1: use average value among companies’ results (9dB) Option2: keep FFS, and discuss it in the next meeting Recommendations for 2nd round: select one option for 120kHz in FR2 Issue 2-1-2: Define measurement accuracy for CLI-RSSI Tentative agreements: Re-use LAA RSSI accuracy requirements for CLI-RSSI for FR1, and allow 1.5dB relaxation for CLI-RSSI for FR2 Recommendations for 2nd round: N/A Issue 2-1-3: Measurement accuracy for extreme condition and high Io Tentative agreements: - Extreme condition with Io=-70dBm/BW is derived by adding 4.5/3dB for FR1/FR2 to the absolute L1-RSRP accuracy in normal condition. - Normal condition with Io=-50dBm/BW is derived by adding 3.5/3dB for FR1/FR2to the absolute L1-RSRP accuracy in normal condition with Io=-70dBm/BW. - Extreme condition with Io=-50dBm/BW is derived by adding x/xdB for FR1/FR2 to the absolute L1-RSRP accuracy in extreme condition with Io=-70dBm/BW. Candidate options: - Extreme condition with Io=-50dBm/BW - Option 1: x = 2 - Option 2: x = 3 Recommendations for 2nd round: select one option Issue 2-2-1: Test case list Recommendations for 2nd round: need further discussion for DRX test and EN-DC test cases Issue 2-2-2 ~ Issue 2-2-6: Test configuration Tentative agreements: - Sub-test for SRS-RSRP and CLI-RSSI - SRS-RSRP for FR1: 3 sub-tests with different SRS-RSRP and Io levels (similar to SS-RSRP) - SRS-RSRP for FR2: 2 sub-tests with different SRS-RSRP and Io levels (similar to SS-RSRP) - CLI-RSSI : 1 sub-test (similar to LAA RSSI) - BW and SCS for test cases - 15kHz+10MHz, 30kHz+40MHz, 120k+100MHz - timing difference between DL and CLI measurement resource - TC × NTA_offset + 4.67us for FR1 and TC × NTA_offset + 3.67us for FR2 Recommendations for 2nd round: need further discussion for SRS configuration (Issue 2-2-5) and AoA setup for FR2 (Issue 2-2-6) Issue 2-3-1: Minimum SRS_RP Tentative agreements: use Option 1 for FR1 and FR2 Recommendations for 2nd round: LG will provide the CR for minimum SRS_RP |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on CLI RRM performance | LG Electronics |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2001624 | To be revised (revised T-doc number is needed) :The CR will capture 2nd round decision for measurement accuracy values |
| R4-20xxxxx | Request new CR for Issue 2-3-1[Minimum SRS_RP] Title : CR for conditions for cross link interference measurements (section B) Source to WG: LG Electronics Work item code: NR_CLI_RIM-Perf Category : B Release : Rel-16 |

Discussion on 2nd round (if applicable)

Remaining issues for performanc part will be treated on sub-thread with WF (LG).

Companies views’ collection for 2nd round (capture from e-mail reflector)

- Define measurement accuracy for SRS-RSRP for 120kHz in FR2 (slide 2) [LG] We prefer Option 1. The difference is 1.5dB among companies. In general, RAN4 has taken an average approach. We can add [ ] for FR2 120kHz, i.e., [±9]dB. [Huawei] We prefer option 2. In our view 1.5dB is not small for accuracy requirements and for all the other cases the results from companies are much more aligned, so we suggest to have more investigations into it. As accuracy requirements is Perf part, having decision in next meeting would not cause delay to the WI completion. [Nokia] We still think AWGN shall be used when deriving the accuracy value. Since AWGN has been assumed in all test configurations for measurement test cases, it would be too relaxed to use the number based on the worse channel model. The accuracy should be 0.5dB less than current values in the table. <Qualcomm> On the 120kHz we would be fine with either option. Since HW would like more time, we would be fine with deciding on this in the next meeting too. For 30 kHz, we would have actually preferred to go with the HW results which were at 3.5 dB too. I got internal results pretty late last week and they align more with the 3.5 dB accuracy. [LG] For Nokia comments, in my understanding, RRM measurement accuracy is defined based on worst case among all channels in RAN4. I would like to ask companies to double check. Is it Ok to use [ ] for all the accuracy levels except for 120kHz in slide 2? LG is fine to define measurement accuracy for 120kHz in the next meeting with additional simulation results. [Nokia] From the test cases defined for other measurements in 38.133, AWGN is explicitly assumed. Shall we follow it for CLI as well? [LG] AWGN for CLI measurement test should be considered. <Qualcomm> We would be fine with using [] for 120 kHz. As mentioned in our comment above we would also want 15 kHz to be captured as [3-3.5] dB [LG] In this meeting, measurement accuracy level would be captured with [ ]. But it’s better to capture single value not range for 15kHz. Could you provide simulation results in the next meeting? then measurement accuracy values based on provided companies’ simulation results would be finalized. - Measurement accuracy for extreme condition with Io=-50dBm/BW (slide 3) [LG] We prefer to use the same approach as other measurement accuracy for Extreme condition with Io=-50dBm/BW. So, the Option 1 is preferred. (x=2dB) [Huawei] no strong preference, either option is fine for us. <Qualcomm> I was reading the proposal wrong earlier. We are fine with the 2dB too which aligns with other measurement requirements. [LG] Can we agree X=2dB? <Qualcomm> Agree - Test case list for SRS-RSRP and CLI-RSSI (slide 4) [LG] Option 1 is preferred. CLI feature could be supported for DRX/non-DRX and SA/EN-DC. [Huawei] We share same view as LGE that CLI is supported for DRX/non-DRX and SA/EN-DC. On the other hand, we also agree with QC’s comment in the 1st round that number of test cases should be limited. Can we reduce the number of delay test cases e.g. with following combination? [Nokia] CLI in EN-DC has not been discussed in RAN4. It is not clear whether the CLI requirements still apply if the UE applies a constant at the NR side. We need more discussion before adding it to the test cases. About DRX test cases, how does the UE behave if DRX is configured? Would it be sufficient to only consider non-DRX for now? 1 SRS-RSRP measurement delay with non-DRX FR1 EN-DC 2 SRS-RSRP measurement delay with DRX FR1 EN-DC 3 SRS-RSRP measurement delay with non-DRX FR2 EN-DC 4 SRS-RSRP measurement delay with DRX FR2 EN-DC 5 SRS-RSRP measurement delay with non-DRX FR1 SA 6 SRS-RSRP measurement delay with DRX FR1 SA 7 SRS-RSRP measurement delay with non-DRX FR2 SA 8 SRS-RSRP measurement delay with DRX FR2 SA <Qualcomm> We would be fine with the HW proposal since in this scenario the test cases are reduced which was our major concern. To Nokia, why would the EN-DC requirements be any different from SA ones. [LG] LG is fine with Huawei proposal. And For Nokia comments, in my understanding, there is no issue for EN-DC and SA also DRX. Could you provide more detail technical concern? [Nokia] The concern comes from the constant offset applied due to CLI measurement. Would this time shifting bring interruption to LTE side? [LG] I don’t think that there is interruption to LTE. - AoA setup for FR2 (slide 5) [LG] Option 1 is preferred. The reason why using AoA setup#1 is that a UE do not need to sweep Rx beam for CLI measurement resource, and RAN4 has already made agreement for QCL assumption for CLI measurement. So we don’t need to 2 AoA setup, and it just increases test complexity. [Huawei] We share same view as LGE and support option 1. <Qualcomm> From our perspective this is not about beam sweeping but how the real world scenarios would happen. In those cases, the UE would most likely see the CLI interference coming from a different angle than its serving base station. We can leave it TBD for now and decide in the next meeting. [LG] Basically, it will be the best if all test can be done in real world scenarios. However, we cannot... Our intention to use 1 AoA setup is to reduce complexity for test if requirements could be verified. In this meeting, since only LG provided view on AoA setup, we keep two options of the WF, then let’s discuss and decide in the next meeting. - Others <Qualcomm> For the CLI-RSSI part, we would like to check more on how the LAA RSSI number came about. It seems that LAA just re-used the UMTS numbers. So for the time being we are fine to use that as baseline but the actual numbers may need some adjusting. We propose to keep the current proposal as option 1 (with the 1.5 db relaxation in [] and say that other options are not precluded. The other question I have on the RSSI is if this requirement is for AWGN channel or even in fading. [Huawei] On the last slide for SRS configuration, we understand it is also to be used for delay test? Currently it seems only for accuracy test. Also could you please add a bullet that this configuration is used as baseline, and update may be needed as necessary?

Issue 1: Review on the CRs

Companies are encouraged to review the following CRs.

Based on session chairman’s decision, revised and new T-doc number for CRs are assigned.

- CR : R4-2001624 CR on CLI measurement accuracy requirement, Huawei

  - Revised T-doc number is R4-2002254

- CR (new) : R4-2002255 CR for conditions for cross link interference measurements (section B), LG

  - Please find the CR “draft_R4-20xxxxx(CR_CLI_sectionB).docx” in https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_94_e/Inbox/Drafts/%2349_NR_CLI_RIM_RRM

Companies views’ collection for 2nd round

| Company | Comments for Issue 1 |
| --- | --- |
| Nokia, Nokia Shanghai Bell | The accuracy value depends on the 2nd round discussion. |
| LG | For measurement accuracy CR, the accuracy values should be based on the 2nd round conclusion, but I would like know if companies are concerned about measurement accuracy values of tentative agreement for Issue 2-1-1 except for 120kHz. |

Issue 2: Work split for draft CR of performance part

I think that we need following five CRs for performance part

- Minimum SRS-RP in Annex B (LG will try to provide and finalize the CR in this meeting)

- Event triggered reporting test cases for SRS-RSRP

- Event triggered reporting test cases for CLI-RSSI

- Measurement accuracy test cases for SRS-RSRP

- Measurement accuracy test cases for CLI-RSSI

For efficient work for CRs of CLI performance part, please indicate your interest part with your company name if you prefer to work on the CR.

| # | Required CR for performance part | Company |
| --- | --- | --- |
| 1 | Minimum SRS-RP in Annex B | LG (in this meeting if possible) |
| 2 | Event triggered reporting test cases for SRS-RSRP | LG |
| 3 | Event triggered reporting test cases for CLI-RSSI | LG |
| 4 | Measurement accuracy test cases for SRS-RSRP | Huawei |
| 5 | Measurement accuracy test cases for CLI-RSSI | Huawei |

Companies views’ collection for 2nd round

| Company | Other comments for work split for CRs of CLI performance part |
| --- | --- |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2002254 | Agreeable (CR on CLI measurement accuracy requirement, Huawei) |
| R4-2002255 | Agreeable (CR for conditions for cross link interference measurements (section B), LG) |
| R4-2002222 | Agreeable (WF on CLI RRM Performance Requirements, LG) All companies’ comments are captured |