3GPP TSG-RAN4 Meeting #94-e	R4-2002716

# Online, , 24th Feb 2020 - 6th Mar 2020 									Revison of R4-2000415

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0165 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-3: Remove delta Tib and delta Rib for FR1+FR2 CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R16_3BDL_2BUL-Core |  |  |  |  |  | Date: |  |  | 2020-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Since ΔTIB and ΔRIB are independent for FR1 and FR2, there is no reason to include tables for ΔTIB and ΔRIB for CA combinations with FR1 and FR2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the tables for ΔTIB and ΔRIB for CA combinations with FR1 and FR2, and added text pointing to 38.101-1 for FR1 and 38.101-2 for FR2. Rev1 changes ΔTIB and ΔRIB to zero for FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will contain unnecessary tables. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4.2.1, 7.3A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.101-1 CR 0297 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

#### 6.2A.4.2	ΔTIB,c for CA

##### 6.2A.4.2.1	ΔTIB,c for Inter-band CA between FR1 and FR2

ΔTIB,c is independent between FR1 and FR2. For inter-band CA between FR1 and FR2, ΔTIB,c for the FR1 band(s) in TS 38.101-1 [2] applies and ΔTIB,c for the FR2 NR band(s) is set to zero. Otherwise ΔTIB,c is set to zero.

Table 6.2A.4.2.1-1: Void

Table 6.2A.4.2.1-2: Void

Table 6.2A.4.2.1-3: Void

<Next changed section>

## 7.3A	Reference sensitivity for CA

### 7.3A.1	General

For NR CA operation NR single carrier REFSENS requirements defined in TS 38.101-1 [2] and TS 38.101-2 [3] apply to all downlink bands part of NR CA configurations listed in Table 5.2A.1-1unless sensitivity degradation is allowed as defined in clause 7.3A.

### 7.3A.2	Reference sensitivity power level for CA

### 7.3A.3	ΔRIB,c for CA

For the UE which supports inter-band NR CA configuration, the minimum requirement for reference sensitivity in clause 7.3.2, 7.3A2 in TS 38.101-1 [2] and clause 7.3.2, 7.3A.2in TS 38.101-2 [3] shall be increased by the amount given in ΔRIB,c in Tables below. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1GHz, the applicable additional ∆RIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

#### 7.3A.3.1	ΔRIB,c for Inter-band CA between FR1 and FR2

ΔRIB,c is independent between FR1 and FR2. For inter-band CA between FR1 and FR2, ΔRIB,c for the FR1 band(s) from TS 38.101-1 [2] applies and ΔRIB,c for the FR2 NR band(s) is set to zero. Otherwise ΔRIB,c is set to zero.

Table 7.3A.3.1-1: Void

Table 7.3A.3.1-2: Void

Table 7.3A.3.1-3: Void

<End of changes>