3GPP TSG-RAN WG4 Meeting #94-e	R4-2002874

Electronic Meeting, Feb.24th – Mar.6th 2020

Agenda item:			6, 6.1.1

Source:	Moderator (Ericsson)

Title:	Email discussion summary for RAN4#94e_#1_NR_NewRAT_DC

Document for:	Information

# Introduction

This Email discussion concerns Rel-15 maintenance in two areas under AI 6 and AI 6.1.1:

- AI 6: release independent specification of NR UE power classes

- AI 6.1.1: UE power-class restrictions for NE-DC

One set of CRs is submitted against the respective area, no other documents. The aim is to decide on these Rel-15 changes.

# Topic #1: NR UE power class (release independence)

Release-independent specification of a NR UE power class can depend on the duplex mode and the release in which a new power class is introduced.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000557 | Nokia, Nokia Shanghai Bell | TS 38.307 CR 0017 (Cat F) Summary of change: PC2 mapping is removed from FDD and PC1 is added. (One other editorial correction.) |
| R4-2000524 | Nokia, Nokia Shanghai Bell | TS 38.307 CR 0017 (Cat A) |

## Open issues summary

The issue: should release-independent specification of the NR UE power classes 1, 2 and 3 depend on the duplex mode?

### Sub-topic 1-1 Release independent specification of NR UE power classes

The issue: should release-independent specification of the NR UE power classes 1, 2 and 3 depend on the duplex mode?

Issue 1-1: Release independent specification of power classes

- Proposals

  - Option 1: The proposed changes in TS 38.307 CR 0017 (R4-2000557)

  - Option 2: No change

- Recommended WF

  - Option 1: not all power classes specified in Rel-15 are applicable for all duplex modes (FDD, TDD and SUL)

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| OPPO | Sub topic 1-1: OK with option 1. Sub topic 1-2: …. Others: |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| TS 38.307 CR 0017 | Intel: PC1 does not exist for Rel-15 NR. Suggest removing it. Support option 1 in recommended WF |
| TS 38.307 CR 0016 | Intel: This is Cat A CR. Same comments as in Cat F CR. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Tentative agreements: Option 1 (also including PC1 since this is specified for n14 in Rel-16, release independence applies but pending further discussions in the second round). Candidate options: Recommendations for 2nd round: CRs to be agreed |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| TS 38.307 CR 0017 | Will be agreed at this meeting. |
| TS 38.307 CR 0018 | Cat A (tentatively pending discussions in the 2nd round) |

## Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-1: the TS 38.307 CR 0017 is technically correct and should be agreed, but there is an editorial correction that could be made (an error in a table heading). It is proposed to revise the CR to this end. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| TS 38.307 CR 0017 | CR “to be revised” (new tdoc number not needed for Cat A) |
| TS 38.307 CR 0017r1 | CR “agreeable” (R4-2002910) |
| TS 38.307 CR 0018 | Cat A |

# Topic #2: UE power-class restrictions for NE-DC

Restrictions on the configured UE output power may apply for higher power-class operations (e.g. UE Power Class 2) when the UE is configured with NE-DC.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2001227 | OPPO | TS 38.101-3 CR 0198 Summary of change: remove the restriction on the LTE U/D and special-subframe configurations for NE-DC PC2, not applicable for inter-band NE-DC. PC2 is not specified for any Rel-15 NE-DC band combination. |
| R4-2001228 | OPPO | TS 38.101-3 CR 0199 (Cat A) |

## Open issues summary

### Sub-topic 1-1: power-class restrictions for NE-DC

The issue: remove the restriction on the LTE U/D and special-subframe configurations for inter-band NE-DC PC2?

Issue 1-1: power-class restriction for NE-DC

- Proposals

  - Option 1: the proposed changes in TS 38.101-3 CR 0198 (R4-2001227)

  - Option 2: No change

- Recommended WF

  - Option 1: the restriction of the LTE U/D and special-subframe configuration for NE-DC PC2 not relevant/applicable for inter-band NE-DC.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| TS 38.101-3 CR 0198 | Company A |
|  | Company B |
| TS 38.101-3 CR 0199 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Tentative agreements: Option 1, no comments received Candidate options: Recommendations for 2nd round: 2nd round not needed, CRs agreeable |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| TS 38.101-3 CR 0198 | Cat F CR agreeable |
| TS 38.101-3 CR 0199 | Cat A |

## Discussion on 2nd round (not applicable)

## Summary on 2nd round (not applicable)