3GPP TSG-RAN WG4 Meeting #94bis-e                                                    R4-2003126

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-21-11: DC_3-41_n28

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_3-41_n28 according to the request in [1].

# 2. Reference

- RP-200271, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_3-41_n28

### 5.1.x.1		Operating bands for DC

Table 5.1.x.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41_n28 | CA_3-41 | n28 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-41A_n28A | DC_3A_n28A DC_41A_n28A | CA_3A-41A | n28A |
| DC_3A-41C_n28A | DC_3A_n28A DC_41A_n28A DC_41C_n28A | CA_3A-41C | n28A |

### 5.1.x.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there are IMD impacting on the three band in this band combination, as below list:

-	2nd  order IMD generated by dual uplink of Band 41 + Band n28 may also fall into own Rx of band 3, it could reuse the MSD value of DC_3A-7A_n28A.

-     There is no IMD impact to B41 considering the Operator’s spectrum.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_3-41_n28, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_3A-28A-41A

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.3 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41_n28 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 5.1.x.5	REFSENS requirements

Table 5.1.x.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2682 | 10 | 50 | 2682 | N/A | TDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | TDD | IMD2 ¦fB41-fBn28¦ |

<End of Text Proposal>