3GPP TSG-RAN WG4 Meeting #94bis-e                                R4-2003155

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-21-21: DC_1-18-41_n3-n257

Agenda item:			8.7.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-21 to include DC_1A-18A-41A_n3A-n257A and DC_1A-18A-41C_n3A-n257A according to the request in [1].

# 2. Reference

- RP-200080, Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), LG Electronics.

# 3. Text Proposal

<Start of Text Proposal>

## 8.x	DC_1-18-41_n3-n257

### 8.x.1	Operating bands for DC

Table 8.x.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-18-41_n3-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 8.x.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA including FR2 specified in TS 38.101-3.

Table 8.x.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-18A-41A_n3A-n257A | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41A_n257A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 485 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-41C_n3A-n257A | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41C_n3A DC_41A_n257A DC_41C_n257A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 505 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-41A_n3A-n257I | DC_18A_n3A DC_41A_n3A DC_18A_n257A DC_41A_n257A DC_18A_n257I DC_41A_n257I | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 485 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-41C_n3A-n257I | DC_18A_n3A DC_41A_n3A DC_41C_n3A DC_18A_n257A DC_41A_n257A DC_41C_n257A DC_18A_n257I DC_41A_n257I DC_41C_n257I | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 505 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.x.3	Co-existence studies

Co-existence studies of DC_1-18-41_n3-n257 and protected bands are already covered in the constituent fall-back modes.

### 8.x.4	∆TIB and ∆RIB values

For DC_1-18-41_n3-n257, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 8.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18-41_n3-n257 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18-41_n3-n257 | 1 | 0 |
|  | 18 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.x.5	MSD

All MSD requirements for DC_1-18-41_n3-n257 are already covered by constituent fallback modes. No additional MSD requirement is needed.

<End of Text Proposal>