3GPP RAN WG4 Meeting #94-e-Bis	R4-2003290

Electronic Meeting, 20th Apr. – 30th Apr. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RSTD measurement period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RSTD measurement period is missing in 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | RSTD measurement period is introduced in 38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RSTD measurement period is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: 9.X, 9.X.2, 9.X.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.X NR measurements for positioning

9.X.1 Introduction

9.X.2 DL RSTD measurements

9.X.2.1 FDD intra-frequency measurements

When the physical layer cell identities of neighbour cells together with the OTDOA assistance data are provided, the UE shall be able to detect and measure intra-frequency RSTD, specified in TS 38.215 [4], for at least n=16 cells, including the reference cell, on the same carrier frequency f1 as that of the reference cell within TRSTD IntraFreqFDD, NR ms as given below:

TRSTD IntraFreqFDD, NR=TPRS⋅(M-1)+Δ  ms

Where TPRS is the configured periodicity of DL PRS resource in subframe,

M is the number of PRS positioning occasion as defined in table 2.4-1 for all cells to be detected and measured,

Δ  is the measurement time for a single PRS positioning resource which includes the sampling time and the processing time.

Table 2.4-1: Number of PRS positioning occasions within TRSTD IntraFreqFDD, NR

| Positioning subframe configuration period | Number of PRS positioning occasions |  |
| --- | --- | --- |
|  | f1 Note1 | f1 and f2 Note2 |
| ≤160 ms | 16 | 32 |
| >160 ms | 8 | 16 |
| Note 1: When only intra-frequency RSTD measurements are performed over cells belonging to the serving FDD carrier frequency f1. Note 2: When intra-frequency RSTD and inter-frequency RSTD measurements are performed over cells belonging to the serving FDD carrier frequency f1 and one inter-frequency carrier frequency f2, respectively. |  |  |

When the intra-frequency handover occurs while intra-frequency RSTD measurements are being performed then the UE shall complete the ongoing OTDOA measurement session. The UE shall also meet the intra-frequency OTDOA measurement and accuracy requirements. However in this case the RSTD measurement period shall be according to the following expression:

TRSTD IntraFreqFDD, NR,HO=TRSTD IntraFreqFDD, NR+k*TPRS+THO  ms

Where k is the number of times the intra-frequency handover occurs during TRSTD IntraFreqFDD, NR,HO .

THO is the time during which the intra-frequency RSTD measurement may not be possible due to intra-frequency handover; it can be up to [45] ms.