3GPP RAN WG4 Meeting #94-e-bis	R4-2003292

# Electronic Meeting, 20th Apr. – 30th Apr. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RSTD measurement report mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RSTD measurement report mapping is missing in 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | RSTD measurement report mapping is introduced in 38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RSTD measurement report mapping is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections: 10.1.X, 10.1.X.Y1, 10.1.X.Y2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 10.1.X	Reference Signal Time Difference (RSTD)

### 10.1. X.Y1 RSTD measurement accuracy requirement

10.1. X.Y2 RSTD measurement report mapping

The reporting range of RSTD is defined from -985024Tc to 985024Tc with resolution Tc for FR2 and 4Tc for FR1.

The mapping of measured quantity is defined in Table 10.1. X1.Y2-1 and Table 10.1. X1.Y2-2 respectively.

Table 10.1. X1.Y2-1: RSTD report mapping for FR1

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RSTD_000000 | -985024 > RSTD | Tc |
| RSTD_000001 | -985024 RSTD < -985020 | Tc |
| RSTD_000002 | -985020 RSTD < -985016 | Tc |
|  |  | … |
| RSTD_246254 | -12 RSTD -8 | Tc |
| RSTD_246255 | -8 RSTD -4 | Tc |
| RSTD_246256 | -4 RSTD 0 | Tc |
| RSTD_246257 | 0 < RSTD 4 | Tc |
| RSTD_246258 | 4 < RSTD 8 | Tc |
| RSTD_246259 | 8 < RSTD 12 | Tc |
| … | … | … |
| RSTD_492511 | 985016 < RSTD 985020 | Tc |
| RSTD_492512 | 985020 < RSTD 985024 | Tc |
| RSTD_492513 | 985024 < RSTD | Tc |

Table 10.1. X1.Y2-2: RSTD report mapping for FR2

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RSTD_000000 | -985024 > RSTD | Tc |
| RSTD_000001 | -985024 RSTD < -985023 | Tc |
| RSTD_000002 | -985023 RSTD < -985022 | Tc |
|  |  | … |
| RSTD_985022 | -3 RSTD -2 | Tc |
| RSTD_985023 | -2 RSTD -1 | Tc |
| RSTD_985024 | -1 RSTD 0 | Tc |
| RSTD_985025 | 0 < RSTD 1 | Tc |
| RSTD_985026 | 1 < RSTD 2 | Tc |
| RSTD_985027 | 2 < RSTD 3 | Tc |
| … | … | … |
| RSTD_1970047 | 985022 < RSTD 985023 | Tc |
| RSTD_1970048 | 985023 < RSTD 985024 | Tc |
| RSTD_1970049 | 985024 < RSTD | Tc |