3GPP TSG-RAN WG4 Meeting # 94-e-Bis                                                       R4-2003316

Electronic Meeting, 20 – 30 Apr., 2020

Title:	LS on RAN4 IAB-MT feature list agreement

Response to:	[]

Release:	Rel-16

Work Item:	NR_IAB-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Huiping Shan

E-mail Address:	shanhuiping@catt.cn

Attachments:	none

1. Overall Description

RAN4 has discussed RF and RRM features in IAB-MT feature list based on TR 38.822 vf01 and would like to inform RAN2 of their agreement.

RAN4 made the following agreements after the discussion,

- 1-6 (pi/2-BPSK for PUSCH) and 1-7 (pi/2-BPSK for PUCCH format 3/4) are changed to optional for FR2.

- 2-1 (Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC) and 2-8 (UE power class) need to be considered by RAN2. IAB-MT would declare its supported channel bandwidth and its maximum output power.

- 2-10 (Multiple frequency band indication) and 2-12 (Multiple NS/P-Max) are not needed.

- 2-16 (PA architectures for intra-band EN-DC) and 2-17 (PA architectures for intra-band UL CA) can be set absent.

Specially, RAN4 would like RAN2 to pay attention to RAN4 views on 2-8 (UE power class). RAN4 will define IAB-MT class, but the maximum output power is declared by manufacturer and IAB-MT is allowed to transmit less power than the upper power limit specified in TS 38.714. RAN2 needs to evaluate the signalling needed due to this newly introduced behaviour.

2. Actions

To RAN2 group:

Action: RAN4 respectfully asks RAN2 to take the above into consideration in its further work.

3. Date of Next TSG-RAN WG4 Meetings:

RAN4 Meeting #95e	25 May – 05 June 2020	E-meeting

RAN4 Meeting #96		24 – 28 Aug. 2020		Toulouse, FR

4. Reference

- TR 38.822, vf01