3GPP TSG-RAN4 Meeting #94bis-e	R4-2003402

# Online, 20th - 30th Apr 2020

Title:	LS on RSTD measurement report mapping for NR positioning

Reply To:

Release:	Rel-16

Work Item:	NR_pos-Core

Source:	RAN WG 4

To:	RAN WG 1, RAN WG 2

Contact Person:

Name:	Jie Cui

E-mail Address:	jie_cui@apple.com

Attachments:	N.A.

1. Overall Description:

RAN4 has discussion on the RSTD measurement report mapping for NR positioning, and following conclusions are achieved,

- RAN4 conclusion on Parameter k (T=Tc2k)

  - UE reports granularity capability of k to network

  - Network is expected to configure k equal to or larger than UE reported k for the RSTD reporting

- The RSTD report mapping table and relative quantity mapping table for FR1 is designed as in table x-1, x-2 (k=4), and x-3 (k=3).

For FR1, the UE shall report a reference quantity based on Table x-1 and a relative quantity defined in Table x-2, so that the difference between the measured RSTD quantity and the lower bound of the corresponding range from Table x-1 is between and + resolutionStep. resolutionStep is 16Tc for 15kHz SCS, 8Tc for 30kHz SCS and 8Tc for 60kHz in the range from RSTD_2260 to RSTD_10451, and 32Tc for 15kHz SCS, 16Tc for 30kHz SCS and 16Tc SCS for 60kHz in the range from RSTD_0000 to RSTD_2259 or in the range from RSTD_10452 to RSTD_12711. The applicability of relative quantity value is defined in Table x-2 and Table x-3. Table x-1 RSTD report mapping for FR1 Reported Value; Measured Quantity Value; Unit. RSTD_0000; -15391 > RSTD; 64Tc. RSTD_0001; -15391 RSTD < -15386; 64Tc. …; …. RSTD_2258; -4106 RSTD < -4101; 64Tc. RSTD_2259; -4101 RSTD < -4096; 64Tc. RSTD_2260; -4096 RSTD < -4095; 64Tc. RSTD_2261; -4095 RSTD < -4094; 64Tc. …. RSTD_6353; -3 RSTD -2; 64Tc. RSTD_6354; -2 RSTD -1; 64Tc. RSTD_6355; -1 RSTD 0; 64Tc. RSTD_6356; 0 < RSTD 1; 64Tc. RSTD_6357; 1 < RSTD 2; 64Tc. RSTD_6358; 2 < RSTD 3; 64Tc. …; …; …. RSTD_10450; 4094 < RSTD 4095; 64Tc. RSTD_10451; 4095 < RSTD 4096; 64Tc. RSTD_10452; 4096 < RSTD 4101; 64Tc. RSTD_10453; 4101 < RSTD 4106; 64Tc. …; …; …. RSTD_12709; 15381 < RSTD 15386; 64Tc. RSTD_12710; 15386 < RSTD 15391; 64Tc. RSTD_12711; 15391 < RSTD; 64Tc. Table x-2 Relative quantity mapping for SCS=15kHz in FR1 (k=4) Reported Relative Quantity Value; Measured Relative Quantity Value,; Unit; Applicable SCS(kHz); Applicable RSTD report range in table x-1. RSTD_delta_0; 0; Tc; 15; all RSTD reported values in table x-1. RSTD_delta_1; 16; Tc; 15; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_2; 32; Tc; 15; all RSTD reported values in table x-1. RSTD_delta_3; 48; Tc; 15; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_4; 64; Tc; 15; all RSTD reported values in table x-1. RSTD_delta_5; 96; Tc; 15; all RSTD reported values in table x-1. RSTD_delta_6; 128; Tc; 15; all RSTD reported values in table x-1. RSTD_delta_7; 160; Tc; 15; all RSTD reported values in table x-1. RSTD_delta_8; 192; Tc; 15; all RSTD reported values in table x-1. RSTD_delta_9; 224; Tc; 15; all RSTD reported values in table x-1. RSTD_delta_10; 256; Tc; 15; all RSTD reported values in table x-1. RSTD_delta_11; 288; Tc; 15; all RSTD reported values in table x-1. Table x-3 Relative quantity mapping for SCS=30/60kHz in FR1 (k=3) Reported Relative Quantity Value; Measured Relative Quantity Value,; Unit; Applicable SCS(kHz); Applicable RSTD report range in table x-1. RSTD_delta_0; 0; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_1; 8; Tc; 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_2; 16; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_3; 24; Tc; 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_4; 32; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_5; 40; Tc; 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_6; 48; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_7; 56; Tc; 30/60; from RSTD_2260 to RSTD_10451 in table x-1. RSTD_delta_8; 64; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_9; 80; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_10; 96; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_11; 112; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_12; 128; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_13; 144; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_14; 160; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_15; 176; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_16; 192; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_17; 208; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_18; 224; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_19; 240; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_20; 256; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_21; 272; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_22; 288; Tc; 30/60; all RSTD reported values in table x-1. RSTD_delta_23; 304; Tc; 30/60; all RSTD reported values in table x-1.

- The RSTD report mapping table and relative quantity mapping table for FR2 is designed as in table y-1, y-2 (k=1), and y-3 (k=0)

For FR2, the UE shall report a reference quantity based on Table y-1 and a relative quantity defined in Table y-2, so that the difference between the measured RSTD quantity and the lower bound of the corresponding range from Table y-1 is between and + resolutionStep. resolutionStep is 2Tc for 60kHz SCS and 1Tc for 120kHz SCS in the range from RSTD_2260 to RSTD_10451, and 4Tc for 60kHz SCS and 2Tc for 120kHz SCS in the range from RSTD_0000 to RSTD_2259 or in the range from RSTD_10452 to RSTD_12711. The applicability of relative quantity value is defined in Table y-2 and Table y-3. Table y-1 RSTD report mapping for FR2 Reported Value; Measured Quantity Value; Unit. RSTD_0000; -15391 > RSTD; 8Tc. RSTD_0001; -15391 RSTD < -15386; 8Tc. …; …. RSTD_2258; -4106 RSTD < -4101; 8Tc. RSTD_2259; -4101 RSTD < -4096; 8Tc. RSTD_2260; -4096 RSTD < -4095; 8Tc. RSTD_2261; -4095 RSTD < -4094; 8Tc. …. RSTD_6353; -3 RSTD -2; 8Tc. RSTD_6354; -2 RSTD -1; 8Tc. RSTD_6355; -1 RSTD 0; 8Tc. RSTD_6356; 0 < RSTD 1; 8Tc. RSTD_6357; 1 < RSTD 2; 8Tc. RSTD_6358; 2 < RSTD 3; 8Tc. …; …; …. RSTD_10450; 4094 < RSTD 4095; 8Tc. RSTD_10451; 4095 < RSTD 4096; 8Tc. RSTD_10452; 4096 < RSTD 4101; 8Tc. RSTD_10453; 4101 < RSTD 4106; 8Tc. …; …; …. RSTD_12709; 15381 < RSTD 15386; 8Tc. RSTD_12710; 15386 < RSTD 15391; 8Tc. RSTD_12711; 15391 < RSTD; 8Tc. Table y-2 Relative quantity mapping for SCS=60kHz in FR2 (k=1) Reported Relative Quantity Value; Measured Relative Quantity Value,; Unit; Applicable SCS(kHz); Applicability. RSTD_delta_0; 0; Tc; 60; all RSTD reported values in table y-1. RSTD_delta_1; 2; Tc; 60; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_2; 4; Tc; 60; all RSTD reported values in table y-1. RSTD_delta_3; 6; Tc; 60; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_4; 8; Tc; 60; all RSTD reported values in table y-1. RSTD_delta_5; 12; Tc; 60; all RSTD reported values in table y-1. RSTD_delta_6; 16; Tc; 60; all RSTD reported values in table y-1. RSTD_delta_7; 20; Tc; 60; all RSTD reported values in table y-1. RSTD_delta_8; 24; Tc; 60; all RSTD reported values in table y-1. RSTD_delta_9; 28; Tc; 60; all RSTD reported values in table y-1. RSTD_delta_10; 32; Tc; 60; all RSTD reported values in table y-1. RSTD_delta_11; 36; Tc; 60; all RSTD reported values in table y-1. Table y-3 Relative quantity mapping for SCS=120kHz in FR2 (k=0) Reported Relative Quantity Value; Measured Relative Quantity Value,; Unit; Applicable SCS(kHz); Applicability. RSTD_delta_0; 0; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_1; 1; Tc; 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_2; 2; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_3; 3; Tc; 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_4; 4; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_5; 5; Tc; 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_6; 6; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_7; 7; Tc; 120; from RSTD_2260 to RSTD_10451 in table y-1. RSTD_delta_8; 8; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_9; 10; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_10; 12; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_11; 14; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_12; 16; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_13; 18; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_14; 20; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_15; 22; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_16; 24; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_17; 26; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_18; 28; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_19; 30; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_20; 32; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_21; 34; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_22; 36; Tc; 120; all RSTD reported values in table y-1. RSTD_delta_23; 38; Tc; 120; all RSTD reported values in table y-1.

2. Actions:

To RAN WG1 and WG2 group.

ACTION: 	RAN4 kindly asks RAN1 and RAN2 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 #95-e		  25th - 29th May 2020		Emeeting

TSG-RAN4 #96		  24th - 28th Aug 2020		Toulouse , FR