3GPP TSG-RAN4 #94bis-e 	R4-2003423

Online, 20th – 30th Apr. 2020

Source:	Charter Communications

Title:	TP for TR 38.716-02-00 for CA_n48-n46

Agenda item:			6.1.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal to include TP for TR 38.716-02-00 for CA_n48-n46 as defined in WID [1] and captured on revised WID [2].

# 2. Text Proposal

# ---Start of changes---

## 6.x	n46-n48

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n46 and n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz |  |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n46 and n48

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz |  | 25 MHz | 30 MHz |  | 40 MHz |  | 50 MHz |  | 60 MHz |  | 80 MHz |  |  | 90 MHz |  |  |  | 100 MHz |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n46A-n48A | CA_n46A-n48A | N46 | 15 |  | Yes6 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | 30 |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | [Yes] |  |  |  |  | [Yes] |  |  |  | [Yes] |  | [Yes] |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N48 | 15 | Yes5 | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  | Yes6 |  |  | Yes6,4 |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  | Yes6 |  |  | Yes6,4 |  |  |  |  |  |
| CA_n48A-n46B | CA_n48A-n46A | N46 | 15 |  | Yes6 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | 30 |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | [Yes] |  |  |  |  | [Yes] |  |  |  | [Yes] |  |  | [Yes] |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N48 | 15 | Yes5 | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  |  | Yes6 |  |  |  |  | Yes6,4 |  |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  |  | Yes6 |  |  |  |  | Yes6,4 |  |  |  |
| CA_n48A-n46C | CA_n48A-n46A | N46 | 15 |  | Yes6 |  | Yes |  |  | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | [Yes] |  |  |  |  | [Yes] |  |  |  | [Yes] |  |  | [Yes] |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N48 | 15 | Yes5 | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  |  | Yes6 |  |  |  |  | Yes6,4 |  |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  |  | Yes6 |  |  |  |  | Yes6,4 |  |  |  |
| CA_n48A-n46D | CA_n48A-n46A | N46 | 15 |  | Yes6 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | 30 |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | [Yes] |  |  |  |  | [Yes] |  |  |  | [Yes] |  |  | [Yes] |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N48 | 15 | Yes5 | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  |  | Yes6 |  |  |  |  | Yes6,4 |  |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  |  | Yes6 |  |  |  |  | Yes6,4 |  |  |  |
| CA_n48A-n46E | CA_n48A-n46A | N46 | 15 |  | Yes6 |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  | 30 |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  | [Yes] |  |  |  |  | [Yes] |  |  |  | [Yes] |  |  | [Yes] |  |  |  |  |  |  |  |  |  |  |  |
|  |  | N48 | 15 | Yes5 | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  |  | Yes6 |  |  |  |  | Yes6,4 |  |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes |  |  |  |  | Yes |  | Yes6 |  | Yes6 |  |  |  |  |  | Yes6 |  |  |  |  | Yes6,4 |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3		UE co-existence studies

Table 6.x.1.3-1 lists up to 7th harmonics for n46A-n48A.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  |  | 7th Harmonic |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |  | UL Low Band Edge | UL High Band Edge |  | UL Low Band Edge | UL High Band Edge |  |
| n46 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 | 20600 | 23700 | 25750 | 29625 |  | 30900 | 35550 |  | 36050 | 41475 |  |
| n48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |  | 21300 | 22200 |  | 24850 | 25900 |  |

#### 6.x.1.4		∆TIB and ∆RIB values

For CA_n46-n48, the TIB,c and RIB,c values are derived from LTE combination CA_1-3 and are given in the tables below.

# Table 6.x.1.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n46-n48 | n46 | 0 |
|  | n48 | 0.8 |

# Table 6.x.1.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n46-n48 | n46 | 0 |
|  | n48 | 0.5 |

#### 6.x.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 6.x.1.3 there are no harmonics issues.

### 6.x.2	Specific for 2 bands UL CA

#### 6.x.2.1		UE co-existence studies

Table 6.x.2.1-1 lists Band n46 + Band n48 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

# Table 6.x.2.1-1: Band n46 and Band n48 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) |  | 5150 |  | 5925 |  | 3550 |  | 3700 |  |
| 2nd harmonics frequency limits |  | 2* fy_low |  | 2* fy_high |  | 2*fx_low |  | 2*fx_high |  |
| 2nd harmonics frequency limits (MHz) |  | 10300 |  | 11850 |  | 7100 |  | 7400 |  |
| 3rd harmonics frequency limits |  | 3* fy_low |  | 3* fy_high |  | 3*fx_low |  | 3*fx_high |  |
| 3rd harmonics frequency limits (MHz) |  | 15450 |  | 17775 |  | 10650 |  | 11100 |  |
| 4th harmonics frequency limits |  | 4* fy_low |  | 4* fy_high |  | 4*fx_low |  | 4*fx_high |  |
| 4th harmonics frequency limits (MHz) |  | 20600 |  | 23700 |  | 14200 |  | 14800 |  |
| 5th harmonics frequency limits |  | 5* fy_low |  | 5* fy_high |  | 5*fx_low |  | 5*fx_high |  |
| 5th harmonics frequency limits (MHz) |  | 25750 |  | 29625 |  | 17750 |  | 18500 |  |
| 6th harmonics frequency limits |  | 6* fy_low |  | 6* fy_high |  | 6*fx_low |  | 6*fx_high |  |
| 6th harmonics frequency limits (MHz) |  | 30900 |  | 35550 |  | 21300 |  | 22200 |  |
| 7th harmonics frequency limits |  | 7* fy_low |  | 7* fy_high |  | 7*fx_low |  | 7*fx_high |  |
| 7th harmonics frequency limits (MHz) |  | 36050 |  | 41475 |  | 24850 |  | 25900 |  |
| 2nd order IMD products |  | ¦fy_high – fx_low¦ |  | ¦fy_low – fx_high¦ |  | ¦fy_low + fx_low¦ |  | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  | 1450 |  | 2375 |  | 8700 |  | 9625 |  |
| 3rd order IMD products |  | ¦fy_high – 2*fx_low¦ |  | ¦fy_low – 2*fx_high¦ |  | ¦2*fy_low – fx_high¦ |  | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) |  | 6600 |  | 8300 |  | 1175 |  | 2250 |  |
| 3rd order IMD products |  | ¦2*fx_low + fy_low¦ |  | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ |  | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) |  | 13850 |  | 15550 |  | 12250 |  | 13325 |  |
| Two-tone 4th order IMD products |  | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high – 2*fy_low¦ |  | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |
| IMD frequency limits (MHz) |  | 2900 |  | 4750 |  | 17400 |  | 19250 |  |
| Two-tone 4th order IMD products |  | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ |  | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 9525 |  | 14225 |  | 4725 |  | 5950 |  |
| Two-tone 4th order IMD products |  | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high +1* fy_high¦ |  | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 19000 |  | 21475 |  | 15800 |  | 17025 |  |
| Two-tone 5th order IMD products |  | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ |  | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 9650 |  | 10500 |  | 20150 |  | 16900 |  |
| Two-tone 5th order IMD products |  | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ |  | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 19350 |  | 20725 |  | 24150 |  | 27400 |  |
| Two-tone 5th order IMD products |  | ¦2*fx_low – 3*fy_high¦ |  | ¦2*fx_high – 3*fy_low¦ |  | ¦2*fy_low – 3*fx_high¦ |  | ¦2*fy_high – 3*fx_low¦ |  |
| IMD frequency limits (MHz) |  | 800 |  | 1200 |  | 10675 |  | 8050 |  |
| Two-tone 5th order IMD products |  | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ |  | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |  |
| IMD frequency limits (MHz) |  | 20950 |  | 22950 |  | 22550 |  | 25175 |  |

Based on Table 6.x.2.1-1 there are no IMD issues affecting own Rx frequencies of either band n46 or band n48.

Table 6.x.2.1-2 lists the protected bands required for the 2UL bands CA configuration as to be used in Table 6.5A.3.2.3-1 of TS 38.101-1

# Table 6.x.2.1-2: Protected bands for the 2UL bands CA configuration

| E-UTRA and NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n48_n46 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

##### 7.3x.x.x.1 Reference sensitivity exceptions due to UL harmonic interference for NR-CA in NR FR1

Table 7.3x.x.x.1-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR-CA in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n48 |  | n46 | - | 16.2 | - | 13.5 | - | - | 10.9 | - | 9.4 | 8.7 | - |  |

Table 7.3x.x.x.1-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR-CA

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n48 | n46 | 30 |  | 270 |  | 270 |  |  | 270 |  | 270 | 270 |  |  |

---End of changes---

# Reference

[1] 		RP-192615, “Band combination proposals for NR-U WI in Rel-16”, Ericsson

# [2] RP-200168, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)”, ZTE