3GPP TSG-RAN WG4 Meeting #94-e-Bis		R4-2003464

Electronic meeting, 20 – 30 April, 2020

Agenda item:	8.18.2

Source:	CableLabs

Title:	Summary of channel raster analysis in DSS (band 48/n48)

Document for:	Approval

# Introduction

RAN#86 meeting approved WID RP-193213 targeting “Channel raster: Confirm that NR channel raster can be aligned with LTE center frequencies.” Two NR channel raster options, 100 and 300 kHz, were discussed in RAN4 #94-e meeting but no agreement was achieved. A third and fourth raster option of 15 and 30 kHz are in the current draft of the standard for band n48. A WF for further study was proposed. In this contribution, we provide a brief summary of the of DSS in band 48/n48 channel raster challenges and makes appropriate observations and proposals.

Based on the analysis presented in companion document R4-2003462, we find the following high level outcomes for each channel raster under analysis:

1: 15 kHz Channel Raster

Requires further study to see if any of the issues found with other raster options are mitigated by using 15 kHz.

2: 30 kHz Channel Raster

Requires no change to existing specification content. However, potential use of non-standard channel bandwidth may require further study.

3: 100 kHz Channel Raster

It breaks the NR release 15/16 specifications (e.g. ARFCN and GSCN compliance).

4: 300 kHz Channel Raster

It breaks the NR release 15/16 specifications (e.g. GSCN compliance). However, potential use of non-standard channel bandwidth may require further study.

# Observations

Based on the analysis in our companion document R4-2003462, our observations are:

Observation 1: NR channels are not defined by center frequency.

Observation 2: The proposed 100 kHz channel raster for band n48 does not align with the Δf_global 15 kHz global raster defined for frequencies above 3 GHz.

Observation 3: The proposed 100 kHz channel raster for band n48 does not align with existing GSCN Index values and/or SSB definitions such that each FCC PAL 10 MHz could contain an SSB.

Observation 4: The proposed 300 kHz channel raster for band n48 does not align with existing GSCN Index values and/or SSB definitions such that each FCC PAL 10 MHz could contain an SSB.

Observation 5: Using the 30 kHz channel raster is a reasonable compromise to match the NR channel BW with the FCC PAL channel definition. Further study may be required to clarify some aspects (e.g. non-standard channel BW).

Observation 6: NR channels based on both 30 kHz (or 15 kHz) channel raster can be aligned with LTE channels and the FCC PAL channel definitions.

Observation 7: Nominal channel spacing formulas support configuring Point A NR-ARFCN to align with LTE and FCC PAL channel edge frequencies.

Observation 8: Using the 300 kHz channel raster, a non-standard channel bandwidth (e.g. 23 PRBs) may require further study to match an NR channel with FCC PAL channel definitions using valid values for signal parameters.

# Conclusions

Proposal 1: The 15/30 kHz channel raster for band n48 supports NR/LTE channel allocations accordingly with PAL FCC definitions.

Proposal 2: Further study may be required on 15 kHz channel configurations.

Proposal 3: Further study may be required on 30 kHz channel configurations (e.g. non standard channel BW support).