3GPP TSG-RAN WG4 Meeting #94-e-Bis                       	            R4-2003499

Electronic Meeting, 20 – 30 Apr., 2020

Agenda Item:	6.4.5.3

Source:	MediaTek Inc.

Title:	Link-level simulation results for NR V2X SL L1-RSRP Measurements

Document for:		Discussion

Introduction

In this contribution, we provide the SL L1-RSRP simulation results based on the simulation assumptions proposed in [1].

Simulation Results

PSCCH-RSRP

The simulation results of PSCCH RSRP with different channel type and SCS at SNR=-6dB, -4dB, -2dB are as followings.

Table 1, Table 2, Table 3 provides the delta PSCCH-RSRP statistic based on different SNR and different channel types with SyncRef UE SCS=15KHz, 30KHz and 60KHz.

Table 1: delta RSRP based on 1 samples(SCS = 15KHz)

|  |  | CDF of delta RSRP (dB) |  |  | Absolute Accuracy (dB) | Relative Accuracy (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel Type | SNR (dB) | 95% | 50% | 5% |  |  |
| AWGN | -6 | 2.49 | 0.67 | -1.38 | 2.49 | 1.82 |
|  | -4 | 1.93 | 0.36 | -1.41 | 1.93 | 1.57 |
|  | -2 | 1.48 | 0.17 | -1.25 | 1.48 | 1.31 |
| TDL-C (30ns, 1400Hz) | -6 | 5.49 | 0.49 | -1.80 | 5.49 | 2.29 |
|  | -4 | 3.36 | 0.05 | -1.87 | 3.36 | 1.92 |
|  | -2 | 2.05 | -0.16 | -1.78 | 2.05 | 1.62 |
| TDL-C (100ns,300Hz) | -6 | 5.32 | 0.36 | -1.92 | 5.32 | 2.28 |
|  | -4 | 3.17 | -0.03 | -1.93 | 3.17 | 1.9 |
|  | -2 | 1.86 | -0.22 | -1.83 | 1.86 | 1.61 |

Table 2: delta RSRP based on 1 samples(SCS = 30KHz)

|  |  | CDF of delta RSRP (dB) |  |  | Absolute Accuracy (dB) | Relative Accuracy (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel Type | SNR (dB) | 95% | 50% | 5% |  |  |
| AWGN | -6 | 2.49 | 0.67 | -1.38 | 2.49 | 1.82 |
|  | -4 | 1.93 | 0.36 | -1.41 | 1.93 | 1.57 |
|  | -2 | 1.48 | 0.17 | -1.25 | 1.48 | 1.31 |
| TDL-C (30ns, 1400Hz) | -6 | 5.72 | 0.45 | -1.86 | 5.72 | 2.31 |
|  | -4 | 3.53 | 0 | -1.90 | 3.53 | 1.9 |
|  | -2 | 2.15 | -0.19 | -1.81 | 2.15 | 1.62 |
| TDL-C (100ns,300Hz) | -6 | 4.72 | 0.35 | -2.02 | 4.72 | 2.37 |
|  | -4 | 2.88 | -0.09 | -2.08 | 2.88 | 1.99 |
|  | -2 | 1.64 | -0.32 | -2.02 | 1.64 | 1.7 |

Table 3: delta RSRP based on 1 samples(SCS = 60KHz)

|  |  | CDF of delta RSRP (dB) |  |  | Absolute Accuracy (dB) | Relative Accuracy (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel Type | SNR (dB) | 95% | 50% | 5% |  |  |
| AWGN | -6 | 2.49 | 0.67 | -1.38 | 2.49 | 1.82 |
|  | -4 | 1.93 | 0.36 | -1.41 | 1.93 | 1.57 |
|  | -2 | 1.48 | 0.17 | -1.25 | 1.48 | 1.31 |
| TDL-C (30ns, 1400Hz) | -6 | 5.17 | 0.37 | -1.98 | 5.17 | 2.35 |
|  | -4 | 3.20 | -0.03 | -2.01 | 3.20 | 1.98 |
|  | -2 | 1.86 | -0.25 | -1.88 | 1.88 | 1.63 |
| TDL-C (100ns,300Hz) | -6 | 3.55 | 0.19 | -2.07 | 3.55 | 2.26 |
|  | -4 | 2.07 | -0.21 | -2.16 | 2.16 | 1.95 |
|  | -2 | 1.21 | -0.42 | -2.10 | 2.10 | 1.63 |

From the tables, we can conclude the following observations:

Observation 1: Based on the simulation, it can be seen that the absolute accuracy of PSCCH RSRP can be achieved at SNR=-4dB before adding RF impairment margin:

- AWGN: 2dB

- TDL-C 30ns 1400Hz: 3.6dB

- TDL-C 100ns 300Hz: 3.2dB

Proposal 1: Side condition of SNR = -4dB can be used for V2X PSCCH-RSRP measurement with absolute accuracy of ±6.0dB.

PSSCH-RSRP

The simulation results of PSSCH RSRP with different channel type and SCS at SNR=-6dB, -4dB, -2dB are as followings.

Table 4, 5, 6 provides the delta PSSCH-RSRP statics based on different SNR and different channel types with SyncRef UE SCS=15KHz, 30KHz and 60KHz.

Table 4: delta RSRP based on 1 samples(SCS = 15KHz)

|  |  | CDF of delta RSRP (dB) |  |  | Absolute Accuracy (dB) | Relative Accuracy (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel Type | SNR (dB) | 95% | 50% | 5% |  |  |
| AWGN | -6 | 1.99 | 0.63 | -0.86 | 1.99 | 1.36 |
|  | -4 | 1.53 | 0.40 | -0.78 | 1.53 | 1.13 |
|  | -2 | 1.17 | 0.24 | -0.70 | 1.17 | 0.93 |
| TDL-C (30ns, 1400Hz) | -6 | 2.59 | 0.32 | -1.33 | 2.59 | 1.65 |
|  | -4 | 1.68 | 0.08 | -1.28 | 1.68 | 1.36 |
|  | -2 | 1.15 | -0.05 | -1.15 | 1.15 | 1.1 |
| TDL-C (100ns,300Hz) | -6 | 3.42 | 0.26 | -1.43 | 3.42 | 1.69 |
|  | -4 | 2.03 | 0.03 | -1.34 | 2.03 | 1.37 |
|  | -2 | 1.35 | -0.08 | -1.22 | 1.35 | 1.14 |

Table 5: delta RSRP based on 1 samples(SCS = 30KHz)

|  |  | CDF of delta RSRP (dB) |  |  | Absolute Accuracy (dB) | Relative Accuracy (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel Type | SNR (dB) | 95% | 50% | 5% |  |  |
| AWGN | -6 | 1.99 | 0.63 | -0.86 | 1.99 | 1.36 |
|  | -4 | 1.53 | 0.40 | -0.78 | 1.53 | 1.13 |
|  | -2 | 1.17 | 0.24 | -0.70 | 1.17 | 0.93 |
| TDL-C (30ns, 1400Hz) | -6 | 3.11 | 0.31 | -1.40 | 3.11 | 1.71 |
|  | -4 | 1.95 | 0.08 | -1.32 | 1.95 | 1.4 |
|  | -2 | 1.31 | -0.04 | -1.19 | 1.31 | 1.15 |
| TDL-C (100ns,300Hz) | -6 | 2.98 | 0.28 | -1.45 | 2.98 | 1.73 |
|  | -4 | 1.89 | 0.04 | -1.39 | 1.89 | 1.43 |
|  | -2 | 1.18 | -0.07 | -1.27 | 1.18 | 1.2 |

Table 6: delta RSRP based on 1 samples(SCS = 60KHz)

|  |  | CDF of delta RSRP (dB) |  |  | Absolute Accuracy (dB) | Relative Accuracy (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Channel Type | SNR (dB) | 95% | 50% | 5% |  |  |
| AWGN | -6 | 1.99 | 0.63 | -0.86 | 1.99 | 1.36 |
|  | -4 | 1.53 | 0.40 | -0.78 | 1.53 | 1.13 |
|  | -2 | 1.17 | 0.24 | -0.70 | 1.17 | 0.93 |
| TDL-C (30ns, 1400Hz) | -6 | 3.20 | 0.32 | -1.39 | 3.20 | 1.71 |
|  | -4 | 1.99 | 0.08 | -1.32 | 1.99 | 1.4 |
|  | -2 | 1.35 | -0.032 | -1.20 | 1.35 | 1.168 |
| TDL-C (100ns,300Hz) | -6 | 2.37 | 0.13 | -1.56 | 2.37 | 1.69 |
|  | -4 | 1.51 | -0.07 | -1.55 | 1.55 | 1.48 |
|  | -2 | 0.99 | -0.18 | -1.39 | 1.39 | 1.17 |

From the tables, we can conclude the following observations:

Observation 2: Based on the simulation, it can be seen that the absolute accuracy of PSSCH RSRP can be achieved at SNR=-4dB before adding RF impairment margin:

- AWGN: 1.6dB

- TDL-C 30ns 1400Hz: 2.0dB

- TDL-C 100ns 300Hz: 2.0dB

Proposal 2: Side condition of SNR = -4dB can be used for V2X PSSCH-RSRP measurement with absolute accuracy of ±4.5dB.

L1 SL-RSRP supports both PSSCH and PSCCH DMRS RSRP measurement. The higher layer parameter RSforSensing will be used to indicate UE use PSSCH-RSRP or PSCCH-RSRP to reselect resources. Based on the simulation results above, it can be seen that the accuracy of PSSCH-RSRP is much better than PSCCH-RSRP. It’s too pessimistic to use united accuracy for both PSCCH-RSRP and PSSCH-RSRP. Thus, the accuracy requirement for both PSSCH-RSRP and PSCCH-RSRP measurements shall be defined independently.

Proposal 3: Introduce L1 SL-RSRP measurement requirements for both PSSCH-RSRP and PSCCH-RSRP.

Summary

In this paper, we give the simulation results for both PSSCH-RSRP and PSCCH-RSRP measurement for reference. Based on the simulation results, we list some observations and proposals as followings:

Observation 1: Based on the simulation, it can be seen that the absolute accuracy of PSCCH RSRP can be achieved at SNR=-4dB before adding RF impairment margin:

- AWGN: 2dB

- TDL-C 30ns 1400Hz: 3.6dB

- TDL-C 100ns 300Hz: 3.2dB

Observation 2: Based on the simulation, it can be seen that the absolute accuracy of PSSCH RSRP can be achieved at SNR=-4dB before adding RF impairment margin:

- AWGN: 1.6dB

- TDL-C 30ns 1400Hz: 2.0dB

- TDL-C 100ns 300Hz: 2.0dB

Proposal 1: Side condition of SNR = -4dB can be used for V2X PSCCH-RSRP measurement with absolute accuracy of ±6.0dB.

Proposal 2: Side condition of SNR = -4dB can be used for V2X PSSCH-RSRP measurement with absolute accuracy of ±4.5dB.

Proposal 3: Introduce L1 SL-RSRP measurement requirements for both PSSCH-RSRP and PSCCH-RSRP.

Reference

- R4-2002232, “Simulation assumption of PSSCH-RSRP and PSCCH-RSRP measurement”, LG Electronics