3GPP TSG-RAN WG4 Meeting #94bis-e                             R4-2003535

Online, April 20th – May 1st, 2020

Source: 	MediaTek Inc.

Title: 	Minimum peak EIRP and REFSENS link budgets for new FR2 FWA UE

Agenda Item:	10.1.1

Document for:	Discussion

# Introduction

A new WID on “Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258” was approved in last plenary e-meeting [1]. One of the objective is to define UE RF requirements for operating bands n257 and n258 with UL TRP confined to below 23 dBm to constrain UL co-channel interference and maximum EIRP limited to below 43 dB to comply with regional regulation. On the other hand, the minimum peak EIRP shall be higher than that of PC3 UE in order to achieve longer communication range. In this contribution, we share our link budget numbers for the minimum peak EIRP and REFSENS to facilitate the development of the UE RF requirements for this new FR2 FWA UE.

# Discussion

Table 2-1 and Table 2-2 summarize our minimum peak EIRP and REFSENS link budget numbers respectively. Notice that for minimum peak EIRP, we only provide the worst-case numbers as the nominal values would not be relevant to the intended specifications.

| Parameter | Unit | Nominal value | Worst value |
| --- | --- | --- | --- |
| Operating bands | GHz | n257/n258 |  |
| Pout per element | dBm |  | 12.0 |
| # of antennas in an array |  | 8 | 8 |
| Total conducted power per polarization | dBm |  | 21.0 |
| Average antenna element gain | dBi |  | 3.3 |
| Antenna roll-off loss versus frequency | dB |  | -1.5 |
| Realized antenna array gain | dBi |  | 10.8 |
| Polarization gain | dB |  | 2.5 |
| Mismatch and transmission line loss including load pull | dB |  | -1.5 |
| Beam forming loss (phase shifter and amplitude error) | dB |  | -1.4 |
| Finite beam table | dB |  | 0 |
| Beam forming loss (one beam table fits all) | dB |  | -0.5 |
| Form factor integration losses | dB |  | -5.0 |
| Total implementation loss (nominal) | dB |  |  |
| Total implementation loss (worst case) | dB |  |  |
| Peak EIRP (Nominal) | dBm |  |  |
| Tolerance (+/-) | dB |  |  |
| Peak EIRP (Minimum) | dBm |  | 26.0 |

Table 2-1 Minimum peak EIRP link budget numbers

| Parameter | Unit | Value |
| --- | --- | --- |
| Operating bands |  | n257/n258 |
| Frequency range | GHz | 24.5 – 29.5 |
| Modulation |  | QPSK |
| SNR requirement | dB | -1.0 |
| Bandwidth | MHz | 50 |
| Thermal noise | dBm/Hz | -174.0 |
| Noise Figure | dB | 10.5 |
| Number of antenna in an array |  | 8 |
| Array gain | dB | 9.0 |
| Element gain | dBi | 3.3 |
| Diversity gain | dB | 0.0 |
| Antenna gain roll-off over frequency | dB | -1.5 |
| Beamforming loss | dB | -1.5 |
| Total insertion loss | dB | -6.0 |
| REFSENS for 50MHz channel BW | dBm | -90.8 |

Table 2-2 REFSENS link budget numbers

# Conclusion

In this contribution, we share our link budget numbers for minimum peak EIRP and REFSENS for the newly proposed FR2 FWA UE operating in n257 and n258 for consideration.

# References

- RP-200503 “New WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258”, SoftBank, 3GPP TSG-RAN Meeting #87-e, Online, March 16th – 19th, 2020