3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2003563

Online, 20 – 30 April, 2020

Agenda item:	5.10.4

Source: 	Qualcomm Incorporated

Title: 	Simulation results for MPDCCH performance improvement tests in R16

Document for:	Discussion

# Introduction

In RAN4#94-e, the demod requirements for MTC R16 was discussed and the WF captured in [1]. In this paper, we present partial simulation results on MPDCCH performance improvement tests and present our view on the need for multi-TB scheduling tests

# Discussion

In [1], the following was captured:

# Agreements on UE/BS demodulation requirements

- No new MPDCCH/PDSCH demodulation requirements with the use of LTE control region.

- FFS to define new demodulation requirements of PUSCH with multi-TB scheduling.

- FFS to define new demodulation requirements of PDSCH with multi-TB scheduling.

As pointed in email discussions by several companies, we do not agree to defining any new demodulation requirements for PDSCH/PUSCH with multi-TB scheduling. Our view is aligned with the reasoning described in [2].

Proposal 1. RAN4 to not define any new demodulation requirements of PDSCH/PUSCH with multi-TB scheduling.

Simulation assumptions for MPDCCH performance improvement tests were listed in [1]. Here, we present our simulation results for the proposed tests.

Table 1 below summarizes the 1% BLER simulation results without implementation margin and compares them with existing requirements in TS 36.101.

| Test configuration | mPDCCH-NumRepetition | SNR (dB) for 1% BLER |
| --- | --- | --- |
| Mode A Test 1 (8.11.2.1.1-1) | 32 | -4.8 |
| Mode B Test 1 (8.11.2.1.2-1) | 64 | -12.0 |
| Proposed Mode A Test 1 | 16 | -6.2 |
| Proposed Mode A Test 1 | 32 | -8.6 |
| Proposed Mode B Test 1 | 32 | -11.3 |
| Proposed Mode B Test 1 | 64 | -13.5 |

In our view, for CE Mode A, mPDCCH-NumRepetition of 16 can be chosen for the new R16 test as the SNR requirement is more suited to the range of CE Mode A. Moreover, mPDCCH-NumRepetition of 32 is already tested in existing requirements.

Proposal 2. CE Mode A test for MPDCCH performance improvement to be with mPDCCH-NumRepetition = 16.

Similarly, for CE Mode B, mPDCCH-NumRepetition of 32 can expand the coverage as the existing test already has mPDCCH-NumRepetition of 64.

Proposal 3. CE Mode B test for MPDCCH performance improvement to be with mPDCCH-NumRepetition = 32.

# Conclusions

Proposal 1. RAN4 to not define any new demodulation requirements of PDSCH/PUSCH with multi-TB scheduling.

Proposal 2. CE Mode A test for MPDCCH performance improvement to be with mPDCCH-NumRepetition = 16.

Proposal 3. CE Mode B test for MPDCCH performance improvement to be with mPDCCH-NumRepetition = 32.

# References

[1] R4-2002426

[2] R4-2001351