3GPP TSG-RAN WG4 Meeting # 94-e-Bis                                            R4-2003616

# Electronic Meeting, 20 – 30 Apr., 2020

Agenda Item:	6.1.5.10

Source:	MediaTek Inc.

Title:	Discussion on RLM requirement for NR-U

Document for:		Discussion

- Introduction

In this paper we provide our view on the remaining issues of RLM requirements for NR-U.

- The set of QCLed SSBs that UE is required to monitor

In last meeting, three options are listed in WF for monitoring on QCL-ed SSBs, as attached below for reference [1][2].

- The set of SSB that UE is required to monitor

  - Option 1: UE is required to monitor at least one SSB from the set of SSBs that are QCLed with each other

  - Option 2: UE is required to monitor SSBs from the set of SSBs that are QCLed with each other within the set of configured RLM-RS resources, until it detects an SSB during this SMTC during RLM or link recovery procedures

  - Option 3: UE shall monitor all SSBs configured for RLM, regardless of QCL information

Our views are provided in our companion paper to discuss the measurement on QCL-ed SSBs and the corresponding measurement capability for NR-U [3], and it proposes:

Proposal 1: UE is required to monitor at least one SSB from the set of SSBs that are QCLed with each other, for the requirements of RLM, IDLE mode measurement, and CONNECTED mode measurement, including intra-/inter-frequency measurement. With this proposal we can avoid the introduction of UE measurement capability expressed in terms of candidate SBI.

- OOS evaluation period

In the last meeting, there were discussions on how to extend the OOS evaluation period [1], and the options are listed below for reference [1][2].

- Whether to extend the OOS evaluation period based on the number of unavailable SSB

  - Option 1: No. Out-of-sync evaluation period is extended by a fixed number of samples

  - Option 1a: No. Out-of-sync evaluation period excluding the available SSB is scaled by a fixed factor of N

  - Option 2: Yes. OOS evaluation is based on Lout, where Lout ≤Lout,max is the number of SSBs not available at the UE during TEvaluate_out_SSB

  - Option 3: Select option 2 in FBE networks and option 1 in LBE networks

Extending OOS evaluation period based on Lout (Option 2) is relying on UE detection for the presence of SSBs. Since DL LBT failure is unknown to the UE, the UE would just perform SINR estimation on the candidate SSB positions by assuming the SSB is transmitted, even if the SSB was not transmitted by BS. One conjecture in the previous meeting was that UE may be able to differentiate whether DRS was transmitted or not, based on RSRP estimation. To address this comment, we conducted simulation in Appendix to further check the reliability of SSB presence detection in a single cell scenario. And it can observed that

Observation 1: Extending OOS evaluation period based on Lout is not practical under low SNR condition. The mis-detection rate of SSB presence detection would be >10% when the SNR<-5dB.

Since UE may not be able to reliably detect the presence of SSB in low SNR region as shown above, the UE would not know the cause of unavailable SSBs, since it could be caused by bad channel condition or LBT failure at BS. Due to this fact, we propose Option 1, based on the assumption that UE is not required to detect the presence of SSB.

Proposal 2: The Out-of-sync evaluation period is extended by a fixed number of samples, and is not based on the number of unavailable SSB.

Proposal 3: UE is not required to detect the presence of SSB for OOS evaluation.

One concern was how to select a suitable scaling factor for the OOS evaluation period extension. Too low scaling factor could trigger OOS indication while UE is well in-sync but with few LBE failures; too large scaling factor will introduce long delay. In the last meeting, the maximum extension of RLM in-sync has been agreed, and the scaling factor of OOS can apply the same structure. As the duration of the non-scaled period of OOS is twice the non-scaled period of INS, the L value is proposed to be twice than that of in-sync evaluation.

Proposal 4: OOS evaluation based on (10 + L) samples, where L is:

- L = 14 for Max(TDRX,TSSB)≤40 where TDRX=0 for non-DRX

- L = 10 for 40<Max(TDRX,TSSB)≤320

- L = 6 for TDRX>320

And the corresponding OOS evaluation period is provided below for reference.

| Configuration | TEvaluate_out_SSB (ms) |
| --- | --- |
| no DRX | Max(200, Ceil((10+L) P) TSSB) |
| DRX cycle≤320ms | Max(200, Ceil((1.5*(10+ L)) P) Max(TDRX,TSSB)) |
| DRX cycle>320ms | Ceil((10+ L) P) TDRX |

Option 1a would be similar to Option 1. However, more clarification is required for the definition of “available SSB” in Option 1a.

Observation 2: Regarding Option 1a, further discussion needed on the definition of “available SSB”.

Option 3 will introduce 2 sets of RLM requirements for NR-U. However, for FBE network, it is still concerning that UE is required to perform SSB presence detection under low SNR condition, since sometimes the SSB would not be transmitted in FBE network and its presence depends on the DL LBT result, which is performing before the frame transmission.

Observation 3: For FBE network, SSB may not be transmitted consistently, as its presence depends on the DL LBT result before the frame transmission.

- Transmit power of RLM-RS across different occasions

One anther discussion point is on whether UE can expect gNB to transmit RLM-RS with same transmit power across different occasions regardless of all possible LBT procedure outcomes. One extreme case is that if gNB configures 100MHz BW to UE but only one subband of 20 MHz passed the LBT, will gNB put all transmit power on that 20MHz with 7 dB power boosting? And one relevant RAN1 agreement is attached below for reference.

Agreement:

A UE shall not average CSI-RS measurements for channel estimation across different transmission bursts from the UE's perspective.

FFS: Potential issues due to AGC

One concern is that if UE is not allowed to average RLM-RS measurements, then the RLM procedure, which in Rel-15 requires 5 samples for SSB and 10 samples for CSI-RS, will be not reliable. And not only RLM but also all CSI-RS based measurement would be impacted. At least, regarding the above RAN1 agreement, it can observe that is irrelevant to SSB measurement.

Observation 4: RAN1 agreement is for CSI-RS measurement, and it is irrelevant to SSB based measurement.

Proposal 5: For SSB based RLM, UE can assume the same level of power boosting, as specified PDCCH transmission parameters, and consistent power per RE across different transmission bursts.

Besides, one interpretation on the above RAN1 agreement would be that is only applicable to some application of CSI-RS, e.g. CSI-RS for acquisition, or the case when timeRestrictionForChannelMeasurement is configured, and thus it is not applicable to other CSI-RS measurements such as RLM, beam management. Therefore, more discussion on the CSI-RS measurement is required.

Proposal 6: For CSI-RS based measurement, further discuss whether UE can assume the power per RE is constant across different transmission bursts.

- How to handle CSI-RS based RLM

Since CSI-RS was not designed for detection purpose, the presence detection on CSI-RS is less reliable than that of SSB. Therefore, the extension of CSI-RS measurement or evaluation period should not depend on the number of not available CSI-RS samples, since UE would not know whether the CSI-RS is available or not.

Observation 5: UE would not know whether the CSI-RS is available or not, since CSI-RS is not designed for detection purpose.

Therefore, it needs more discussion on how to determine the extended period for CSI-RS measurements. Besides, other clarification on CSI-RS based measurement is also required as discussed in Section 4.  However, some common issues of SSB based and CSI-RS based RLM are unsolved, it would suggest to focus on SSB based RLM requirement first and deprioritize the discussion of CSI-RS based measurement.

Proposal 7: RAN4 deprioritizes defining requirements for CSI-RS based RLM in Rel-16 NR-U networks.

- Summary

In this paper, RLM requirements for NR-U have been discussed. We have the following observations and proposals:

Proposal 1: UE is required to monitor at least one SSB from the set of SSBs that are QCLed with each other, for the requirements of RLM, IDLE mode measurement, and CONNECTED mode measurement, including intra-/inter-frequency measurement. With this proposal we can avoid the introduction of UE measurement capability expressed in terms of candidate SBI.

Observation 1: Extending OOS evaluation period based on Lout is not practical under low SNR condition. The mis-detection rate of SSB presence detection would be >10% when the SNR<-5dB.

Proposal 2: The Out-of-sync evaluation period is extended by a fixed number of samples, and is not based on the number of unavailable SSB.

Proposal 3: UE is not required to detect the presence of SSB for OOS evaluation.

Proposal 4: OOS evaluation based on (10 + L) samples, where L is:

- L = 14 for Max(TDRX,TSSB)≤40 where TDRX=0 for non-DRX

- L = 10 for 40<Max(TDRX,TSSB)≤320

- L = 6 for TDRX>320

Observation 2: Regarding Option 1a, further discussion needed on the definition of “available SSB”.

Observation 3: For FBE network, SSB may not be transmitted consistently, as its presence depends on the DL LBT result before the frame transmission.

Observation 4: RAN1 agreement is for CSI-RS measurement, and it is irrelevant to SSB based measurement.

Proposal 5: For SSB based RLM, UE can assume the same level of power boosting, as specified PDCCH transmission parameters, and consistent power per RE across different transmission bursts.

Proposal 6: For CSI-RS based measurement, further discuss whether UE can assume the power per RE is constant across different transmission bursts.

Observation 5: UE would not know whether the CSI-RS is available or not, since CSI-RS is not designed for detection purpose.

Observation 1: For single cell scenario, the mis-detection rate for the presence of SSB will be >50% at SNR<-10 dB and will be <10% at SNR > -5dB.

Proposal 7: RAN4 deprioritizes defining requirements for CSI-RS based RLM in Rel-16 NR-U networks.

References

[1] R4-2002335 WF on NR-U RRM Requirements (Part 2), MediaTek Inc.

[2] R4-2002278 WF on NR-U RRM Requirements (Part 3), Nokia.

[3] R4-2003617 Discussion on measurement on QCL-ed SSBs and measurement capability for NR-U, MediaTek Inc.

Appendix A1: Simulation results for SSB’ presence detection

The CDF of RSRP estimated at UE side under different SNR side conditions are shown in Figure A.1 and Figure A.2, for SCS of 15kHz and 30kHz respectively. The detailed simulation parameters are listed in the Appendix A2. In the figures, the leftmost curve shows the RSRP would be estimated when the DRS samples is not transmitted. The RSRP overlapping region of the leftmost curve and the other curves indicate how likely UE would consider transmitted DRS samples as “not transmitted”.

Figure A.1 CDF of 1-sample RSRSP estimation (TDL-C, SCS=15kHz).

Figure A.2 CDF of 1-sample RSRSP estimation (TDL-C, SCS=30kHz).

Table A.1 summarizes the mis-detection rate under different SNR side conditions, wherein the mis-detection rate is defined as the probability of the detected RSRP of transmitted SSB below the 95%-tile CDF of the detected RSRP when SSB is not transmitted.

Table A.1: Mis-detection rate on the presence of a SSB, under fading channel used in R15 RLM test case.

| SNR side condition | Mis-detction rate |  |
| --- | --- | --- |
|  | SCS = 15 kHz | SCS = 30 kHz |
| 0dB | 0.2% | 0.2% |
| -5dB | 7.3% | 7.4% |
| -10dB | 53.6% | 53.6% |
| -12dB | 69.9% | 72.5% |
| -15dB | 85.7% | 86.0% |

Based on above analysis, the reliability of this kind of detection is questionable.

- When SNR is below -10dB, it will lead to high probability (>50%) that UE would consider a SSB as “not transmitted”. Therefore UE is very likely to keep extending the corresponding measurement/evaluation period.

- When the SNR above a certain level (e.g. [-5]dB), UE may be able to detect the present of SSB.

Note that above evaluation is based on one single cell. In other words, if BS does not transmit SSB, the estimated RSRP is purely contributed by noise. If the multi-cell scenario is considered, there could be still a neighboring cell which is transmitting SSB when UE’s serving cell fails LBT. In this case, the estimated RSRP for the case of SSB not transmitted would go higher, since the interference from other cells would be received and introduce additional estimation errors.

Observation 1: For single cell scenario, the mis-detection rate for the presence of SSB will be >50% at SNR<-10 dB and will be <10% at SNR > -5dB.

Appendix A2: Simulation Assumptions

Table A.2: Simulation Parameters

| Parameters | Value |
| --- | --- |
| Deployment | Single cell |
| SCS | 30 kHz |
| SNR range | -15dB ~ 0dB |
| Noc level | -100dBm/15kHz; - 97dBm/30kHz |
| Number of samples (M) | 1 (single sample measurement) |
| Propagation condition | Fading channel (TDL-C 300ns 100Hz, as used in RLM test case) |