3GPP TSG-RAN WG4 Meeting # 94-e-Bis                                            R4-2003618

# Electronic Meeting, 20 – 30 Apr., 2020

Agenda Item:	6.1.5.11

Source:	MediaTek Inc.

Title:	Discussion on SFTD measurements towards NR-U with LBT

Document for:		Discussion

- Introduction

In last meeting, the requirements for inter-RAT SFTD measurements towards NR-U with LBT have been discussed as WF [1]. In this paper, our view on the maximum scaling, k, for inter-RAT SFTD measurements for NR-U is provided.

- Discussion

Background from RAN4#93

- UE behavior upon exceeding Tmeasure_SFTD_LBT_max: UE shall stop the search

- FFS whether UE abandons the measurement

- Tmeasure_SFTD_LBT_max = k× Tmeasure_SFTD1, k=TBD≤10

Options in RAN4#94e

- Option 1: k = 6

- Option 2: k =2

  - Rel-15 side conditions apply

For the inter-RAT SFTD requirement, the corresponding WF [1] is attached below for reference.

In RAN4#93, the maximum period for EN-DC SFTD towards NR-U with LBT was agreed as

- NLBT-fail,max

| Condition | NLBT-fail,max |
| --- | --- |
| max(TDRX,TSMTC)≤40 | [7] |
| 40<max(TDRX,TSMTC)≤320 | [5] |
| TDRX > 320 | [3] |

Therefore, the maximum scaling for inter-RAT SFTD requirement can be similar to the agreed maximum scaling of EN-DC SFTD, in order to cope with the similar level of LBT failure probability. The maximum scaling can be derived based on the number of samples for R15 non-scaled period and the corresponding NLBT-fail,max. For example, for the EN-DC SFTD measurement with 40<max(TDRX,TSMTC)≤320, the maximum scaling will be 1.6, because it requires 8 samples in Rel-15 and the corresponding NLBT-fail,max is 5, (8+5)/5 = 1.6. The maximum scaling for EN-DC SFTD is summarized in Table 1.

Table 1. Maximum scaling for EN-DC SFTD

| Procedure | # of samples in Rel-15 | NLBT-fail,max | Condition | Max. Scaling |
| --- | --- | --- | --- | --- |
| ENDC SFTD | 5 | 7 | max(TDRX,TSMTC)≤40 | 2.4x |
|  | 8 | 5 | 40<max(TDRX,TSMTC)≤320 | 1.63x |
|  | 5 | 3 | TDRX > 320 | 1.6x |

Observation 1: For EN-DC SFTD measurement in NR-U, the maximum scaling is ranged from 1.6 to 2.4.

For the inter-RAT SFTD measurements, over-design with a large maximum scaling, i.e. k value, will significantly consume UE resources to search NR-U cells within the whole SMTC window, since the SMTC offset is unknown to UE. In addition, considering the inter-RAT SFTD measurement period is already longer than the EN-DC SFTD measurement period, a slightly small k=2 is proposed.

Proposal 1: k=2, as the maximum scaling of inter-RAT SFTD measurements towards NR-U (Option 2).

- Summary

In this paper, our view of the maximum scaling for inter-RAT SFTD measurements towards NR-U with LBT is provided. We have the following observations and proposals:

Observation 1: For EN-DC SFTD measurement in NR-U, the maximum scaling is ranged from 1.6 to 2.4.

Proposal 1: k=2, as the maximum scaling of inter-RAT SFTD measurements towards NR-U (Option 2).

References

[1] R4-2002278 WF on NR-U RRM Requirements (Part 3), Nokia.