3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003681

# Electronic Meeting, 20th Apr – 30th Apr, 2020

Title: 	Discussion and simulation on URLLC UE performance requirements for verifying PDSCH mapping Type B and processing capabiltiy 2

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.2

Document for:	Discussion

Background

When considering the low latency of URLLC UE performance requirements, RAN4 agreed to verify the related features. According to the conclusion from RAN4 #94 e-meeting, three features were agreed to be verified and two of them (PDSCH mapping type B and PDSCH processing capability) were agreed to be verified together. Thus, two performance requirements will be defined for URLLC UE low latency.

- Define performance requirements for PDSCH mapping type B and PDSCH processing capability 2.

- Define performance requirements for preemption indication for eMBB UE.

In this paper, the performance requirements for PDSCH mapping type B and PDSCH processing capability 2 will be discussed according to the agreements and open issues made in #94 e-meeting [1]:

Agreements for FR1:

- Slots to be scheduled:

    - FDD: All DL slots

    - TDD: S slots with K1=0

- Starting symbol: 2

- Symbol length: 2

- Slot aggregation level: 1

- Max number of HARQ transmissions: 1

- Verify PDSCH processing capability 2 and type B mapping together

There are also some open issues will be discussed further for this verification:

Open Issues

- Whether to introduce UE performance requirements in FR2 for URLLC for low latency

- For test parameters for FR1:

- TDD Pattern (30KHz SCS)

  - Option 1: 7D1S2U, S=6D:4G:4U

  - Option 2: DDDSUUDDDD, S=6D:4G:4U

  - Option 3: DSUU, S=12D:2G

  - Option 4: DDDSU, S=10D+2G+2U

- Number of HARQ Processes

  - Option 1: 2

  - Option 2: 4 or 8

- Symbol length (L)

  - Option 1: Also include 4os

  - Option 2: Also include 7os with K1 = 1

  - Option 3: Only 2os

- Other test parameters

  - Channel Model

  - FRC

  - Antenna configuration

  - SCS/BW, Allocation

In this paper, based on the agreed parameters, the open issues are anlysed and our views are provided

Discussion

  - Whether to introduce UE performance requirements in FR2 for URLLC for low latency

From current deployment request, we do not observe there is any request for FR2 URLLC deployment, it is better to focus on FR1 firstly and deprioritize FR2.

Proposal 1: No need to define UE performance requirements in FR2 for URLLC for low latency.

Open issues

Regarding to the mapping Type B symbol length, although L=7 has been defined in the existing Rel-15, the performance requirements was only for mapping Type B and did not verify the PDSCH processing capability 2 together. The new defined requirements should consider both features. Thus, as the typical mapping symbol length for Type B, L=7 should be defined.

Proposal 2: Symbol length of 7 should be defined.

The remained open issues are interrelated. For PDSCH processing capability 2, at least 4.5 symbols are needed between PDSCH and corresponding HARQ-ACK information. By considering start symbol = 2, we have following proposals:

Proposal 3: We propose TDD pattern of ‘DDDSU, S=10:2:2’ to be defined for this performance requirements.

Proposal 4: K1=1 for symbol length of 7.

The number of HARQ processes is dependence to the TDD pattern. For ‘DDDSU’, the number of HARQ processes can be configured to 4.

Proposal 5: The number of HARQ processes is 4.

For other parameters, we propose to reuse the configurations from the existing Rel-15 PDSCH Type B requirements

| Test cases | FR1 FDD | FR1 TDD |
| --- | --- | --- |
| Channel model | TDLA30-10 | TDLA30-10 |
| Antenna configuration | 2x2 and 2x4, ULA Low | 2x2 and 2x4, ULA Low |
| MCS | MCS 4 in Table 1 | MCS 4 in Table 1 |
| Number of contiguous PRB | 10MHz / 15kHz with full bandwidth | 40MHz / 30kHz with full bandwidth |

Proposal 6: We propose to use TDLA30-10 as the channel model.

Proposal 7: We propose the antenna configuration is: 2x2 and 2x4, ULA Low

Proposal 8: We propose to use MCS 4 in Table 1.

Proposal 9: We propose to configure 10 MHz/15 kHz for FDD, 40 MHz/30 kHz for TDD with full bandwidth frequency allocation.

Simulation results

Based on the above discussion, performance for mapping Type B for FDD and TDD with different symbol lengths are simulated.

- FDD

Table 3-1: Test parameters for UE PDSCH FDD performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Duplex mode |  | FDD |
| Antenna configuration |  | 2x2 and 2x4, ULA low |
| PDSCH configuration | Mapping type | Type B |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 7 |
|  | PDSCH aggregation factor | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
| CSI-RS configuration | Periodicity and offset | 5 slots, 0 slots |
|  | Number of ports | 2 |
|  | Frequency domain allocation | Row3, ‘000001 |
|  | First OFDM symbol in time domain | 7 |
|  | Cdm Type | CDM2 |
|  | Density | 1 |
| Maximum number of HARQ transmission |  | 1 |
| Propagation condition |  | TDLA30-10 |
| MCS Table |  | Table 1, MCS 4 |
| SCS and BW |  | 15KHz & 10MHz |
| Frequency domain resource |  | Full BW |
| Testing metric |  | Throughput: 70% |

The simulation performances of PDSCH FDD with symbol lengths of 2 and 4 are shown in below:

Figure 3-1: Throughput vs SNR for PDSCH FDD with symbol lengths of 2 and 7 for antenna configuration 2x2 and 2x4

According to the simulation results shown in Figure 3-1, the SNR values of 70% throughput are listed in table below:

Table 3-2: SNR values of 70% throughput for FDD

| Symbol length SNR Antenna configuration | 2x2 | 2x4 |
| --- | --- | --- |
| L=2 | -2.4 | -5.7 |
| L=7 | -2.8 | -5.8 |

- TDD

The PDSCH mapping Type B for TDD was agreed that only scheduled on S slot. The agreed and proposed parameters are listed in table 3-3:

Table 3-3: Test parameters for UE PDSCH TDD performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Duplex mode |  | TDD |
| Antenna configuration |  | 2x2 and 2x4, ULA low |
| PDSCH configuration | Mapping type | Type B |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 7 |
|  | PDSCH aggregation factor | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
| CSI-RS configuration | Periodicity and offset | 5 slots, 0 slots |
|  | Number of ports | 2 |
|  | Frequency domain allocation | Row3, ‘000001 |
|  | First OFDM symbol in time domain | 7 |
|  | Cdm Type | CDM2 |
|  | Density | 1 |
| Maximum number of HARQ transmission |  | 1 |
| Propagation condition |  | TDLA30-10 |
| MCS Table |  | Table 1, MCS 4 |
| TDD Pattern |  | DDDSU, S=10:2:2 |
| SCS and BW |  | 30KHz & 40MHz |
| Frequency domain resource |  | Full BW |
| Testing metric |  | Throughput: 70% |

The simulation results of PDSCH TDD with symbol lengths of 2 and 4 are shown in below:

Figure 3-2: Throughput vs SNR for PDSCH TDD with symbol lengths of 2 and 7 for antenna configuration 2x2 and 2x4

According to the simulation results shown in Figure 3-2, the SNR values of 70% throughput are listed in table below:

Table 3-4: SNR values of 70% throughput for TDD

| Symbol length SNR Antenna configuration | 2x2 | 2x4 |
| --- | --- | --- |
| L=2 | -2.2 | -5.5 |
| L=7 | -2.3 | -5.5 |

Proposals

In this contribution, we discussed the open issues for the performance requirements for PDSCH mapping Type B and PDSCH processing time capability 2. Proposals are list as follows:

Proposal 1: No need to define UE performance requirements in FR2 for URLLC for low latency.

Proposal 2: Symbol length of 7 should be defined.

Proposal 3: We propose TDD pattern of ‘DDDSU, S=10:2:2’ to be defined for this performance requirements.

Proposal 4: K1=1 for symbol length of 7.

Proposal 5: The number of HARQ processes is 4.

Proposal 6: We propose to use TDLA30-10 as the channel model.

Proposal 7: We propose the antenna configuration is: 2x2 and 2x4, ULA Low

Proposal 8: We propose to use MCS 4 in Table 1.

Proposal 9: We propose to configure 10 MHz/15 kHz for FDD, 40 MHz/30 kHz for TDD with full bandwidth frequency allocation.

Reference

- R4-2002428, “Way forward for NR URLLC performance requirements”, Intel Corporation.