3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003682

# Electronic Meeting, 20th Apr – 30th Apr, 2020

Title: 	Discussion and simulation results for URLLC UE CQI reporting requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.2

Document for:	Discussion

Background

As discussed in R4-1915125, a new 4-bit CQI table (CQI table 3) with lower coding rates for PDSCH is designed for 1-10-5 BLER target. According to the WF [1], some open issues are remained to be further discussed:

Open issues:

- Propagation channel for CQI reporting

  - Option 1: AWGN

  - Option 2: Fading channel

- Target BLER

  - Option 1: 10^-3

  - Option 2: 10^-2

  - Option 3: 10^-5

  - Option 4: No BLER metric in fading channel

- Test metric

  - Option 1: Reuse existing BLER criteria test metrics

  - Option 2: Percentage based of the maximum theoretical throughput

  - Option 3: Reuse existing BLER criteria test metrics with a minimum median CQI

  - Option 4: TP ratio with follow CQI vs median CQI and CQI not in set metric

- Feasibility to define CQI reporting test case and FMCS case at the same SNR

  - Option 1: Consider defining FMCS test and CQI reporting test under AWGN with the same SNR and define an applicability rule between CQI reporting test and FMCS test under AWGN to reduce the number of long tests

In this paper, options for CQI reporting requirements are discussed. Our views on the target BLER and the test metrics are provided.

Discussion

The URLLC service requires very high reliability of 99.999% and the CQI Table 3 is also design for 1-10-5 BLER target. In order to test the new table, we prefer to define reporting requirements in AWGN channel to use the BLER metrics. However, the experience of studying URLLC high reliability test feasibility tells us that the test time will be very long for the low BLER target (1-10-5). The CQI reporting test is more complex than the high reliability test. Thus, we propose to consider a higher BLER target, e.g. 1-10-3. New CQI reporting test can reuse the BLER criteria test metrics presented in TS38.101-4 Section 6

Proposal 1: Introduce CQI reporting requirements to verify the support of CQI Table 3 in AWGN.

Proposal 2: Consider a higher BLER target, e.g. 1-10-3.

Proposal 3: The BLER criteria test metrics presented in TS38.101-4 Section 6 can be reused.

Simulation assumption

Following the frame presented in TS38.101-4 Section 6 in AWGN, FDD and TDD CQI reporting tests should be defined.

FDD

The following table illustrates some key parameters for FDD.

Table 3-1 CQI reporting definition test

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Duplex Mode |  |  | FDD |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Propagation channel |  |  | AWGN |  |
| Antenna configuration |  |  | 2×2 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 2 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3,(6,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 |  |
| ReportConfigType |  |  | Periodic |  |
| CQI-table |  |  | Table 3 |  |
| reportQuantity |  |  | CQI |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| Csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  | slot | 5/1 |  |
| CQI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 1 |  |

TDD

The following table illustrates some key parameters with 2Rx for TDD.

Table 3-2 CQI reporting definition test

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD UL-DL pattern |  |  | 7D1S2U(S=6:4:4) |  |
| Propagation channel |  |  | AWGN |  |
| Antenna configuration |  |  | 2×2 with static channel |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  | CSI-RS periodicity and offset | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 2 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3,(6,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 |  |
| ReportConfigType |  |  | Periodic |  |
| CQI-table |  |  | Table 3 |  |
| reportQuantity |  |  | CQI |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 16 |  |
| Csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  | slot | 10/1 |  |
| Maximum number of HARQ transmission |  |  | 1 |  |

Proposal 4: Define CQI reporting test for 2Rx with FDD and TDD modes.

Proposals

In this paper, following are proposed:

Proposal 1: Introduce CQI reporting requirements to verify the support of CQI Table 3 in AWGN.

Proposal 2: Consider a higher BLER target, e.g. 1-10-3.

Proposal 3: The BLER criteria test metrics presented in TS38.101-4 Section 6 can be reused.

Reference

[1] R4-2002428, Way forward for NR UE URLLC performance requirements, #94, Intel Corporation.