3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003683

# Electronic Meeting, 20th Apr – 30th Apr, 2020

Title: 	Discussion for URLLC BS PUSCH demodulation requirements for high reliability

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.3

Document for:	Discussion

Background

RAN4 has agreed that the BLER target 10^-5 and the higher BLER target will be discussed separately when introduce URLLC BS PUSCH demodulation requirements for high reliability, the higher BLER target was discussed on #94 meeting and 10^-2 was agreed as the BLER target. The PUSCH demodulation requirements for high reliability with target BLER 10^-2 are discussed in this paper. From #94 e-meeting discussion results, some agreements for FR1 were made [1]:

Agreement:

    - Target BLER : 1%

    - Target confidence level: 99%

    - Calculate the target BLER after all transmission if HARQ activated.

    - Number of HARQ transmission: 4

    - MCS: MCS 5 in table 3

    - Waveform: CP-OFDM

    - TDD pattern:

      - 15kHz SCS: 3D1S1U, S=10D: 2G: 2U

      - 30kHz SCS: 7D1S2U, S=6D: 4G: 4U

    - Symbol length is 14 with starting symbol 0

    - DM-RS configuration: Type 1 with single-symbol: 1+1

    - Antenna configuration: 1x2, low

    - Propagation condition: TDLB100-400

    - Mapping Type: Type A and B with test applicability rule defined.

    - SCS&BW for FR 1

      - 10MHz/15kHz

      - 40MHz/30kHz

      - Other sets are not precluded

According to the WF [1], some open issues are remained for FR1:

Open issues:

- PUSCH aggregation level

      - Option 1: 2

      - Option 2: 4

      - Option 3: 2, 4

      - Option 3: 8

- Number of PRBs

      - Option 1: 25

      - Option 2: Full bandwidth

      - Option 3: Other options not precluded (depending on bandwidth set)

- Whether to introduce DFT-s-OFDM:

      - Option 1: No

      - Option 2: Yes

- Issue 4: Safety critical aspects:

      - Proposals

      - Proposal 11: If high reliability will be tested with BLER metric, add the following note to the test specification: “Note that this test procedure will only provide an indication to a certain confidence level that the target reliability requirements are likely to be satisfied, and it is assumed that for critical applications further testing would be done to ensure suitability of the equipment for the intended application.” (Nokia)

- Since the URLLC features of 5G NR will potentially be used in safety critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians, before requirements and test can be used as basis for safety critical implementations. All statistical analysis and discussions provided in this meeting are to be taken as a best effort and is not to be taken as due diligence. (Nokia)

      - Option 1: Need to be clarified in 3GPP specification.

      - Option 2: No need to be clarified in 3GPP specification.

The above issues relate to FR1, there are also some open issues will be discussed for FR2:

- Whether to define BS FR2 URLLC performance requirements for high reliability

      - Option 1: Do not define

      - Option 2: Define.

- Test applicability rule for FR1 and FR2 performance requirements if both requirements are defined

      - Option 1: Based on BS declaration of support FR1 or FR2

      - Option 2:  If BS supports both FR1 and FR2, the performance requirements for both FR1 and FR2 should be tested

- SCS&BW for FR2 if FR2 will be defined

      - 60kHz SCS

      - Option 1: 50/100MHz

      - Option 2: No test

      - Option 3: 50 MHz

      - 120kHz SCS

      - Option 1: 50/100/200MHz

      - Option 2: No test

      - Option 3: 100 MHz

In this paper, the remained open issues are discussed and our views are provided.

Open issues discussion for FR1

PUSCH aggregation level

PUSCH repetition is the key feature to improve the reliability for URLLC. At this stage, we are defining the performance requirements for multi-segment transmission. The number of repetition relies on the number of consecutive available slots. As TDD pattern ‘7D1S2U’ for 30 kHz SCS was agreed by RAN4, we propose AL=2 for PUSCH.  As RAN4 are trying to align the performance requirements for TDD and FDD for uplink, by configuring the maximum number of HARQ transmission to 4, the reliability can be guaranteed.

Proposal 1: We propose PUSCH aggregation level is 2.

Number of PRBs

Regarding to the allocated number of PRBs, it is related to the data packet size of URLLC use cases. According to the TR 38.824, use cases listed in Table 2.2-1 were identified to be considered. The data packet sizes are different from 32 bytes to 10K bytes for uplink.

Table 2.2-1: Representative use cases for Rel-16 NR URLLC evaluation [2]

| Use case | Reliability (%) | Data packet size |
| --- | --- | --- |
| Power distribution | 99.9999 | DL & UL: 100 bytes |
|  | 99.999 | DL & UL: 250 bytes |
| Factory automation | 99.9999 | DL & UL: 32 bytes |
| Rel-15 enabled use case (AR/VR) | 99.999 | DL & UL: 32 and 200 bytes |
|  | 99.9 | DL & UL: 4096 and 10K bytes |
| Transport industry | 99.999 (with 5 ms end to end latency) | DL & UL: 2083 and 5220 bytes |
|  | 99.999 (with 10 ms end to end latency) | DL & UL: 1370 bytes |

For uplink SCS and bandwidth, 30kHz/40MHz & 15kHz/10MHz were agreed in RAN4 #94 e-meeting, along with the agreements of PUSCH configuration, DM-RS configuration and coding rate of 99/1024, the maximum payloads in a slot are 180 bytes for 15kHz SCS and 366 bytes for 30kHz SCS. By considering all the use cases above, we propose to use full bandwidth for frequency domain allocation.

Proposal 2: Use full bandwidth for frequency domain allocation.

Whether to introduce DFT-s-OFDM:

At current stage, we do not see any use cases using DFT-s-OFDM. We propose to define performance requirements only for CP-OFDM.

Proposal 3: No need to define performance requirements for DFT-s-OFDM.

Safety critical aspects:

The whole URLLC system safety can only be ensured by considering many aspects and not only the demodulation performances, RAN4 just focus on the demodulation performance from physical layer features designed for URLLC and not a verification body, it is hard to capture such concern in 3GPP specification.

Proposal4: RAN4 does not need to clarify the safety statement.

Performance simulation for FR1

Based on the discussion above, the simulation parameters for BS PUSCH performance requirements can be conclude in the table below:

Table 3-1: Parameters for BS PUSCH performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, Low |
| Number of Rx |  | 2 |
| Antenna configuration |  | ULA low |
| PUSCH configuration | Mapping type | Type A and Type B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PUSCH aggregation factor | 2 |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
| Propagation condition |  | TDLB100-400 Low |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | 15 kHz / 10 MHz 30 kHz / 40 MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| Maximum number of HARQ transmissions |  | 4 |
| Testing metric |  | Target BLER: 10-2 |

Simulation result for FDD

Based on the parameters listed in Table 3-1, the BLER vs SNR curves for FDD with aggregation levels of 2 and 4 are shown in figure below.

Figure 3.1-1: BLER vs SNR for PUSCH FDD (AL=2, 4)

Figure 3.1-1 shows that the SNR value is about -12.5 dB when aggregation level is 4. The SNR value is about -9.5 when aggregation level is 2.

Simulation result for TDD

Based on the parameters listed in Table 3-1, the BLER vs SNR curve is shown in figure below.

Figure 3.2-1: BLER vs SNR for PUSCH TDD (AL=2)

Figure 3.1-1 shows that the SNR value is around -8.8dB with target BLER=0.01.

Whether to define BS FR2 URLLC performance requirements for high reliability

From current deployment request, FR2 is not a common use case for URLLC. For the link level evaluation studies from RAN1, the common carrier frequency for different use cases is 4 GHz and 700 MHz. The following table lists some sources:

Table 4-1: Carrier frequency for URLLC use cases

| Use case | Carrier frequency for evaluation | Source |
| --- | --- | --- |
| Electrical power distribution | 4 GHz | R1-1901248, R1-1900077, R1-1901350 |
|  | 700 MHz | R1-1901352 |
| Factory automation | 4 GHz | R1-1901555, R1-1903400, R1-1903447 |
| Rel-15 enabled use case | 4 GHz | R1-1900079, R1-1903451, R1-1903451, R1-1901250 |
|  | 700 MHz | R1-1901250 |
| Transport industry | 4 GHz | R1-1901247, R1-1900080, R1-1901351 |
|  | 700 MHz | R1-1901553, R1-1900238 |

At this stage, we think it is better to focus on FR1 firstly and deprioritize FR2.

Proposal 5: No need to define BS FR2 URLLC performance requirements for high reliability.

Proposals

By discussing the open issues for URLLC BS PUSCH performance requirements for high reliability, we have following proposals:

Proposal 1: We propose PUSCH aggregation level is 2.

Proposal 2: Use full bandwidth for frequency domain allocation.

Proposal 3: No need to define performance requirements for DFT-s-OFDM.

Proposal 4: RAN4 does not need to clarify the safety statement.

Proposal 5: No need to define BS FR2 URLLC performance requirements for high reliability.

Reference

[1] R4-2002429, Way forward for NR BS URLLC performance requirements, #94, Huawei, Hisilicon.

[2] 3GPP TR38.824: “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”