3GPP TSG-RAN WG4 Meeting #94e-bis	R4-2003706

# Electronic Meeting, 20th - 30th Apr, 2020

Title: 	Simulation results on UE performance requirements for LTE-based 5G terrestrial broadcast

Source: 	Huawei, HiSilicon

Agenda item:	5.14.2

Document for:	Discussion

Background

During RAN4#94e meeting, way forward [1] for LTE-based 5G terrestrial broadcast demodulation requirements was approved. In this contribution, we share our views about LTE-based 5G terrestrial broadcast demodulation requirements.

Simulations

PMCH rooftop scenario

| Test number | Cell | Bandwidth (MHz) | MCS | Antenna configuration | MBSFN RS type | Number of MBSFN subframes per 40ms | 1% PMCH BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Dedicated | 10 | 18 | 1x1 | 1 | 13 |  |
| 2 | Dedicated | 10 | 18 | 1x1 | 2 | 13 |  |
| 3 | Dedicated | 10 | 21 | 1x1 | 1 | 13 |  |
| 4 | Dedicated | 10 | 21 | 1x1 | 2 | 13 |  |
| 5 | Dedicated | 10 | 22 | 1x1 | 1 | 13 |  |
| 6 | Dedicated | 10 | 22 | 1x1 | 2 | 13 |  |
| 7 | Dedicated | 10 | 24 | 1x1 | 1 | 13 |  |
| 8 | Dedicated | 10 | 24 | 1x1 | 2 | 13 |  |

(to be updated)

PMCH mobility scenario

| Test number | Cell | Bandwidth (MHz) | MCS | Antenna configuration | Number of MBSFN subframes per 40ms | 1% PMCH BLER |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | Dedicated | 10 | 12 | 1x2 low | 39 | 9.5 |

CAS scenario

PBCH repetition

| Test number | PBCH repetition pattern | PBCH Bandwidth | Transmission bandwidth | Propagation condition | Antenna configuration | 1% Pm-bch |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Table 6.6.4.1-1 in TS 36.211 | 1.4MHz | 10MHz | AWGN | 1x1 | -9.7 |
| 11 |  | 1.4MHz | 10MHz | EVA162Hz | 1x2 low | -9.8 |

PDCCH AL16

| Test number | PBCH repetition pattern | PBCH Bandwidth | Transmission bandwidth | Propagation condition | Antenna configuration | 1% Pm-bch |
| --- | --- | --- | --- | --- | --- | --- |
| 12 | Table 6.6.4.1-1 in TS 36.211 | 1.4MHz | 10MHz | AWGN | 1x1 | -7.5 |
| 13 |  | 1.4MHz | 10MHz | EVA162Hz | 1x2 low | -6.4 |

Observation 1: There is both about 2.5 dB gain for AWGN and EVA162Hz channel for PDCCH AL 16 comparing PDCCH AL8.

Proposals

In this contribution, we discuss and provide our simulation results on LTE-based 5G terrestrial broadcast demodulation requirements. Our observations and proposals are:

Observation 1: There is both about 2.5 dB gain for AWGN and EVA162Hz channel for PDCCH AL 16 comparing PDCCH AL8.

Reference

- R4-2002423, WF for 5G broadcast demod requirement, Qualcomm