3GPP TSG-RAN4 WG4 Meeting # 94Bis-e	R4-2003721

# Electronic Meeting, 20 - 30 April, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR: Test structure for NR PDSCH FR1 CA 4Rx performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Revised Rel-16 NR performance requirements enhancement WI RP-200472 is approved in RAN#87-e meeting. NR CA PDSCH normal demodulation requirements for NR CA are agreed to be defined for the following CA configs: - FDD CA with 15kHz SCS - TDD CA - 30kHz SCS + 30kHz SCS - 15kHz SCS + 30kHz SCS - TDD FDD CA - FDD 15kHz SCS + TDD 15kHz SCS - FDD 15kHz SCS + TDD 30kHz SCS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the test structure for the NR Rel-16 PDSCH normal demodulation requirements for 4Rx FR1 CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<< Start of Changes >>

5.2	PDSCH demodulation requirements

The parameters specified in Table 5.2-1 are valid for all PDSCH tests unless otherwise stated.

<<Unchanged Sections Skipped>>

5.2A	PDSCH demodulation requirements for CA

The parameters specified in Table 5.2A-1 are valid for all PDSCH CA tests unless otherwise stated.

Table 5.2A-1: Common test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  | Symbols | 0, 1 |
|  | Number of PRBs in CORESET |  |  | Table 5.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI state |  |  | TCI state #1 |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | ‘FD-CDM2’ |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | ‘FD-CDM2’ |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for Rank 1 and Rank 2 tests |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| PT-RS configuration |  |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| Precoding configuration |  |  |  | SP Type I, Random per slot with PRB bundling granularity |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |

Table 5.2-2: Number of PRBs in CORESET

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 24 | 48 | 78 | 102 | 132 | 156 | 216 | 270 | N/A | N/A | N/A |
| 30 | 6 | 24 | 36 | 48 | 60 | 78 | 102 | 132 | 162 | 216 | 270 |

5.2A.1	1RX requirements

(Void)

5.2A.2	2RX requirements

5.2A.3	4RX requirements

5.2A.3.1	Minimum requirements

The requirements for single carrier with multiple bandwidth are specified in Table 5.2A.3.1-2, with the parameters in Table 5.2A.3.1-1 and the downlink physical channel setup according to Annex C.3.1. The performance requirements in Table 5.2A.3.1-2 do not apply for single carrier test.

Table 5.2A.3.1-1: Test parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12 TDD 15kHz SCS: 8 for special slot 12 for normal downlink slot TDD 30kHz SCS: 4 for special slot 12 for normal downlink slot |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | FDD: 4 TDD: 8 |
| TDD UL-DL pattern |  |  | 15kHz SCS: FR1.15-1 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | FDD: 2 TDD 15kHz SCS: As defined in Annex A.1.2 Table A.1.2-1 TDD 30kHz SCS: As defined in Annex A.1.2 Table A.1.2-2 |

Table 5.2A.3.1-2: FDD single carrier with 15 kHz SCS performance for multiple CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 10 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 15 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 20 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 25 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 30 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 40 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 50 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |

Table 5.2A.3.1-3: TDD single carrier with 15 kHz SCS performance for multiple CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 10 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 15 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 20 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 25 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 30 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 40 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 50 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |

Table 5.2A.3.1-4: TDD single carrier with 30 kHz SCS performance for multiple CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 10 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 15 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 20 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 25 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 30 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 40 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 50 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 60 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 80 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 90 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |
| 100 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | TBD |

Table 5.2A.3.1-5: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.3.1-2 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1-4 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1-2 and Table 5.2A.3.1-4 per CC |
| 4 | FDD 15 kHz + TDD 15 kHz | As defined in Table 5.2A.3.1-2 and Table 5.2A.3.1-3 per CC |
| 5 | TDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.3.1-3 and Table 5.2A.3.1-4 per CC |
| Note 1: The applicability of requirements for different CA duplex, CA configuration and bandwidth combination sets is defined in 5.1.1.x. |  |  |

<< End of Changes >>