3GPP TSG-RAN4 WG4 Meeting #94Bis-e	R4-2003726

# Electronic Meeting, 20 - 30 April, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | - | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Addition of the performance requirements for intra-band contiguous CA with minimum channel spacing for FDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2020-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some FDD bands with available spectrum that can only fully be used by intra-band contiguous CA with channel spacing less than the nominal one, but there is no UE performance tests to verify the UE functionality to support such deployment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the related test applicability rule that is only applicable to intra-band contiguous CA with channel spacing less than the nominal channel spacing. - Added the performance tests for FDD intra-band contiguous CA with channel spacing less than the nominal channel spacing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE performance under intra-band contiguous CA with minimum channel spacing can’t be verified for FDD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.3, 8.2.1.10(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<The First Change>

8.1.2.3	Applicability and test rules for different CA configurations and bandwidth combination sets

The performance requirement for CA UE demodulation tests in Clause 8 are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.6A. For UEs supporting different CA configurations and bandwidth combination sets, the applicability and test rules are defined for the tests for 2 DL CCs in Table 8.1.2.3-1 and 3 or more DL CCs in Table 8.2.2.3-2. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 8.1.2.3-1: Applicability and test rules for CA UE demodulation tests with 2 DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capbility where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 2CCs in Clause 8.2.1.1.1, 8.2.1.4.3 | Any one of the supported CA capabilities | Any one of the supported FDD CA configurations | 10+10 MHz, 20+20 MHz, 5+5 MHz, 10MHz+5MHz, 15MHz+5MHz |
| CA tests with 2CCs in Clause 8.2.1.3.1 | Each supported CA capability | Any one of the supported FDD CA configurations in each CA capability | 10+10 MHz, 20+20 MHz, 5+5 MHz, 10MHz+5MHz, other combinations |
| CA tests with 2CCs in Clause 8.2.1.3.1A, 8.7.1 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported FDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.1.7.1 | CA_C | Supported FDD intra-band contiguous CA configurations covering the lowest and highest operating bands | Largest aggregated CA bandwidth combinations |
| CA tests with 2CCs in Clause 8.2.2.1.1, 8.2.2.4.3 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.2.3.1 | Each supported CA capability | Any one of the supported TDD CA configurations in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in Clause 8.2.2.3.1A, 8.7.2 | Any one of the supported CA capabilities with largest aggregated CA bandwidth | Any one of the supported TDD CA configurations with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 2CCs in 8.2.2.7.1 | CA_C | Supported TDD intra-band contiguous CA configurations covering the lowest and highest operating bands | Largest aggregated CA bandwidth combinations |
| CA tests with 2CCs in Clause 8.2.1.8.1 | CA_N | CA_3A-3A defined in Table 5.6A.1-3 | 10+10 MHz |
| CA tests with 2CCs in Clause 8.2.2.8.1 | CA2_C | CA_41C defined in Table 5.6A.1-1 | 20+20 MHz |
| CA tests in Clause 8.2.1.10.1 (NOTE 4) | CA2_C | CA_1C and CA_3C defined in Table 5.6A.1-1 | 20+20 MHz |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all tests. NOTE 4: For UE supporting minimum channel spacing for FDD, the operation conditions are as following: - Inter-band CA with ≥3 CCs, where Band 1 is with 1 CC and normal spacing (PCell) and Band 2 is with 2 intra-band contiguous CCs and minimum channel spacing (SCell), or - Band 1 is configured with 1CC and normal spacing, and UE accesses the CC on Band 1, and then handed over to Band 2 with 2 intra-band contiguous CCs and minimum channel spacing. |  |  |  |

<The Second Change>

8.2.1.10	Intra-band contiguous carrier aggregation with minimum channel spacing

The requirements in this section verify the ability of an UE supporting intra-band contiguous carrier aggregation with minimum channel spacing to demodulate the signal transmitted by the PCell and SCell(s). Throughput is measured on each cell. The minimum channel spacing of intra-band contiguous carrier aggregation refers to the possible minimum channel spacing as any multiple of 300 kHz less than the nominal channel spacing defined in 5.7.1A.

8.2.1.10.1	Minimum Requirement

For CA the requirements are specified in Table 8.2.1.10.1-2, with the addition of the parameters in Table 8.2.1.10.1-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.2.1.10.1-1: Test Parameters for CA

| Parameter |  | Unit | Test 1-2 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | 64QAM |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection |
| PDSCH transmission mode |  |  | 1 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.1.10.1-2: Single carrier performance for multiple CA configurations with minimum channel spacing

| Band-width | Reference channel | OCNG pattern | Propagation condition | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.6 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | [17.4] |
| 10MHz | R.7 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | [17.7] |
| 15MHz | R.8 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | [17.7] |
| 20MHz | R.9 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | [17.6] |

Table 8.2.1.10.1-3: Minimum performance (FRC) for intra-band CA with minimum channel spacing for CA with 2DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 20MHz+20MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 3 | 20MHz+10MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 4 | 20MHz+5MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 5 | 15MHz+15MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 6 | 10MHz+10MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| 7 | 5MHz+10MHz | As specified in Table 8.2.1.10.1-2 per CC | ≥5 |
| Note 1: The applicability and test rules of requirements for different CA configurations and bandwidth combination sets are defined in 8.1.2.3. |  |  |  |

<End of Changes>