3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2003732

# Electronic Meeting, 20 – 30 April, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS38.101-1 draft CR: adding 30KHz SSB SCS for n38 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For n38 only 15KHz SSB SCS is supported in the NR specification. But there would be two potential use cases where 30KHz SSB SCS is preferred. The first one is LTE-NR co-existence use case, where 30KHz SSB SCS should be used to avoid the collision with LTE CRS like for the other bands including n66/n5/n90. The second one is NR CA case with n38 aggregated with the other band(s) where 30KHz data SCS is utilized. For the second case, 30KHz SCS would be preferred on n38, because the feature of simultaneous reception on separate CCs with different numerologies in CA is optional. For those use cases, the configuration of 15KHz SCS SSB plus 30KHz SCS data would be problematic in the real deployment: - Firstly, the guard band is needed between SSB with 15KHz SCS and data with 30KHz SCS, which would cause the loss of spectrum efficiency, because SSB RE is not orthogonal with data RE and the duration of SSB and data symbols are unequal. - Secondly, UE has to stop reception of data from serving cell(s) on Band n38 when conducting measurement, which would lead to the long interruption and downlink throughput loss, because the simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement is an optional feature. Besides, there is no RRM requirement for some measurements for UE which does not support this optional feature. - Thirdly, the implementation complexity would be increased for BS, UE and test equipment, which may delay the deployment on n38, because the additional hardware is needed to process SSB and data, e.g., single FFT does not work and dual FFT is needed. - Fourthly, the 30KHz SCS SSB cannot avoid the collision with LTE CRS in LTE-NR downlink co-existence scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 30KHz SSB SCS for n38 in the synchronization raster configuration table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The features including intra-frequency measurement, CA with 30KHz data SCS configured on all the CCs and LTE-NR co-existence cannot be well supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 6.3B.1.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The change has no impact on ASN.1. In order to address the backward compatible issue, the solution with both 15KHz SSB and 30KHz SSB transmitted might be considered. But since for NSA, 30KHz SCS SSB should be supported on NR PSCell or SCell when the RRC signaling indicates the SSB SCS is 30KHz. It seems feasible for UE to support both 15KHz and 30KHz SCS SSB. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 5279 – <1> – 5419 |
| n2 | 15 kHz | Case A | 4829 – <1> – 4969 |
| n3 | 15 kHz | Case A | 4517 – <1> – 4693 |
| n5 | 15 kHz | Case A | 2177 – <1> – 2230 |
|  | 30 kHz | Case B | 2183 – <1> – 2224 |
| n7 | 15 kHz | Case A | 6554 – <1> – 6718 |
| n8 | 15 kHz | Case A | 2318 – <1> – 2395 |
| n12 | 15 kHz | Case A | 1828 – <1> – 1858 |
| n20 | 15 kHz | Case A | 1982 – <1> – 2047 |
| n25 | 15 kHz | Case A | 4829 – <1> – 4981 |
| n28 | 15 kHz | Case A | 1901 – <1> – 2002 |
| n34 | 15 kHz | Case A | 5030 – <1> – 5056 |
| n38 | 15 kHz | Case A | 6431 – <1> – 6544 |
|  | 30 kHz | Case C | [6437 – <1> – 6538] |
| n39 | 15 kHz | Case A | 4706 – <1> – 4795 |
| n40 | 15 kHz | Case A | 5756 – <1> – 5995 |
| n41 | 15 kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| n50 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n51 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n66 | 15 kHz | Case A | 5279 – <1> – 5494 |
|  | 30 kHz | Case B | 5285 – <1> – 5488 |
| n70 | 15 kHz | Case A | 4993 – <1> – 5044 |
| n71 | 15 kHz | Case A | 1547 – <1> – 1624 |
| n74 | 15 kHz | Case A | 3692 – <1> – 3790 |
| n75 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n76 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n77 | 30 kHz | Case C | 7711 – <1> – 8329 |
| n78 | 30 kHz | Case C | 7711 – <1> – 8051 |
| n79 | 30 kHz | Case C | 8480 – <16> – 8880 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8] |  |  |  |

<< End of change >>