# 3GPP TSG-RAN WG4 Meeting #94bis-e														R4-2003818

# Electronic meeting, 20 – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | <CR#> | rev | <Rev#> | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.141-1 with TPDR and EVM test model updates Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR conformance tests specification TS 38.141-1 for Total power dynamic range (TPDR) and Modulation quality (EVM) in respective test procedures includes description which test model should be used. However, due to quite many changes in these parts of test procedures and optimizations made, these texts are not always clear and easy to understand, also in same cases there is ambiguity and misalignment. This CR introduce changes to clause with test model (NR-FR1-TM2) and TPDR and EVM tests with changes that should made these parts of specification clear. All details for proposed changes are described in our discussion contribution [1]. [1] R4-200xxxx Discussion on EVM and TPDR tests in NR conformance specifications, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updates in TM-FR1-TM2 to allowed use this TM with lower modulation order then 64QAM. - Introduction of tables for test model to be used for TPDR and EVM tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification would be ambiguity and not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.2.2.3, 6.3.3.4, 6.3.3.4.2, 6.5.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.9.2.2.3	FR1 test model 2 (NR-FR1-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol TX power limit (OSTP) at min power)

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in clause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM2 are defined in table 4.9.2.2.3-1.

Table 4.9.2.2.3-1: Specific physical channel parameters of NR-FR1-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| # of PDSCH PRBs which are not allocated | NRB-1 |

For Total power dynamic range and Modulation quality tests, physical channel parameters are defined in table 4.9.2.2.3-1 with all 64AQM PDSCH PRBs, or all 64QAM PDSCH PRBs replaced by QPSK or 16QAM PDSCH PRBs depends on the highest modulation supported by BS as described in clause 6.3.3.4.2 and 6.5.3.4.2

<next modified section>

### 6.3.3	Total power dynamic range

#### 6.3.3.1	Definition and applicability

The BS total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

NOTE:	The upper limit of the total power dynamic range is the OFDM symbol TX power (OSTP) for a BS at maximum output power when transmitting on all RBs. The lower limit of the total power dynamic range is the average power for single RB transmission. The OFDM symbols shall carry PDSCH and not contain PDCCH, RS, or SSB.

#### 6.3.3.2	Minimum requirement

The minimum requirement applies per single-band connector, or per multi-band connector.

The minimum requirement for BS type 1-C and for BS type 1-H is in TS 38.104 [2], clause 6.3.3.2.

#### 6.3.3.3	Test purpose

The test purpose is to verify that the total power dynamic range is within the limits specified by the minimum requirement.

#### 6.3.3.4	Method of test

##### 6.3.3.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested:	M; see clause 4.9.1.

##### 6.3.3.4.2	Procedure

For BS type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector(s) under test as shown in annex D.1.1 for BS type 1-C and in annex D.3.1 for BS type 1-H. All connectors not under test shall be terminated.

2)	Set each connector under test to transmit according to the applicable test configuration in clause 4.8 using the corresponding test models in clause 4.9.2 at rated carrier output power Prated,c,AC for BS type 1-C and Prated,c,TABC for BS type 1-H (D.21).

3)	For BS type 1-C and BS type 1-H, set the BS to transmit a signal according to test models with the highest supported modulation order as described in table 6.3.3.4.2-1.

Table 6.3.3.4.2-1: Test models to be used for Total power dynamic range test (all PRBs) for BS type 1-C and BS type 1-H

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 256QAM without power back off | NR-FR1-TM3.1a |
| 256QAM with power back off | NR-FR1-TM3.1 |
| 64QAM | NR-FR1-TM3.1 |
| 16QAM | NR-FR1-TM3.2 |
| QPSK | NR-FR1-TM3.3 |

4)	Measure the OFDM symbol TX power (OSTP) as defined in the annex H.

5)	For BS type 1-C and BS type 1-H, set the BS to transmit a signal according to the highest supported by BS modulation order as described in table 6.3.3.4.2-2.

Table 6.3.3.4.2-2: Test models to be used for Total power dynamic range test (1 PRB) for BS type 1-C and BS type 1-H

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 256QAM without power back off | NR-FR1-TM2a |
| 256QAM with power back off | NR-FR1-TM2 |
| 64QAM | NR-FR1-TM2 |
| 16QAM | NR-FR1-TM2 with 16QAM PDSCH PRBs |
| QPSK | NR-FR1-TM2 with QPSK PDSCH PRBs |

6)	Measure the OFDM symbol TX power (OSTP) as defined in the annex H.

In addition, for multi-band connectors, the following steps shall apply:

7)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

<next modified section>

### 6.5.3	Modulation quality

#### 6.5.3.1	Definition and applicability

Modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector.

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

#### 6.5.3.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2], clause 6.5.2.2.

#### 6.5.3.3	Test purpose

The test purpose is to verify that modulation quality is within the limit specified by the minimum requirement.

#### 6.5.3.4	Method of test

##### 6.5.3.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: B, M and T; see clause 4.9.1.

RF bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW, MRFBW and TRFBW in single-band operation, see clause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.9.1.

##### 6.5.3.4.2	Procedure

The minimum requirement is applied to all antenna connectors or TAB connectors, they may be tested one at a time or multiple antenna connectors or TAB connectors may be tested in parallel as shown in annex D.1.1 for BS type 1-C or in annex D.3.1 for BS type 1-H. Whichever method is used the procedure is repeated until all antenna connectors or TAB connectors necessary to demonstrate conformance have been tested.

1)	For an antenna connector or TAB connector declared to be capable of single carrier operation only (D.16), set the antenna connector or the TAB connector under test to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models with the highest supported by BS modulation order as described in table 6.5.3.4.2-1.

Table 6.5.3.4.2-1: Test models to be used for Modulation quality test (all PRBs) for BS type 1-C and BS type 1-H

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 256QAM without power back off | NR-FR1-TM3.1a |
| 256QAM with power back off | NR-FR1-TM3.1a at manufacturer's declared rated output power, and NR-FR1-TM3.1 with 64QAM at maximum power, |
| 64QAM | NR-FR1-TM3.1 |
| 16QAM | NR-FR1-TM3.2 |
| QPSK | NR-FR1-TM3.3 |

For an antenna connector or TAB connector declared to be capable of multi-carrier and/or CA operation (D.15-D.16), set the antenna connector or the TAB connector under test to transmit according to the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models on all carriers configured with the highest supported by BS modulation order as described in table 6.5.3.4.2-1.:

2)	Measure the EVM and frequency error as defined in annex H.

3)	Repeat steps 1 and 2 by setting the BS to transmit a signal according to rules described in table 6.5.3.4.2-3.

Table 6.5.3.4.2-3: Test models to be used for Modulation quality test (1 PRB) for BS type 1-C and BS type 1-H

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 256QAM without power back off | NR-FR1-TM2a |
| 256QAM with power back off | NR-FR1-TM2a |
| 64QAM | NR-FR1-TM2 |
| 16QAM | NR-FR1-TM2 with 16QAM PDSCH PRBs |
| QPSK | NR-FR1-TM2 with QPSK PDSCH PRBs |

For NR-FR1-TM2 and NR-FR1-TM2a the OFDM symbol TX power (OSTP) shall be at the lower limit of the dynamic range according to the test procedure in clause 6.3.3.4 and test requirements in clause 6.3.3.5.

In addition, for multi-band connector(s), the following steps shall apply:

4)	For multi-band connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.