# 3GPP TSG-RAN WG4 Meeting #94bis-e														R4-2003819

# Electronic meeting, 20 – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | <CR#> | rev | <Rev#> | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.141-2 with TPDR and EVM test model updates Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR conformance tests specification TS 38.141-2 for OTA total power dynamic range (TPDR) and OTA modulation quality (EVM) in respective test procedures includes description which test model should be used. However, due to quite many changes in these parts of test procedures and optimizations made, these texts are not always clear and easy to understand, also in same cases there is ambiguity and misalignment. This CR introduce changes to clause with test model (NR-FR2-TM2 and NR-FR2-TM3.1) and OTA TPDR and OTA EVM tests with changes that should made these parts of specification clear. All details for proposed changes are described in our discussion contribution [1]. [1] R4-200xxxx Discussion on EVM and TPDR tests in NR conformance specifications, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updates in TM-FR2-TM2 and NR-FR2-TM3.1 to allowed use these TMs with lower modulation order then 64QAM. - Introduction of tables for test model to be used for OTA TPDR and OTA EVM tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification would be ambiguity and not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.2.2.2, 4.9.2.2.3, 6.4.3.4.2, 6.6.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.141-1 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.9.2.2.2	NR FR2 test model 2 (NR-FR2-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol TX power limit (OSTP) at min power)

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in clause 4.9.2.2. Specific physical channel parameters for NR-FR2-TM2 are defined in table 4.9.2.2.2-1.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| # of PDSCH PRBs which are not allocated | NRB-1 |

For OTA total power dynamic range and OTA modulation quality tests, physical channel parameters are defined in table 4.9.2.2.3-1 with all 64AQM PDSCH PRBs, or all 64QAM PDSCH PRBs replaced by QPSK or 16QAM PDSCH PRBs depends on the highest modulation order supported by BS as described in clause 6.4.3.4.2 and 6.6.3.4.2.

##### 4.9.2.2.3	NR FR2 test model 3.1 (NR-FR2-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol TX power limit (OSTP) at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

NOTE:	EVM shall be evaluated over PDSCH allocated PRBs with nRNTI=0 and nRNTI=2.

Common physical channel parameters are defined in clause 4.9.2.2. Specific physical channel parameters for NR-FR2-TM3.1 shall be defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 64QAM.

For OTA total power dynamic range and OTA modulation quality tests, physical channel parameters are defined in table 4.9.2.2.1-1 with all 64QAM PDSCH PRBs, or all 64QAM PDSCH PRBs replaced by 16QAM or QPSK PDSCH PRBs depends on the highest modulation order supported by BS as described in clause 6.4.3.4.2 and 6.6.3.4.2.

### <next modified section>

### 6.4.3	OTA total power dynamic range

#### 6.4.3.1	Definition and applicability

The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE:	The upper limit of the OTA total power dynamic range is the BS maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbols shall carry PDSCH and not contain PDCCH, RS or SSB.

#### 6.4.3.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], clause 9.4.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], clause 9.4.3.3.

#### 6.4.3.3	Test purpose

The test purpose is to verify that the total power dynamic range is within the limits specified by the minimum requirement.

#### 6.4.3.4	Method of test

##### 6.4.3.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

Beams to be tested:	Declared beam with the highest intended EIRP for the narrowest intended beam corresponding to the smallest BeWθ, or for the narrowest intended beam corresponding to the smallest BeWϕ (D.3, D.11).

Directions to be tested: The OTA peak directions set reference beam direction pair (D.8).

##### 6.4.3.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	For BS type 1-O, set the BS to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models with the highest supported by BS modulation order as described in table 6.4.3.4.2-1.

-

Table 6.4.3.4.2-1: Test models to be used for OTA total power dynamic range test (all PRBs) for BS type 1-O

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 256QAM without power back off | NR-FR1-TM3.1a in TS 38.141-1 [3] |
| 256QAM with power back off | NR-FR1-TM3.1 in TS 38.141-1 [3] |
| 64QAM | NR-FR1-TM3.1 in TS 38.141-1 [3] |
| 16QAM | NR-FR1-TM3.2 in TS 38.141-1 [3] |
| QPSK | NR-FR1-TM3.3 in TS 38.141-1 [3] |

For BS type 2-O, set the BS to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models with the highest supported by BS modulation order as described in table 6.4.3.4.2-2.

6)	Measure the OFDM symbol TX power as defined in annex L by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	For BS type 1-O, set the BS to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models with the highest supported by BS modulation order as described in table 6.4.3.4.2-3.

Table 6.4.3.4.2-3: Test models to be used for OTA total power dynamic range test (1 PRB) for BS type 1-O

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 256QAM without power back off | NR-FR1-TM2a in TS 38.141-1 [3] |
| 256QAM with power back off | NR-FR1-TM2 in TS 38.141-1 [3] |
| 64QAM | NR-FR1-TM2 in TS 38.141-1 [3] |
| 16QAM | NR-FR1-TM2 with 16QAM PDSCH PRBs in TS 38.141-1 [3] |
| QPSK | NR-FR1-TM2 with QPSK PDSCH PRBs in TS 38.141-1 [3] |

For BS type 2-O, set the BS to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models with the highest supported by BS modulation order as described in table 6.4.3.4.2-4.

Table 6.4.3.4.2-4: Test models to be used for OTA total power dynamic range test (1 PRB) for BS type 2-O

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 64QAM without power back off | NR-FR2-TM2 |
| 64QAM with power back off | NR-FR2-TM2 |
| 16QAM | NR-FR2-TM2 with 16QAM PDSCH PRBs |
| QPSK | NR-FR2-TM2 with QPSK PDSCH PRBs |

8)	Measure the OFDM symbol TX power (OSTP) as defined in annex L by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

The measured OFDM symbols shall not contain RS or SSB.

In addition, for multi-band RIB(s), the following steps shall apply:

9)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

<next modified section>

### 6.6.3	OTA modulation quality

#### 6.6.3.1	Definition and applicability

OTA modulation quality is defined by the difference between the measured carrier signal and an idealsignal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector.

OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 6.6.3.2	Minimum Requirement

The minimum requirement for BS type 1-O, is in TS 38.104 [2], clause 9.6.2.2.

The minimum requirement for BS type 2-O, is in TS 38.104 [2], clause 9.6.2.3.

#### 6.6.3.3	Test purpose

The test purpose is to verify that OTA modulation quality is within the limit specified by the minimum requirement.

#### 6.6.3.4	Method of test

##### 6.6.3.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier:

-	B and T; see clause 4.9.1.

Base station RF bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW and TRFBW in single-band operation, see clause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.9.1.

Directions to be tested:

-	The OTA coverage range reference direction (D.35).

-	The OTA coverage range maximum directions (D.36).

Polarizations to be tested: For dual polarized systems the requirement shall be tested and met for both polarizations.

##### 6.6.3.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beamforming settings of the BS according to the direction to be tested.

5)	Set the BS to output according to the applicable test configuration in clause 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

For BS type 1-O declared to be capable of single carrier operation only, set the BS to transmit a signal according to the highest supported by BS modulation order as described in table 6.6.3.4.2-1.

Table 6.6.3.4.2-1: Test models to be used for OTA modulation quality test (all PRBs) for BS type 1-O

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 256QAM without power back off | NR-FR1-TM3.1a in TS 38.141-1 [3] |
| 256QAM with power back off | NR-FR1-TM3.1a at manufacturer's declared rated output power (PRated,c,EIRP) in TS 38.141-1 [3], and NR-FR1-TM3.1 with 64QAM at maximum power (PRated,c,EIRP) in TS 38.141-1 [3] |
| 64QAM | NR-FR1-TM3.1 in TS 38.141-1 [3] |
| 16QAM | NR-FR1-TM3.2 in TS 38.141-1 [3] |
| QPSK | NR-FR1-TM3.3 in TS 38.141-1 [3] |

For BS type 1-O declared to be capable of multi-carrier and/or CA operation, set the BS to transmit according to the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test models on all carriers configured with the highest supported by BS modulation order as described in table 6.6.3.4.2-1.

For BS type 2-O declared to be capable of single carrier operation only, set the BS to transmit a signal according to the applicable test signal configuration and corresponding power setting specified in clause 4.7.2 and 4.8 using the corresponding test models on all carriers configured with the highest supported by BS modulation order as described in table 6.6.3.4.2-2.

Table 6.6.3.4.2-2: Test models to be used for OTA modulation quality test (all PRBs) for BS type 2-O

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 64QAM without power back off | NR-FR2-TM3.1 |
| 64QAM with power back off | NR-FR2-TM3.1 at manufacturer's declared rated output power (PRated,c,EIRP), and NR-FR2-TM3.1 with 16QAM at maximum power (PRated,c,EIRP) |
| 16QAM | NR-FR2-TM3.1 with 16QAM PDSCH PRBs |
| QPSK | NR-FR2-TM3.1 with QPSK PDSCH PRBs |

For BS type 2-O declared to be capable of multi-carrier and/or CA operation, set the BS to transmit according to the highest supported by BS modulation order as described in table 6.6.3.4.2-2.

6)	For each carrier, measure the EVM and frequency error as defined in annex L.

7)	For BS type 1-O repeat steps 5 and 6 by setting the BS to transmit a signal according to rules described in table 6.6.3.4.2-3.

Table 6.6.3.4.2-3: Test models to be used for OTA Modulation quality test (1 PRB) for BS type 1-O

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 256QAM without power back off | NR-FR1-TM2a |
| 256QAM with power back off | NR-FR1-TM2 |
| 64QAM | NR-FR1-TM2 |
| 16QAM | NR-FR1-TM2 with 16QAM PDSCH PRBs |
| QPSK | NR-FR1-TM2 with QPSK PDSCH PRBs |

For NR-FR1-TM2 and NR-FR1-TM2a the OFDM symbol power (in the conformance direction) shall be at the lower limit of the dynamic range according to the test procedure in clause 6.4.3.4.2 and test requirements in clause 6.4.3.5.1.

For BS type 2-O repeat steps 5 and 6 by setting the BS to transmit a signal according to rules described in table 6.6.3.4.2-4.

Table 6.6.3.4.2-4: Test models to be used for OTA Modulation quality test (1 PRB) for BS type 2-O

| The highest modulation supported by BS | Test model to be used |
| --- | --- |
| 64QAM without power back off | NR-FR2-TM2 |
| 64QAM with power back off | NR-FR2-TM2 |
| 16QAM | NR-FR2-TM2 with 16QAM PDSCH PRBs |
| QPSK | NR-FR2-TM2 with QPSK PDSCH PRBs |

For NR-FR2-TM2 the OFDM symbol power (in the conformance direction) shall be at the lower limit of the dynamic range according to the test procedure in clause 6.4.3.4.2 and test requirements in clause 6.4.3.5.2.

In addition, for multi-band RIB, the following steps shall apply:

8)	For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.