3GPP TSG-RAN WG4 Meeting #94-e-Bis		R4-2003831

Electronic meeting, 20th April – 30th April, 2020

Agenda item:	6.17.2.1.4

Source:	Ericsson

Title:	Simulation results for HST Multipath fading channels

Document for:	Discussion

# Introduction

In this paper we provide simulation results based on the assumptions found in [1]. We also propose not to schedule PDSCH transmission in the special slots for TDD test cases.

# Simulations

## Test setup

The simulation assumptions from [1].

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 4; MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank = 1 and 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 600Hz | 1200Hz |
| Testing metric | SNR @70% of maximum throughput |  |

# Simulation results

Based on the simulation assumptions, we simulate FDD test cases, and for TDD only transmitting in the 7 downlink slots. Simulation results shown in Figure 2-1, and 2-2 for FDD, and TDD respectively.

Figure 2-1: FDD simulations of Multi-path channel according to simulation assumptions

Figure 2-2: TDD simulations of Multi-path channel with special slot transmission turned off for PDSCH

There are two simulations that does not reach the maximum throughput within the sweeping interval from -10dB to 30dB SNR, which are 2Tx-2Rx MCS = 13 Rank = 2 and 2Tx-2Rx MCS = 17 Rank = 2, both TDD configurations.

# Results

In Table 3-1, and 3-2 the results are presented for FDD, and TDD respectively.

Table 3-1 SNR at 70% throughput for FDD test cases

| Test | Duplex mode BW / SCS Max Doppler | Antennas | Rank | MCS | SNR @ 70% maximum Throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD 10MHz / 15kHz 600Hz | 2Tx2Rx | 1 | 4 | -2.72 |
| 2 |  |  |  | 13 | 6.22 |
| 3 |  |  |  | 17 | 10.05 |
| 4 |  |  | 2 | 4 | 1.66 |
| 5 |  |  |  | 13 | 13.77 |
| 6 |  |  |  | 17 | 17.11 |
| 7 |  | 2Tx4Rx | 1 | 4 | -5.99 |
| 8 |  |  |  | 13 | 2.70 |
| 9 |  |  |  | 17 | 6.51 |
| 10 |  |  | 2 | 4 | -2.41 |
| 11 |  |  |  | 13 | 7.65 |
| 12 |  |  |  | 17 | 11.91 |

Table 3-2 SNR at 70% throughput for TDD test cases

| Test | Duplex mode BW / SCS Max Doppler | Antennas | Rank | MCS | SNR @ 70% maximum Throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | TDD 40MHz / 30kHz 1200Hz | 2Tx2Rx | 1 | 4 | -2.27 |
| 2 |  |  |  | 13 | 7.03 |
| 3 |  |  |  | 17 | 10.38 |
| 4 |  |  | 2 | 4 | 3.08 |
| 5 |  |  |  | 13 | N/A |
| 6 |  |  |  | 17 | N/A |
| 7 |  | 2Tx4Rx | 1 | 4 | -5.45 |
| 8 |  |  |  | 13 | 3.5 |
| 9 |  |  |  | 17 | 6.76 |
| 10 |  |  | 2 | 4 | -0.94 |
| 11 |  |  |  | 13 | 8.77 |
| 12 |  |  |  | 17 | 12.56 |

Observation 1: There are two tests that do not achieve maximum throughput, TDD test case 5 and 6 in Table 3-2.

Proposal 1: For PDSCH with TDD configuration, we shall not schedule data in the special slot in order to achieve maximum throughput.

Proposal 2: For HST Multipath fading channels, we ask RAN4 to set requirements using MCS 17 with Rank 1.

# Conclusions

In this contribution we have evaluated HST Multi-path fading demodulation performance and we observe and propose:

Observation 1: There are two tests that do not achieve maximum throughput, TDD test case 5 and 6 in Table 3-2.

Proposal 1: For PDSCH with TDD configuration, we shall not schedule data in the special slot in order to achieve maximum throughput.

Proposal 2: For HST Multipath fading channels, we ask RAN4 to set requirements using MCS 17 with Rank 1.

# References

- R4-2002418 “WF on UE demodulation for NR HST”, CMCC, RAN4 #94-e, February 2020.