3GPP TSG-RAN WG4 # 94Bis-e	R4-2003835

Electronic Meeting, 20 – 30 Apr., 2020

Source:	Qualcomm Incorporated

Title:	TP for TR 37.716-11-11 for 39A_n257

Agenda item: 	8.3.3

Document for:	Approval

# 1.	Introduction

This contribution is a TP for TR 37.716-11-11 to introduce the combinations DC_39A_257(A, D-M) which are proposed in WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band).

# 2. 	Text Proposal

----- Unchanged sections omitted -----

## 6.2.X	DC_39_n257

### 6.2.X.1	Operating bands for DC

Table 6.2.X.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_39_n257 | 39 | n257 | No |

### 6.2.X.2	Configuration for DC

Table 6.2.X.2-1: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR band |
| --- | --- | --- | --- |
| DC_39A_n257A DC_39A_n257D DC_39A_n257E DC_39A_n257F DC_39A_n257G DC_39A_n257H DC_39A_n257I DC_39A_n257J DC_39A_n257K DC_39A_n257L DC_39A_n257M | DC_39A_n257A | 39A | n257A n257D n257E n257F n257G n257H n257I n257J n257K n257L n257M |

#### 6.2.X.3	Spurious emission band UE co-existence for DC

Table 6.2.X.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_39A_n257A | E-UTRA Band 1, 8, 22, 26, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 2, 3, 4 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 4: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

#### 6.2.X.4	MSD analysis for DC

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.2.X.5	∆TIB and ∆RIB values

For DC_39_n257, the TIB,c and RIB values are given in the tables below.

Table 6.2.X.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_39_n257 | 39 | 0 |
|  | n257 | 0 |

Table 6.2.X.5-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_39_n257 | 39 | 0 |
|  | n257 | 0 |

----- Unchanged sections omitted -----