3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2003869

# Electronic meeting, 20 – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | <CR#> | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Pcmax for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The defintion of PCMAX for CA as a sum of the powers configured for activated serving cells (component carriers) could be interpreted as a requirement that the PCMAX should determined interatively to ensure that the total EIRP (in a different plane of reference than PCMAX) is below the power class. This is not the intention: the defintion should be such that only the powers configured for activated cells with an uplink grant are included in the PCMAX, that is, not necessarily including all configured cells. The defintion of the total measured power PUMAX includes up to n carrier frequencies f, per srving cell c; there can only be one carrier frequency f per serving cell c for FR2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The defintion of the PCMAX is modified to state that the PCMAX for CA is determined by the powers configured for activated serving cells with non-zero transmission grants. The defintion of the total measured power PUMAX is corrected such that the sum only includes one carrier frequency f = f(c) per serving cell c. References to the relevant subclauses for MPR and A-MPR are added. The same MPR and A-MPR applies for all serving cells for intra-band CA (uplink intra-band CA not specified in Rel-15). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification: the defintion of PCMAX for CA as a sum of the powers configured for activated serving cells could be interpreted as a requirement that the PCMAX should determined interatively to ensure that the total EIRP is below the power class. Incorrect specification of the total measured configured power PUMAX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2A.4	Configured transmitted power for CA

A UE configured with carrier aggregation can configure its maximum output power PCMAX,f,c for each uplink carrier f of activated serving cell c and its total configured output power PCMAX. The definition of the configured UE maximum output power PCMAX,f,c for each carrier f of a serving cell c is used for power headroom reporting for carrier f of serving cell c only and is in accordance with that specified in clause 6.2.4 with parameters MPR, A-MPR and P-MPR replaced with those specified in subclause 6.2A.2, 6.2A.3 and 6.2.4, respectively. The total configured power PCMAX in a transmission occasion is determined by the power configured for carriers f of serving cells c with non-zero granted transmission power in their respective reference points.

PCMAX is calculated under the assumption that power spectral density for each RB in each component carrier is the same.

The total configured UE maximum output power PCMAX shall be set such that the corresponding measured total peak EIRP PUMAX is within the following bounds

PPowerclass – MAX(MAX(MPR, A_MPR),P-MPR) – MAX{T(MAX(MPR, A_MPR)),T(P-MPR)} ≤ PUMAX ≤ EIRPmax

with PPowerclass the UE power class as specified in sub-clause 6.2A.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2A.1, MPR as specified in sub-clause 6.2A.2, A-MPR as specified in sub-clause 6.2A.3, P-MPR the power management term for the UE as described in 6.2.4 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2A.1.

The total measured configured power PUMAX is defined as

PUMAX=10log10c,f(c)pUMAX,f,c

where pUMAX,f,c is the linear value of the measured power PUMAX,f,c for carrier f = f(c) of serving cell c.

The tolerance T(ΔP) for applicable values of ΔP (values in dB) is specified in Table 6.2A.4-1.

Table 6.2A.4-1: PUMAX tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n260, n261 | P = 0 | 0 |
|  | 0 < P ≤ 2 | 1.5 |
|  | 2 < P ≤ 3 | 2.0 |
|  | 3 < P ≤ 4 | 3.0 |
|  | 4 < P ≤ 5 | 4.0 |
|  | 5 < P ≤ 10 | 5.0 |
|  | 10 < P ≤ 15 | 7.0 |
|  | 15 < P ≤ X | 8.0 |
| NOTE: X is the value such that Pumax lower bound, PPowerclass - P – T(P) = minimum output power specified in clause 6.3A.1 |  |  |

## 6.2D	Transmitter power for UL MIMO

< end of changes >