3GPP TSG-RAN WG4 Meeting #94b-e	R4-2003958

Electronic Meeting, 20th Apr - 30th Apr, 2020

Source:	ZTE Corporation

Title:	Further discussion on NR HST BS demodulation requirements

Agenda Item:	6.17.2.2

Document for:	Discussion

# Introduction

In RAN4#94e, a way forward was discussed on NR HST BS demodulation requirements [1], and it was agreed except slide 16 [2]:

Agreements: The content in WF R4-2002405 except slide 16 is agreed with additional agreements as following: • TDLC300-100 propagation conditions for short preamble formats and long preamble formats o Do not introduce TDLC300-100 fading channel with frequency offset of 400Hz requirements for short preamble formats as they are already defined in “normal mode” PRACH. Remove the cases from the simulation result summary template. o FFS on whether not to introduce TDLC300-100 fading channel with frequency offset of 400Hz requirements for long preamble formats.

The main open issues are listed below:

Open issues:

For UL TA:

In this contribution, we provide our views on these open issues.

# Discussion

TDLC300-100 for preamble

TDLC300-100 for short preamble was excluded due to the fact that they are already defined in “normal mode” PRACH. Table 8.4.2.2-1 also defines missed detection for TDLC300-100 with long preamble, so it can also be excluded under the similar consideration.

Proposal 1: Do not introduce requirements for TDLC300-100 with 400 frequency offset for long preambles based on the similar consideration as short preambles.

Organization of HST PUSCH requirements in specs

In LTE, a separate section was created for high speed train scenarios. It is natural to follow similar way. Furthermore, most of parameters in respect to 350km/h and 500km/h are the same, so a common table for both speeds is preferred for the sake of readability and conciseness of specs.

Proposal 2: Create a new section capturing requirements for HST PUSCH, and one common table for both 350km/h and 500km/h.

Similar consideration can apply to HST PRACH.

Proposal 3: Create a new section capturing requirements for HST PRACH, and one common table for both 350km/h and 500km/h.

Declaring support of two speeds and testing

Normally, if a BS declare support of HST PUSCH, HST PRACH and UL TA for 350km/h, then the corresponding requirements should be tested. And this is the case for 500km/h.

But there is one essential issue: should 350km/h and 500km/h be treated independently for both declaration and testing? A BS declaring to support 500km/h will reasonably support a lower speed 350km/h, thus declaration support of 500km/h implies its support of 350km/h automatically. Therefore, we propose to slightly change the declared item as “maximum supported speed”, which may be either 350km/h or 500km/h.

Proposal 4: Introduce a new declared item “Maximum supported speed”, either 350km/h or 500km/h, for HST PUSCH, HST PRACH and UL TA.

If a BS declares its maximum supported speed as 500km/h, then only 500km/h requirements should be tested, and if a BS declares it maximum supported speed as 350km/h, then only 350km/h requirements should be tested. In this way test efforts can be reduced as well.

Proposal 5: Only requirements corresponding to the declared maximum supported speed should be tested.

Introduction of 1T1R requirement in tunnel scenario

It was confirmed by some operator that 1T1R in tunnel scenario in real deployments. In this case, some performance requirements should be introduced, but for the sake of reducing testing efforts, only conducted tests are intended, therefore, option 3 would be a good compromise.

Proposal 6: Introduce IT1R requirements in tunnel scenario but do not cover OTA test.

Test metric for UL TA

There is one concern that the test metric for UL TA should take into account the estimated timing offset error. But this is actually an implementation issue. Even there could be some receivers which do not need to estimate the timing offset at all. So from standardization perspective, it is not necessary to specify a test metric associated with a timing offset estimation error.

Proposal 7: Keep test metric for UL TA as it is now.

# Conclusion

In this paper, we have the following proposals for open issues on HST BS demodulation requirements:

Proposal 1: Do not introduce requirements for TDLC300-100 with 400 frequency offset for long preambles based on the similar consideration as short preambles.

Proposal 2: Create a new section capturing requirements for HST PUSCH, and one common table for both 350km/h and 500km/h.

Proposal 3: Create a new section capturing requirements for HST PRACH, and one common table for both 350km/h and 500km/h.

Proposal 4: Introduce a new declared item “Maximum supported speed”, either 350km/h or 500km/h, for HST PUSCH, HST PRACH and UL TA.

Proposal 5: Only requirements corresponding to the declared maximum supported speed should be tested.

Proposal 6: Introduce IT1R requirements in tunnel scenario but do not cover OTA test.

Proposal 7: Keep test metric for UL TA as it is now.

# Reference

R4-2002405, WF on Rel-16 NR HST BS demodulation requirements, Nokia

RAN4#94e BS RF Test Demod chairman report

R4-2002523, Email discussion summary for RAN4#94e_#94_NR_HST_Demod_BS, Nokia