3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2004112

Electronic Meeting, 20 April – 30 April, 2020

Source:	ZTE

Title:	Further discussion on mandatory of measurement gap patterns for NR

Agenda item:	6.15.1.6

Document for:	Discussion

1. Introduction

In the RAN4#94-e meeting, there were extensive discussions on mandatory of measurement gap patterns for NR in Rel-16. The agreements and open issues were captured in the way forward [1].

In this contribution we further provide our views on open issues for mandatory of measurement gap patterns in R16.

2. Discussion

2.1 UE capability and applicability

During the discussion in the last e-meeting, it was agreeable if only NR SA and NR DC scenarios are being considered that NR-only measurement means the target measurement objects to be measured within the measurement gap are all NR carriers. So for the two scenarios, we can conclude as follows.

Observation 1: To mandate additional gap patterns, the UE capability for NR only measurements needs to be introduced and at least is applicable to NR SA and NR DC.

- NR-only measurement means the target measurement objects to be measured within the measurement gap are all NR carriers.

- The UE capability is to indicate if the gap patterns from GP#2 to GP#11 can only be used to do NR only measurements.

Then there are very controversial issues when mandating additional gap patterns in EN-DC, NE-DC and LTE SA scenarios where LTE serving cell(s) are involved. The fundamental question is that UE implementation of handling short measurement gap depending on UE capability. We will further discuss from multi-dimensional analysis.

Case A: LTE measurements are configured

For UE not supporting shortMeasurementGap-r14, UE cannot measure LTE carriers by using measurement gap with MGL <   6ms when there is LTE measurements being configured. Therefore the measurement gap with MGL<6ms cannot be used for any measurements since UE capability (shortMeasurementGap-r14, measGapPatterns-r15) would indicate these gap patterns are not supported.

Case B: When NR only measurements are configured

If there is no LTE measurements, the UE not supporting shortMeasurementGap-r14 may also be able to handle measurement gap with MGL<6ms at LTE side as UE only needs to deal with interruptions on LTE serving cell during short gap length.

For the UE that cannot handle shorter measurement gap even when there is not LTE measurements, there were solutions being provided in [2] that LTE UE baseband can take the gap length as 6ms even if gap with shorter gap length is configured. This is also captured in the WF [1] as option 5 under the bullet ‘applicable scenarios for additional mandatory gap patterns’. The question is such solution requires standardization efforts in 36.133 as current scheduling restriction for measurement gap are aligned with gap length. Furthermore this may also require UE implementation change at LTE side in our view.

Observation 2: UE implementation of handling short measurement gap depending on UE capability.

Furthermore we can differentiate the scenarios with LTE serving cells further.

Scenario A: LTE and EN-DC

In these two scenarios per-UE gap pattern and per-FR gap on FR1 are configured by LTE PCell. NW reads UE capability of supported gap patterns via shortMeasurementGap-r14 and measGapPatterns-r15. In current spec UE will indicate supporting GP#2 and GP#3 via shortMeasurementGap-r14 only when the UE can do LTE measurements with short gap patterns because UE has no idea of how the gap pattern will be used before UE reports the capability. So UE capability of NR only measurement can help in this scenario so that the UE not supporting short measurement gap for LTE measurements can indicate supporting GP#2 and GP#3 for NR only measurement. It’s just this requires change in LTE RRC signaling and there was concern on changing LTE RRC signaling.

Observation 3. Mandating additional gap patterns and introducing UE capability for NR only measurement in LTE SA and EN-DC need change in LTE RRC signaling.

Scenario B: NE-DC

In this scenario per-UE gap pattern and per-FR gap are all configured by NR PCell. NW reads UE capability of supported gap patterns via supportedGapPattern. So there is no need to change LTE RRC signaling when UE capability for NR only measurement is introduced which would be in NR RRC. It is UE’s responsibility to ensure the consistency of supported measurement gap patterns at LTE side and NR side.

Observation 4. Mandating additional gap patterns and introducing UE capability for NR only measurement in NE-DC does not need change in LTE RRC signaling.

In addition it is helpful to analyze how different type of UEs behave in scenario A and scenario B. Based on analysis of Case A and Case B, three type of UE are identified.

Type A:  UE supporting short gap for LTE measurements

The UE reports additional supported gap patterns via shortMeasurementGap-r14 and measGapPatterns-r15.

For this type of UE, mandating, e.g. GP#2 and GP#3 if it is indicated being supported, is feasible in both scenario A and scenario B as UE can do LTE measurement with short gap.

UE capability for NR only measurement is not necessary as UE will always indicate GP#2 and GP#3 can be used for LTE and NR measurements.

It is noted that other gap patterns from GP#4 to GP#11 is also applicable if it is supported by indicating via measGapPatterns-r15

Type B:  UE supporting NR only measurement

The UE may not report additional supported gap patterns via shortMeasurementGap-r14 and measGapPatterns-r15 as it requires the UE to do LTE measurements with short gap. But if the UE is configured with NR only measurement, additional gap patterns can be supported.

For this type of UE, mandating, e.g. GP#2 and GP#3, is feasible in both scenario A and scenario B.

UE capability for NR only measurement is necessary so that NW will not configure GP#2 and GP#3 when there is LTE measurements.

Type C:  UE cannot supporting any measurements with short gap

This type of UE can only handle measurement gap with 6ms MGL (maybe only together with 40ms and 80ms MGRP) at LTE side. It is not be able to handle short measurement gap even if the measurement is NR only.

For this type of UE, mandating, e.g. GP#2 and GP#3, is NOT feasible in both scenario A and scenario B.

UE capability for NR only measurement would not help for this type of UE. To support any short gap would mean modification on LTE implementation.

Observation 5. Mandating additional gap patterns and necessity of UE capability of NR only measurement would be different for different type UEs.

In general the key issue is to address how to mandating gap patterns for type C UE. There were approaches being discussed in the last e-meeting but no consensus has been reached. The different approaches are analyzed further for scenario A and scenario B separately as follows.

For scenario A

Approach A1. No further normalization efforts in Rel-16.

GP#2 and GP#3 (also other gap patters from GP#4 to GP#11 if it is supported by indicating via measGapPatterns-r15) can be configured for both LTE and NR measurements for Type A UE.

GP#2 and GP#3 cannot be configured for Type B UE.

No implementation modification is needed to Type C UE.

Approach A2. Mandating additional gap patterns per UE capability

Mandating additional gap patterns for Type A and Type B UE is possible. The UE capability can be either per-FR gap capability as proposed by company or new UE capability.

Let’s take GP#2 and GP#3 to be mandated for example. UE will set shortMeasurementGap-r14 to indicate GP#2 and GP#3 are supported. Then UE capability of NR only measurement will indicate whether GP#2 and GP#3 can be used for NR only measurement or both LTE and NR measurements. Further the per-FR gap capability or new UE capability (depending on outcome of further discussion) will indicate if GP#2 and GP#3 is actually supported (Type C UE may indicate GP#2 and GP#3 are not supported actually).

No implementation modification is needed to Type C UE.

Approach A3. Mandating additional gap patterns and introducing UE capability for NR only measurements with side condition

Side condition as follows is needed for this approach.

- 6ms gap would be used for the LTE serving cell, but the short gap could still be used with the NR serving cell

As we discussed above this may needs additional standardization efforts, e.g. modifying scheduling restriction requirements, and may also needs UE implementation modification.

However if this is agreeable then mandating GP#2 and GP#3 (and others in FR1 if it is agreed) for all the three type of UEs is feasible. Moreover we think additional UE capability is needed as this side condition only works for Type C UE. It is not necessary for Type A and Type B UE. So additional UE capability could reduce extra implementation burden for Type A/B UE and improve system performance.

For scenario B, two options, i.e. approach B1 and B3, were discussed in the last e-meeting.

Approach B1. Mandating additional gap patterns per UE capability of shortMeasurementGap-r14

GP#2 and GP#3 (also other gap patters from GP#4 to GP#11 if it is supported by indicating via measGapPatterns-r15) can be mandated for both LTE and NR measurements for Type A UE.

GP#2 and GP#3 cannot be mandated for Type B and Type C UE.

No implementation modification is needed to Type C UE.

Since UE capability shortMeasurementGap-r14 is LTE RRC signaling and not known to NR PCell by which the measurement gap is configured, similar UE capability should be introduced in NR RRC.

Approach B2. Mandating additional gap patterns per new UE capability

GP#2 and GP#3 can be configured for NR only measurement for Type A and Type B UE.

GP#2 and GP#3 cannot be configured for NR only measurement for Type C UE

No implementation modification is needed to Type C UE.

Approach B3. Mandating additional gap patterns and introducing UE capability for NR only measurements with side condition

Side condition as follows are needed for this approach.

- 6ms gap would be used for the LTE serving cell, but the short gap could still be used with the NR serving cell

As we discussed above this may needs additional standardization efforts, e.g. modifying scheduling restriction requirements, and may also needs UE implementation modification.

However if this is agreeable then mandating GP#2 and GP#3 (and others in FR1 if it is agreed) for all the three type of UEs is feasible. Moreover we think additional UE capability is needed as this side condition only works for Type C UE. It is not necessary for Type A and Type B UE. So additional UE capability could reduce extra implementation burden for Type A/B UE and improve system performance.

As above analysis it can be summarized in Table 1 and Table 2 for scenario A and scenario B separately.

Table 1. Summary for scenario A

|  |  | Approach A1 | Approach A2Note 1 | Approach A3Note 2 |
| --- | --- | --- | --- | --- |
| Type A | If mandating additional gap patterns is feasible or necessary | Not necessary | Yes | Yes |
|  | If UE capability for NR only measurement is necessary | No | Yes | Yes |
| Type B | If mandating additional gap patterns is feasible or necessary | No | Yes | Yes |
|  | If UE capability for NR only measurement is necessary | No | Yes | Yes |
| Type C | If mandating additional gap patterns is feasible or necessary | No | No | Yes |
|  | If UE capability for NR only measurement is necessary | No | No | Yes |
| Note 1: The UE capability is to be down selected: Option 1: per-FR gap capable UE (This UE capability could potentially mean UE at least can do NR only measurement with short gap. For UE not capable of per-FR gap the mandated gap pattern is allowed not supported) Option 2: New UE capability (For Type C UE) to indicate although a gap is mandated to be supported but it is not supported actually. Note 2: A new UE capability is needed to indicate if side condition applies. Note 3: Approach A2 and Approach A3 needs modification of LTE RRC signaling, however Approach A1 doesn’t. |  |  |  |  |

Table 1. Summary for scenario B

|  |  | Approach B1 Note 1 | Approach B2Note 2 | Approach B3Note 3 |
| --- | --- | --- | --- | --- |
| Type A | If mandating additional gap patterns is feasible or necessary | Yes | Yes | Yes |
|  | If UE capability for NR only measurement is necessary | Yes | Yes | Yes |
| Type B | If mandating additional gap patterns is feasible or necessary | No | Yes | Yes |
|  | If UE capability for NR only measurement is necessary | No | Yes | Yes |
| Type C | If mandating additional gap patterns is feasible or necessary | No | No | Yes |
|  | If UE capability for NR only measurement is necessary | No | No | Yes |
| Note 1: A similar UE capability as shortMeasurementGap-r14 in LTE RRC needs to be introduced in NR RRC. Note 2: A new UE capability (For Type C UE) to indicate although a gap is mandated to be supported but it is not supported actually) Note 3: A new UE capability is needed to indicate if side condition applies. Note 4: All the approaches doesn’t need modification of LTE RRC signaling. |  |  |  |  |

Based on the observations and summary the following proposals are present.

Proposal 1. To mandate additional gap patterns in FR1, the UE capability for NR only measurements needs to be introduced.

  - NR-only measurement means the target measurement objects to be measured within the measurement gap are all NR carriers.

  - The UE capability is to indicate if the gap patterns from GP#2 to GP#11 can only be used to do NR only measurement.

Proposal 2. For EN-DC and LTE SA (Scenario A), mandating additional gap patterns and introducing UE capability for NR-only measurement is to be further decided by choice of following approaches.

  - Approach A1. No further normalization efforts in Rel-16.

  - Approach A2. Mandating additional gap patterns per UE capability

    - New UE capability  (For Type C UE) to indicate although a gap is mandated to be supported but it is not supported actually, or

    - per-FR gap capability

  - Approach A3. Mandating additional gap patterns and introducing UE capability for NR only measurements with side condition

    - New UE capability is needed to indicate if side condition applies

Our preference for scenario A is approach A3 if it is agreeable, but it seems only approach A1 is feasible because there was concern on changing LTE RRC signalling.

Proposal 3. For NE-DC (Scenario A), introducing UE capability for NR-only measurement is to be further decided by choice of following approaches.

  - Approach B1. Mandating additional gap patterns per UE capability of shortMeasurementGap-r14

    - Similar UE capability as shortMeasurementGap-r14 in LTE RRC needs to be introduced in NR RRC

  - Approach B2. Mandating additional gap patterns per new UE capability

    - New UE capability (For Type C UE) to indicate although a gap is mandated to be supported but it is not supported actually

  - Approach B3. Mandating additional gap patterns and introducing UE capability for NR only measurement with side condition

    - New UE capability is needed to indicate if side condition applies

Our preference for scenario B is approach B3 if it is agreeable. Approach B2 can be adopted if approach B3 is not agreeable.

Proposal 4. Additional mandatory gap patterns in FR1 are applicable to NR SA, NR DC and NE-DC. TBD for EN-DC and LTE SA scenarios.

Proposal 5. UE capability for NR only measurement are applicable to NR SA, NR DC and NE-DC. TBD for EN-DC and LTE SA scenarios.

2.2 Mandatory of gap patterns and applicability

The agreements on mandatory gap patterns are as follows as captured in WF [1].

- Mandatory gap patterns for FR1 - Option 1 - GP#2 and GP#3 - Option 2 - GP#2, GP#3, GP#7, GP#8, GP#9, GP#10 and GP#11 - Option 3 - Other options are not precluded - Mandatory gap patterns for FR2 - Option 1 - GP#17 and GP#18 - Option 2 - GP#16, GP#17, GP#18, GP#19 - Option 3 - Other options are not precluded - How to make decisions on additional mandatory gap patterns if consensus cannot be reached - Option 1 - Based on majority view - Option 2 - Any views that is agreeable

Firstly it is import to agree on how to decide additional mandatory gap patterns if consensus cannot be reached. Based on experience of past discussions it is very hard to reach consensus in terms of number and specific gap pattern to be mandated. To move on we suggest to make decisions based on majority views. Each gap pattern has its own use case and different companies have their own favorites.

Proposal 6. Make decisions on additional mandatory gap patterns based on majority view.

Then for the additional gap patterns to be mandated in Rel-16, our proposals are as follows based on analysis provided in our contributions in the past meetings.

Proposal 7. Additional mandatory gap patterns in FR1 is as option 2, i.e. GP#2, GP#3, GP#7, GP#8, GP#9, GP#10 and GP#11.

Proposal 8. Additional mandatory gap patterns in FR2 is as option 2, i.e. GP#16, GP#17, GP#18, GP#19.

For gap patterns in FR2 it is used for UE supporting per-FR gap and for FR2 only measurements when per FR gap is configured. So it is applicable to NR SA, NR DC, NE-DC and EN-DC scenarios.

Proposal 9. Additional mandatory gap patterns in FR2 are applicable to NR SA, NR DC, NE-DC and EN-DC scenarios.

2.3 Mandatory with capability signaling

This falls into UE feature list discussion. The entire feature of mandatory gap patterns is to be mandatory with capability signaling. In Rel-16 mandatory without capability signaling is not friendly to UE implementation unless it is necessary. Since this is to mandate additional gap patterns the feature cannot be optional. So this feature can only be mandatory with capability signaling.

Proposal 10. UE feature of mandatory of gap patterns are mandatory with capability signalling.

3. Conclusion

In this contribution, we further provided our views on mandatory of measurement gap patterns in R16. Based on the observations following proposals are present.

Proposal 1. To mandate additional gap patterns in FR1, the UE capability for NR only measurements needs to be introduced.

  - NR-only measurement means the target measurement objects to be measured within the measurement gap are all NR carriers.

  - The UE capability is to indicate if the gap patterns from GP#2 to GP#11 can only be used to do NR only measurement.

Proposal 2. For EN-DC and LTE SA (Scenario A), mandating additional gap patterns and introducing UE capability for NR-only measurement is to be further decided by choice of following approaches.

  - Approach A1. No further normalization efforts in Rel-16.

  - Approach A2. Mandating additional gap patterns per UE capability

    - New UE capability  (For Type C UE) to indicate although a gap is mandated to be supported but it is not supported actually, or

    - per-FR gap capability

  - Approach A3. Mandating additional gap patterns and introducing UE capability for NR only measurements with side condition

    - New UE capability is needed to indicate if side condition applies

Proposal 3. For NE-DC (Scenario A), introducing UE capability for NR-only measurement is to be further decided by choice of following approaches.

  - Approach B1. Mandating additional gap patterns per UE capability of shortMeasurementGap-r14

    - Similar UE capability as shortMeasurementGap-r14 in LTE RRC needs to be introduced in NR RRC

  - Approach B2. Mandating additional gap patterns per new UE capability

    - New UE capability (For Type C UE) to indicate although a gap is mandated to be supported but it is not supported actually

  - Approach B3. Mandating additional gap patterns and introducing UE capability for NR only measurement with side condition

    - New UE capability is needed to indicate if side condition applies

Proposal 4. Additional mandatory gap patterns in FR1 are applicable to NR SA, NR DC and NE-DC. TBD for EN-DC and LTE SA scenarios.

Proposal 5. UE capability for NR only measurement are applicable to NR SA, NR DC and NE-DC. TBD for EN-DC and LTE SA scenarios.

Proposal 6. Make decisions on additional mandatory gap patterns based on majority view.

Proposal 7. Additional mandatory gap patterns in FR1 is as option 2, i.e. GP#2, GP#3, GP#7, GP#8, GP#9, GP#10 and GP#11.

Proposal 8. Additional mandatory gap patterns in FR2 is as option 2, i.e. GP#16, GP#17, GP#18, GP#19.

Proposal 9. Additional mandatory gap patterns in FR2 are applicable to NR SA, NR DC, NE-DC and EN-DC scenarios.

Proposal 10. UE feature of mandatory of gap patterns are mandatory with capability signalling.

Observation 2: UE implementation of handling short measurement gap depending on UE capability.

Observation 3. Mandating additional gap patterns and introducing UE capability for NR only measurement in LTE SA and EN-DC need change in LTE RRC signaling.

Observation 4. Mandating additional gap patterns and introducing UE capability for NR only measurement in NE-DC does not need change in LTE RRC signaling.

Observation 5. Mandating additional gap patterns and necessity of UE capability of NR only measurement would be different for different type UEs.

4. References

- R4-2002248	WF on R16 NR RRM enhancements – mandatory gap patterns, ZTE

- R4-2001400	Considerations on mandatory gap patterns for NR only measurements in release 16, Ericsson