3GPP TSG-RAN WG4 Meeting #94bis-e                                R4-2004139

Electronic Meeting, 20th April – 30th April, 2020

Source: 	Huawei, HiSilicon

Title: 	Updated TP for TR 37.716-00-00 to clean up

Agenda Item:	8.8.2

Document for:	Approval

Introduction

This contribution provides an updated TP to clean up TR 37.716-00-00.

- Text Proposal

<TP for TR 37.716-00-00>

- 1	Scope

The present document is a technical report for SA NR Supplementary uplink (SUL), NSA NR SUL and NSA NR SUL with UL sharing from the UE perspective (ULSUP) under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address NR SUL for the Rel-16 bands/band combinations in Table 1-1 to Table 1-4.

Table 1-1: Release 16 SUL bands

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n89 | 824 – 849 MHz | N/A | SUL |
| n95 | 2010 – 2015 MHz | N/A | SUL |

Table 1-2: Release 16 SA SUL band combinations

| SA SUL band combination | REL independent from |
| --- | --- |
| SUL_n79-n84 | Rel-15 |
| SUL_n41-n80 | Rel-15 |
| SUL_n41-n81 | Rel-15 |
| SUL_n77_n80 | Rel-15 |
| SUL_n77_n84 | Rel-15 |

Table 1-3: Release 16 NSA SUL band combinations

| NSA SUL band combination | REL independent from |
| --- | --- |
| DC_1_SUL_n78-n80 | Rel-15 |
| DC_7_SUL_n78-n80 | Rel-15 |
| DC_20_SUL_n78-n80 | Rel-15 |
| DC_8_SUL_n78-n80 | Rel-15 |
| DC_1_SUL_n77-n80 | Rel-15 |
| DC_3_SUL_n77-n84 | Rel-15 |
| DC_3_SUL_n78-n84 | Rel-15 |

Table 1-4: Release 16 NSA SUL with ULSUP band combinations

| NSA SUL with ULSUP band combination | REL independent from |
| --- | --- |
| DC_1_SUL_n79-n84 | Rel-15 |
| DC_3_SUL_n41-n80 | Rel-15 |
| DC_8_SUL_n41-n81 | Rel-15 |
| DC_3_SUL_n78-n80 | Rel-15 |
| DC_1-3_SUL_n78-n80 | Rel-15 |
| DC_3-7_SUL_n78-n80 | Rel-15 |
| DC_3-8_SUL_n78-n80 | Rel-15 |
| DC_3_20_SUL_n78-n80 | Rel-15 |
| DC_3_SUL_n77-n80 | Rel-15 |
| DC_1_SUL_n77-n84 | Rel-15 |

This TR contains a general part and specific band combination part. The actual requirements are added to the corresponding technical specifications.

<Next of TP >

- 3	Definitions, symbols and abbreviations

- 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

- 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

- 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

BCS	Bandwidth Combination Set

CA	Carrier Aggregation

CC	Component Carriers

DC	Dual Connectivity

DL	Downlink

E-UTRA 	Evolved UMTS Terrestrial Radio Access

FDD	Frequency Division Duplex

MPR 	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

REFSENS	Reference Sensitivity power level

SCS	Subcarrier spacing

SUL	Supplementary uplink

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

ULSUP	UL sharing from the UE perspective

<Next of TP >

- 7.2	DC_3_SUL_n41-n80

7.2.1	Operating bands

Table 7.2.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3_SUL_n41-n80 | 3 | SUL_n41-n80 | No |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

7.2.2	Configuration

Table 7.2.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A_SUL_n41A-n80A | DC_3A_n41A DC_3A_n80A_ULSUP-TDM_n41A | 3A | SUL_n41A-n80A |
| DC_3C_SUL_n41A-n80A | DC_3C_n41A DC_3C_n80A_ULSUP-TDM_n41A | 3C | SUL_n41A-n80A |

7.2.3 Maximum output power

Table 7.2.3-1: Maximum output power for inter-band EN-DC

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_3A_n41A, DC_3A_n80A_ULSUP-TDM_n41A, DC_3C_n41A, DC_3C_n80A_ULSUP-TDM_n41A | 23 | +2/-3 |

7.2.4	Spurious emission band UE co-existence

Table 7.2.4-1: Spurious emissions for inter-band EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3A_n41A, DC_3A_n80A_ULSUP-TDM_n41A, DC_3C_n41A, DC_3C_n80A_ULSUP-TDM_n41A | E-UTRA Band 1, 5, 8, 26, 27, 28, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 30 |

<Next of TP >

- 7.3	DC_8_SUL_n41-n81

7.3.1	Operating bands

Table 7.3.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8_SUL_n41-n81 | 8 | SUL_n41-n81 | No |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

7.3.2	Configuration

Table 7.3.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A_SUL_n41A-n81A | DC_8A_41A, DC_8A_n81A_ULSUP-TDM_n41A | 8A | SUL_n41A-n81A |

7.3.3  Maximum output power

Table 7.3.3-1: Maximum output power for inter-band EN-DC

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_8A_41A, DC_8A_n81A_ULSUP-TDM_n41A | 23 | +2/-3 |

7.3.4	Spurious emission band UE co-existence

Table 7.3.4-1: Spurious emissions for inter-band EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_8A_41A, DC_8A_n81A_ULSUP-TDM_n41A | E-UTRA Band 1, 28, 34, 39, 40, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |

<Next of TP >

- 7.4	DC_3_SUL_n78-n80

7.4.1	Operating bands

Operating bands for DC_3_SUL_n78-n80 is already specified in current specification.

7.4.2	Configuration

Table 7.4.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3C_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A | 3C | SUL_n78A-n80A |

<Next of TP >

- 7.5	DC_1-3_SUL_n78-n80

7.5.1	Operating bands

Table 7.5.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_SUL_n78-n802 | CA_1-3 | SUL_n78-n801 | DC_3_n78 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

7.5.2	Configuration

Table 7.5.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A | CA_1A-3A | SUL_n78A-n80A |

<Next of TP >

- 7.6	DC_3-7_SUL_n78-n80

7.6.1	Operating bands

Table 7.6.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_SUL_n78-n802 | CA_3-7 | SUL_n78-n801 | DC_3_n78 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

7.6.2	Configuration

Table 7.6.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-7A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_7A_n78A DC_7A_n80A | CA_3A-7A | SUL_n78A-n80A |
| DC_3C-7A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_7A_n78A DC_7A_n80A | CA_3C-7A | SUL_n78A-n80A |

<Next of TP >

- 7.7	DC_3-8_SUL_n78-n80

7.7.1	Operating bands

Table 7.7.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-8_SUL_n78-n802 | CA_3-8 | SUL_n78-n801 | DC_3_n78 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

7.7.2	Configuration

Table 7.7.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-8A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_8A_n78A DC_8A_n80A | CA_3A-8A | SUL_n78A-n80A |

<Next of TP >

- 7.8	DC_3_20_SUL_n78-n80

7.8.1	Operating bands

Table 7.8.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3_20_SUL_n78-n802 | CA_3_20 | SUL_n78-n801 | DC_3_n78 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

7.8.2	Configuration

Table 7.8.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A_20A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_20A_n78A DC_20A_n80A | CA_3A_20A | SUL_n78A-n80A |
| DC_3C_20A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_20A_n78A DC_20A_n80A | CA_3C_20A | SUL_n78A-n80A |

<Next of TP >

- 7.9	DC_3_SUL_n77-n80

7.9.1	Operating bands

Table 7.9.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3_SUL_n77-n802 | 3 | SUL_n77-n80 | DC_3_n77 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

7.9.2	Configuration

Table 7.9.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A_SUL_n77A-n80A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A | 3A | SUL_n77A-n80A |

7.9.3  Maximum output power

Table 7.9.3-1: Maximum output power for inter-band EN-DC

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A | 23 | +2/-3 |

7.9.4	Spurious emission band UE co-existence

Table 7.9.4-1: Spurious emission band UE co-existence for inter-band EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3_n77 DC_3_n80_ULSUP-TDM_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |

<Next of TP >

- 7.10	DC_1_SUL_n77-n84

7.10.1	Operating bands

Table 7.10.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_SUL_n77-n842 | 1 | SUL_n77-n84 | DC_1_n77 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

7.10.2	Configuration

Table 7.10.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_SUL_n77A-n84A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A | 1A | SUL_n77A-n84A |

7.10.3 Maximum output power

Table 7.10.3-1: Maximum output power for inter-band EN-DC

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A | 23 | +2/-3 |

7.10.4	Spurious emission band UE co-existence

Table 7.10.4-1: Spurious emission band UE co-existence for inter-band EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n77 DC_1_n84_ULSUP-TDM_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |

<End of TP >