# 3GPP TSG-RAN WG4 Meeting #94bis-e                                                       R4-2004172

# Electronic Meeting, April 20-24, 2020

Agenda Item:	6.5.1.1

Source: 	Ericsson

Title:	TP to TS 38.174: system parameter

Document for:	Approval

- Introduction

Section 2 proivde some reasoning on the text proposal and Section 3 provides text proposal on chapter 5 system parameter for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Disucssion

The text proposal is based on the WF [1]. We noticed that for the generic description text, it is not within the requirement itself and need some discussion. For some of the generic description, it is not likely to make a reference as not only BS/UE need to be replaced by IAB-DU/IAB-MT,  but there is a need to modify further on the text to selectively to add IAB-DU/IAB-MT, the example for such case is the chapter 5.3. For some other cases, the text itself is very short and almost the same for UE spec and BS spec, rather than do reference to a versioned spec, it will be more convenient to just provide the whole text to give better readability.  Below lists some of the thinking for each of the subchapter text proposal:

5.1 General: in this chapter, the text of UE spec and BS spec is basicall the same, considering the good readability, it is suggested to copy the text directly into the IAB TS.

5.3.1 channel bandwidth:in general section text, the IAB-MT or IAB-DU replacement need to be carefully examined and further modification is needed thus the referening is not possible.

5.3.2 and 5.3.3: requirement can be referenced.

5.3.4: IAB-DU need some modification on the text while IAB-MT can be referenced.

5.3.5: specific text for the supported band need to be additional stated when refereeing.

5.4.: specific text for the supported band need to be additional stated when refereeing.

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

## 2	Reference

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”, V16.2.0

[3]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, v 16.2.0

[4]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”, V16.2.0

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios "

[6]	3GPP TS 38.133: “NR: Requirements for support of radio resource management”

[7]	3GPP TS 38.211: "NR; Physical channels and modulation".

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and IAB-DU or IAB-MT channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and IAB-DU or IAB-MT channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to the present version of the specification are identified as described in table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

## 5.3	Channel bandwidth

### 5.3.1	General

The IAB DU channel bandwidth supports a single NR RF carrier in the uplink or downlink at the IAB node. Different UE or IAB MT channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs or IAB MT connected to the IAB DU. The placement of the UE or IAB MT channel bandwidth is flexible but can only be completely within the IAB DU channel bandwidth. The IAB DU shall be able to transmit to and/or receive from one or more UE or IAB MT Bandwidth parts that are smaller than or equal to the number of carrier resource blocks on the RF carrier, in any part of the carrier resource blocks.

The IAB MT channel bandwidth supports a single NR RF carrier in the uplink or downlink at the IAB MT. From a BS or IAB DU perspective, different IAB MT channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs or IAB MT connected to the IAB DU. Transmission of multiple carriers to the same IAB MT (CA) or multiple carriers to different UEs or IAB MT within the  IAB DU channel bandwidth can be supported.

From a IAB MT perspective, the IAB MT is configured with one or more BWP / carriers, each with its own IAB MTchannel bandwidth. The IAB MT does not need to be aware of the BS or IAB DU channel bandwidth or how the BS or IAB DU allocates bandwidth to different UEs or IAB MT.

The placement of the IAB MT channel bandwidth for each IAB MT carrier is flexible but can only be completely within the BS or IAB-DU channel bandwidth.

The relationship between the IAB DU or IAB MT channel bandwidth, the guardband and the transmission bandwidth configuration is shown in Figure 5.3.1-1.

.

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

### 5.3.2	Transmission bandwidth configuration

For IAB-DU, the transmission bandwidth configuration is specified in TS 38.104 [2], subclause 5.3.2.

### For IAB-MT, the transmission bandwidth configuration is specified in TS 38.101-1[3] for FR1 in  subclause 5.3.2 and in TS 38.101-2 [4] for FR2 in subclause 5.3.2.

### 5.3.3	Minimum guardband and transmission bandwidth configuration

For IAB-DU, the minimum guardband and transmission bandwidth configuration is specified as the same as BS in TS38.104[2], subclause 5.3.3.

For IAB-MT, the minimum guardband and transmission bandwidth configuration is specified as the same as UE in TS38.101-1[3] for FR1 and in TS 38.101-2 [4] for FR2 in subclause 5.3.3.

### 5.3.4	RB alignment

For each IAB-DU channel bandwidth and each numerology, IAB-DU transmission bandwidth configuration must fulfil the minimum guardband requirement specified in clause 5.3.3.

For IAB-DU, for each numerology, its common resource blocks are specified in clause 4.4.4.3 in [7], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology.

For IAB-DU, for each numerology, all UE and IAB-MT transmission bandwidth configurations indicated to UEs or IAB-MT served by the IAB-DU by higher layer parameter carrierBandwidth defined in TS 38.331 [8] shall fall within the IAB-DU transmission bandwidth configuration.

For IAB-MT, the RB alignment is specified as the same as UE in TS38.101-1[3] for FR1 in subclause 5.3.4 and in TS 38.101-2 [4] for FR2 in subclause 5.3.3.

### 5.3.5	IAB-DU and IAB-MT channel bandwidth per operating band

For IAB-DU, the channel bandwidth for all band for FR2 and band n41, n77, n78 and n79 for FR1 is specified as the same as BS in TS38.104[2], subclause 5.3.5.

For IAB-MT, the channel spacing for band n41, n77, n78 and n79 for FR1 is specified as the same as UE in TS38.101-1[3] in subclause 5.3.5 and in TS38.101-2[4] for FR2 in subclause 5.3.5.

## 5.3A	IAB-DU channel bandwidth for CA

The channel bandwidth for CA is specified as the same as BS in TS38.104[2], subclause 5.3A.

## 5.4	Channel arrangement

### 5.4.1	Channel spacing

For IAB-DU, the channel spacing is specified as the same as BS in TS38.104[2], subclause 5.4.1.

For IAB-MT, the channel spacing is specified as the same as UE in TS38.101-1[3] for FR1 in subclause 5.4.1 and in TS38.101-2[4] for FR2 in subclause 5.4.1.

### 5.4.2	Channel raster

#### 5.4.2.1	NR-ARFCN and channel raster

For IAB-DU, the NR-ARFCN and channel raster is specified as the same as BS in TS38.104[2], subclause 5.4.2.1.

For IAB-MT, the NR-ARFCN and channel raster is specified as the same as UE in TS38.101-1[3] for FR1 in subclause 5.4.2.1 and in TS38.101-2[4] for FR2 in subclause 5.4.2.1.

#### 5.4.2.2	Channel raster to resource element mapping

For IAB-DU, the Channel raster to resource element mapping is specified as the same as BS in TS38.104[2], subclause 5.4.2.2.

For IAB-MT, the Channel raster to resource element mapping is specified as the same as UE in TS38.101-1[3] for FR1 in subclause 5.4.2.2 and in TS38.101-2[4] for FR2 in subclause 5.4.2.2.

#### 5.4.2.3	Channel raster entries for each operating band

For IAB-DU, the channel raster entries for all band for FR2 and n41, n77, n78 and n79 for FR1 are specified as the same as BS in TS38.104[2], subclause 5.4.2.3.

For IAB-MT, the channel raster entries for n41, n77, n78 and n79 for FR1 are specified as the same as UE in TS38.101-1[3] in subclause 5.4.2.3 and in TS38.101-2[4] for FR2 in subclause 5.4.2.3.

### 5.4.3	Synchronization raster

#### 5.4.3.1	Synchronization raster and numbering

For IAB-DU, the synchronization raster and numbering are specified as the same as BS in TS38.104[2], subclause 5.4.3.1.

For IAB-MT, the synchronization raster and numbering are specified as the same as UE in TS38.101-1[3] for FR1 in subclause 5.4.3.1 and in TS38.101-2[4] for FR2 in subclause 5.4.3.1.

#### 5.4.3.2	Synchronization raster to synchronization block resource element mapping

For IAB-DU, the synchronization raster to synchronization block resource element mapping is specified as the same as BS in TS38.104[2], subclause 5.4.3.2.

#### For IAB-MT, the synchronization raster to synchronization block resource element mapping is specified as the same as UE in TS38.101-1[3] for FR1 in subclause 5.4.3.2 and in TS38.101-2[4] for FR2 in subclause 5.4.3.2.

#### 5.4.3.3	Synchronization raster entries for each operating band

For IAB-DU, the synchronization raster entries for all band for FR2 and n41, n77, n78 and n79 for FR1 are specified as the same as BS in TS38.104[2], subclause 5.4.3.3.

For IAB-MT, the synchronization raster entries entries for n41, n77, n78 and n79 for FR1 are specified as the same as UE in TS38.101-1[3] in subclause 5.4.3.3 and in TS38.101-2[4] for FR2 in subclause 5.4.3.3.

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-2002484, WF on TS referencing rules, Nokia