3GPP TSG RAN WG4 Meeting #94-e-Bis	R4-2004194

Electronic Meeting, April 20-30, 2020

Agenda Item:	5.11.3.4

Source: 	Ericsson

Title:	Analysis of NTA offset under NR and NB-IoT coexistence

Document for:	Discussion

- Introduction

The in-band mode of NB-IoT can be operated within the bandwidth of an NR cell. This is familiarly called as NB-IoT-NR coexistence. This has no impact on RRM requirements for NB-IoT. However, it has been identified to clarify the applicable NTA offset to be applied by an NR UE when the NB-IoT and NR coexists on the same NR carrier e.g. within the BW of the NR carrier such as in NB-IoT in-band mode [1-2].

In this paper we analyze the applicability of NTA offset under NB-IoT-NR coexistence scenario and expressed our views based on the analysis.

# Analysis of UE timing in NB-IoT-NR Coexistence

The NTA offset specifies a fixed time offset before which the uplink frame i start before the corresponding downlink frame i.

In NR FR1 the UE can be configured with one of the three possible NTA offset values (0, 25600 Tc and 39936 Tc), which the UE applies to its uplink timing for uplink transmission. The default value in FR1 (if NTA offset is not signalled) is 25600 Tc ( 13 µs).

For NB-IoT operation in FDD band (standalone or within LTE BW) the NB-IoT UE applies NTA offset of 0 to its uplink timing for uplink transmission. For NB-IoT operation in TDD band (standalone or within LTE BW) the NB-IoT UE applies NTA offset of 20 µs (624 Ts  39936 Tc) to its uplink timing for uplink transmission. In general, the NB-IoT UE applies the same NTA offset as applied by the LTE UE. Therefore, when NB-IoT operates within the NR bandwidth, the NB-IoT UE shall apply the same NTA offset as legacy NB-IoT UE (0 in FDD band or 20 µs (624 Ts). This implies that in NB-IoT and NR coexistence scenario, the NR UE applies the NTA offset to its uplink timing for uplink transmission corresponding to that of the NB-IoT UE.

NB-IoT is special case of LTE. Therefore, in our view the term LTE-NR coexistence used Table 7.1.2-2 of TS 38.133 is broad enough to also encompass NB-IoT-NR coexistence scenario. Neverthless, for the sake of clarify we suggest to explicitly specify the applicable NTA offset for the NR UE when NB-IoT co-exists with NR in FDD and TDD bands in FR1. This is in principle similar to the approach in [1] but expressed in separate rows to avoid any confusion.

Table 7.1.2-2: The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD band without LTE-NR coexistence case or FR1 TDD band without LTE-NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with LTE-NR coexistence case | 0 (Note 1) |
| FR1 TDD band with LTE-NR coexistence case | 39936 (Note 1) |
| FR1 FDD band without NB-IoT-NR coexistence case or FR1 TDD band without NB-IoT-NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with NB-IoT-NR coexistence case | 0 (Note 1) |
| FR1 TDD band with NB-IoT-NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of can also be provided for a FDD serving cell. Note 2: Void |  |

- Summary

In this paper we analyze the applicability of NTA offset under NB-IoT-NR coexistence scenario and expressed our views based on the analysis.

Following are the main observation and proposals:

- Observation #1: For NB-IoT operation in FDD band (standalone or within LTE BW) the NB-IoT UE applies NTA offset of 0 to its uplink timing for uplink transmission. For NB-IoT operation in TDD band (standalone or within LTE BW) the NB-IoT UE applies NTA offset of 20 µs (624 Ts  39936 Tc) to its uplink timing for uplink transmission.

- Proposal #1: For NB-IoT-NR coexistence in FR1 FDD band (in-band of NR BW) the NR UE shall NTA offset of 0 to its uplink timing for uplink transmission.

- Proposal #2: For NB-IoT-NR coexistence in FR1 TDD band (in-band of NR BW) the NR UE shall NTA offset of 39936 Tc to its uplink timing for uplink transmission.

A CR to TS 38.133 to specify changes corresponding to the above proposals is provided in [3].

- References

- R4-1915169, TA offset setting for NR coexistence with NB-IoT, Nokia.

- R4-2001553, Discussion on the TA offset setting for NR and NB-IoT coexistence, Huawei.

- R4-20xxx, NTA offset under NR and NB-IoT coexistence in 38.133, Ericsson.