3GPP TSG-RAN WG4 Meeting # 94-e-Bis 	R4-2004248

# Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:	6.5.3.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on RLM requirement for IAB-MT

Document for:	Discussion

- Introduction

In last meeting RAN4 94e meeting, the RRM measurement for IAB-MT were discussed. There are some remaining issues about RLM. We further investigate these issues and present views in this paper.

- Discussion

In the last RAN4 94e meeting, it was agreed that RLM requirements for IAB-MTs are defined for no-DRX mode only. And the corresponding requirements apply to local area IAB class [1]. Some issues aroused without agreements from last meeting. We present our views as follows point by point.

Relaxed RLM requirements:

As mentioned in previous discussion, the IAB is deployed as fixed node without mobility, which leads to less possibility of radio link failure. Power consumptions and efforts can be saved by relaxing RLM evaluation period for both out-of-sync and in-sync. It is suggested to extend the evaluation period in TS 38.133 by scaling factor of 5 for IAB MT RLM.

Proposal 1: It is suggested to extend the evaluation period in TS 38.133 by scaling factor of 5 for IAB MT RLM.

Sharing factor:

The sharing factor is related to the configuration of SMTC and measurement gap. Based on the discussion in the last meeting, we prefer to keep the sharing factor P at current state, and the value could be updated.

Proposal 2: There is no need to remove the sharing factor.

Density of CSI-RS:

For the density of CSI-RS for RLM measurement, some companies propose to support the CSI-RS-based RLM when the density is 1. However, based on the discussion and analysis for CSI-RS-based RLM in TS 38.133, it was shown that with large delay spread case, the sparse CSI-RS will lead to performance degradation even with the increased number of CSI-RS. Therefore, the requirement shall apply only for CSI-RS density is 3 over the bandwidth ≥ 24 PRBs.

Proposal 3 : The CSI-RS requirements shall apply only for CSI-RS density is 3 over the bandwidth ≥ 24 PRBs.

- Conclusion

Proposal 1: It is suggested to extend the evaluation period in TS 38.133 by scaling factor of 5 for IAB MT RLM.

Proposal 2: There is no need to remove the sharing factor.

Proposal 3 : The CSI-RS requirements shall apply only for CSI-RS density is 3 over the bandwidth ≥ 24 PRBs.

References

[1] RP-193199 “Proposed Way Forward on RRM Requirements for the IAB Project”