3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004312

# Online, 20th – 30th April, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on conditional handover requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | R16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conditional handover has been introduced in NR mobility enhancement WID, and the corresponding handover delay and interruption requirements shall be defined in TS38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To define conditional handover requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conditional handover requirements will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.1.4	NR Conditional Handover

6.1.4.1	Introduction

The requirements in this clause are applicable to conditional handover to change the NR PCell to another NR Pcell.

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers from NR FR1 cell to NR FR1 cell.

6.1.4.2	NR Conditional Handover Delay

Procedure delays for the procedure that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message containing conditional handover execution conditions for the target cell, the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command.

DConditional_HO = TRRC_processing + Tmeasure + TUE_preparation + Tprocessing + T∆ + Tmargin + TIU ms ms

Where:

TRRC_processing is the RRC processing delay as defined in clause 12 in TS 38.331 [2].

Tmeasure is the measurements time stated in clause 6.1.4.2.1.

TUE_preparation is the time from the end of UE successfully decoding RRC message containing conditional handover execution conditions until the UE starts to execute a conditional handover. TUE_preparation =10ms.

Tprocessing is time for UE processing. Tprocessing can be up to 20ms when source cell and target cell are in the same FR. Tprocessing can be up to 40ms when source cell and target cell are in different FRs.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the conditional handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the conditional handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

The interruption time is the time period from the UE starts to execute a conditional handover until the UE starts the transmission of new PRACH.

6.1.4.2.1	Measurement time

The measurement time is the time period from UE successfully decodes the RRC message containing CHO execution condition for target cell until UE starts to execute conditional handover to the target cell.

Tmeasure= TEvent_DU + Tmeasure_delay ms

Where

TEvent_DU is the delay uncertainty in waiting an event that will trigger the conditional handover to the target cell after UE successfully decoding the RRC message including CHO execution condition.

Tmeasure_delay is the measurement delay between the event that will trigger the conditional handover to the target cell and the point when the UE starts to execute conditional handover to the target cell.

For intra-frequency target cell, Tmeasure_delay shall be less than Tidentify_intra_without_index or Tidentify_intra_with_index as defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. The target cell is detectable only if at least one SSBs measured from the target cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index as defined in clause 9.2.5.1 or clause 9.2.6.2. If the target cell which has been detectable at least for the time period Tidentify intra without index or Tidentify intra with index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period and then the cell becomes detectable again with the same spatial reception parameter and triggers an CHO event, Tmeasure_delay shall be less than TSSB_measurement_period_intra provided the timing to that the target cell has not changed more than  3200 Tc while the measurement gap has not been available and L3 filtering has not been used. When L3 filtering is used, an additional delay can be expected.

For inter-frequency target cell, Tmeasure_delay shall be less than Tidentify_inter_without_index or Tidentify_inter_with_index defined in clause 9.3.4. When L3 filtering is used an additional delay can be expected. The target cell is detectable only if at least one SSB measured from the target cell being configured remains detectable during the time period Tidentify_inter_without_index or Tidentify_inter_with_index defined in clause 9.3.4. If a cell which has been detectable at least for the time period Tidentify_inter_without_index or Tidentify_inter_with_index defined in clause 9.3.4 becomes undetectable for a period and then the cell becomes detectable again with the same spatial reception parameter and then triggers an CHO event, Tmeasure_delay shall be less than TSSB_measurement_period_inter defined in clause 9.3.5 provided the timing to that cell has not changed more than  3200 Tc while measurement gap has not been available and the L3 filtering has not been used. When L3 filtering is used an additional delay can be expected.

<End of Change 1>