3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2004327

Online, 20th – 30th April, 2020

Title: 	Discussion on MRTD requirements for FR2 inter-band DL CA

Source: 	Huawei, HiSilicon

Agenda item:	6.14.2.1

Document for:	Discussion

Introduction

In RAN4#94-e meeting, whether existing MRTD requirement for FR2 inter-band can be reduced has been discussed and no conclusion has been achieved. In this contribution, we will provide further discussion on MRTD requirement for FR2 inter-band CA.

Discussion

The existing maximum received time difference (MRTD) for FR2 inter-band CA is defined as 8us, which allows 5 us propagation delay difference and 3us BS TAE between two serving cells. Since it has been agreed on no change in BS TAE requirement, The MRTD for FR2 inter-band CA shall always be larger than the BS TAE. Hence, MRTD shall be larger than 3us.

For beam managements in FR2 inter-band CA scenario, the following agreements [1] have been achieved in RAN4 RF session.

- UE is assumed to be feasible to have independent beam management for the bands that are part of supported band configuration in inter-band CA for 28 GHz + 39 GHz combinations. - “28GHz” stands for a band group includes n257, n258, n261 - “39GHz” stands for a band group includes n259, n260 - Beam management for the bands that are part of supported band configuration in inter-band CA for 28 GHz + 28 GHz or 39 GHz + 39 GHz combinations. - Alt 1: UE is assumed to have common beam management - Alt 2: UE is assumed to have independent beam management

Two types of FR2 inter-band CA combinations, including FR2 inter-band CA with independent beam management and with common beam management, are discussed in RF session. The feasibility of FR2 inter-band CA with independent beam management has been confirmed for 28GHz+39GHz combinations. For 28GHz+28GHz or 39GHz+39GHz combinations, the feasibility of FR2 inter-band CA with common beam management is still under discussion in RF session. No conclusion has been achieved.

For FR2 inter-band CA combinations with independent beam management, UE is capable for forming beams towards different directions. UE is assumed to have separate RF chains for different bands. Hence, the UE is able to handle 8us MRTD for FR2 inter-band CA. We suggest to keep the existing MRTD requirements, which allows more flexibility for network deployment.

Proposal 1: For FR2 inter-band CA combinations with independent beam management, it is suggested to keep the existing FR2 inter-band CA MRTD requirements.

For FR2 inter-band CA combinations with common beam management, UE is capable for forming beams towards the same beam directions. RAN4 needs to study whether it is feasible for UE to receive the signals from non-co-located serving cells by using same Rx beam. If it was feasible, then the existing MRTD requirements could be reused. If it was not feasible, then the MRTD requirements for FR2 inter-band CA could be reduced to 3us and only applied to co-located deployment.

Proposal 2: It is suggested to study whether FR2 inter-band CA combinations with common beam management can be applied to non-co-located deployment.

Conclusions

This contribution provides our analysis on MRTD requirements for FR2 inter-band CA in Rel-16. The following are provided:

Proposal 1: For FR2 inter-band CA combinations with independent beam management, it is suggested to keep the existing FR2 inter-band CA MRTD requirements.

Proposal 2: It is suggested to study whether FR2 inter-band CA combinations with common beam management can be applied to non-co-located deployment.

Reference

- R4-1916024, “WF on FR2 inter-band DL CA”, Nokia