3GPP TSG-RAN WG4 Meeting#94-e-bis                	R4-2004391

Electronic Meeting, 20 – 30 April 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussions on L1-SINR measurements for Rel-16 NR eMIMO

Agenda item:	6.11.2.1

Document for:	Discussion

Introduction

During the RAN4#94-e meeting, a way forward on NR eMIMO RRM requirements [1] was approved. The open issues addressed in this paper are concerning L-SINR measurements for specifying NR eMIMO RRM requirements.

Simulation results of L1-SINR measurements are presented and then followed by our discussions and conclusions.

Discussion

L1-SINR measurements

Number of samples for L1-SINR measurement

From [1], the following is agreed:

- M = [3] if single shot measurement is not applicable for CMR-only, SSB+NZP-IMR, SSB+ZP-IMR, CSI-RS+NZP-IMR and CSI-RS+ZP-IMR.

Tables 1-3 provide the CDF of L1-SINR measurement accuracy at 5th, 50th and 95th percentile, which is obtained using the simulation assumptions agreed in [2].

Table 1: Absolute L1-RSRP measurement accuracy for SCS = 15 kHz

| CMR | IMR | Number of Samples (M) | 5th percentile | 50th percentile | 95th percentile | Max(abs(5th, 9th) |
| --- | --- | --- | --- | --- | --- | --- |
| CSI-RS | N/A | 1 | -0.75 | 0.66 | 2.13 | 2.13 |
|  |  | 3 | -0.48 | 0.41 | 1.19 | 1.19 |
|  |  | 5 | -0.4 | 0.3 | 0.95 | 0.95 |
| SSB | NZP-IMR | 1 | -1.01 | 0.44 | 1.62 | 1.62 |
|  |  | 3 | -0.65 | 0.23 | 1.06 | 1.06 |
|  |  | 5 | -0.6 | 0.16 | 0.97 | 0.97 |
| SSB | ZP-IMR | 1 | -0.78 | 0.44 | 1.7 | 1.7 |
|  |  | 3 | -0.62 | 0.22 | 1.03 | 1.03 |
|  |  | 5 | -0.51 | 0.15 | 0.88 | 0.88 |
| CSI-RS | NZP-IMR | 1 | -0.88 | 0.55 | 1.8 | 1.8 |
|  |  | 3 | -0.56 | 0.33 | 1.15 | 1.15 |
|  |  | 5 | -0.5 | 0.24 | 1.02 | 1.02 |
| CSI-RS | ZP-IMR | 1 | -0.63 | 0.57 | 1.67 | 1.67 |
|  |  | 3 | -0.48 | 0.37 | 1.04 | 1.04 |
|  |  | 5 | -0.41 | 0.24 | 0.83 | 0.83 |

Table 2: Absolute L1-RSRP measurement accuracy for SCS = 30 kHz

| CMR | IMR | Number of Samples (M) | 5th percentile | 50th percentile | 95th percentile | Max(abs(5th, 9th) |
| --- | --- | --- | --- | --- | --- | --- |
| CSI-RS | N/A | 1 | -1 | 0.69 | 2.15 | 2.15 |
|  |  | 3 | -0.5 | 0.36 | 1.22 | 1.22 |
|  |  | 5 | -0.33 | 0.27 | 0.9 | 0.9 |
| SSB | NZP-IMR | 1 | -1 | 0.43 | 1.62 | 1.62 |
|  |  | 3 | -0.61 | 0.27 | 1 | 1 |
|  |  | 5 | -0.53 | 0.14 | 0.83 | 0.83 |
| SSB | ZP-IMR | 1 | -0.87 | 0.41 | 1.73 | 1.73 |
|  |  | 3 | -0.52 | 0.26 | 0.98 | 0.98 |
|  |  | 5 | -0.52 | 0.15 | 0.83 | 0.83 |
| CSI-RS | NZP-IMR | 1 | -0.96 | 0.56 | 1.87 | 1.87 |
|  |  | 3 | -0.6 | 0.31 | 1.14 | 1.14 |
|  |  | 5 | -0.49 | 0.25 | 0.94 | 0.94 |
| CSI-RS | ZP-IMR | 1 | -0.78 | 0.56 | 1.72 | 1.72 |
|  |  | 3 | -0.45 | 0.31 | 1.15 | 1.15 |
|  |  | 5 | -0.37 | 0.25 | 0.95 | 0.95 |

Table 3: Absolute L1-RSRP measurement accuracy for SCS = 120 kHz

| CMR | IMR | Number of Samples (M) | 5th percentile | 50th percentile | 95th percentile | Max(abs(5th, 9th) |
| --- | --- | --- | --- | --- | --- | --- |
| CSI-RS | N/A | 1 | -1.02 | 0.58 | 1.94 | 1.94 |
|  |  | 3 | -0.65 | 0.25 | 1.1 | 1.1 |
|  |  | 5 | -0.45 | 0.17 | 0.9 | 0.9 |
| SSB | NZP-IMR | 1 | -0.85 | 0.53 | 1.69 | 1.69 |
|  |  | 3 | -0.48 | 0.35 | 1.12 | 1.12 |
|  |  | 5 | -0.57 | 0.24 | 1 | 1 |
| SSB | ZP-IMR | 1 | -0.7 | 0.57 | 1.68 | 1.68 |
|  |  | 3 | -0.49 | 0.36 | 1.17 | 1.17 |
|  |  | 5 | -0.51 | 0.3 | 0.95 | 0.95 |
| CSI-RS | NZP-IMR | 1 | -1.06 | 0.35 | 1.71 | 1.71 |
|  |  | 3 | -0.82 | 0.2 | 1.12 | 1.12 |
|  |  | 5 | -0.63 | 0.15 | 0.92 | 0.92 |
| CSI-RS | ZP-IMR | 1 | -0.91 | 0.4 | 1.69 | 1.69 |
|  |  | 3 | -0.69 | 0.24 | 1.12 | 1.12 |
|  |  | 5 | -0.53 | 0.14 | 0.96 | 0.96 |

From the table above, it can be observed that there is no significant difference (< 0.3 dB) between the absolute L1-SINR accuracy of M = 3 and M = 5 samples as compared with the difference between M = 1 and M = 3. The difference for M = 1 and M = 3 is approximately 1 dB. Therefore, it is acceptable to set M = 3 if single shot measurement is not applicable for CMR-only, SSB+NZP-IMR, SSB+ZP-IMR, CSI-RS+NZP-IMR and CSI-RS+ZP-IMR.

M = 3 if single shot measurement is not applicable for CMR-only, SSB+NZP-IMR, SSB+ZP-IMR, CSI-RS+NZP-IMR and CSI-RS+ZP-IMR.

Measurement period for SSB-based CMR+IMR scenarios

From [1], the following is agreed, but P and N are FFS:

- By following L1-RSRP measurement requirement and restricting CMR and IMR’s 1-to-1 mapping, measurement period requirements for FR1 and FR2 shall be defined as:

Table x: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity |  |

Table y: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity |  |

    - FFS how to define the value of P, and whether or not a single P value for both CMR and IMR.

    - FFS the scaling factor N under different conditions of “repetition” field of IMR.

The periodicity of SSB and IMR is assumed to be the same. To compute P in the above tables, the same approach as for SSB based L1-RSRP reporting specified in TS 38.133, 9.5.4.1 for FR1 and FR2 is used. For the CMR+IMR scenario, the distribution of IMR symbols in the time domain may or may not be the same as SSB. Consequently, the value of P for IMR could differ from SSB. For computing the L1-SINR measurement reporting period, it is proposed to set P as max(PSSB, PIMR), where PSSB and PIMR are the P value that is separately computed for SSB and IMR, respectively.

For the SSB based CMR+IMR scenario, set P = max(PSSB, PIMR), where PSSB and PIMR are the P value that is separately computed for SSB and IMR, respectively.

For the SSB based CMR+IMR scenario, L1-SINR is computed using the signal power and interference measured on CMR and IMR, respectively, during SSB beam sweeping. Thus, N = 8 for SSB based L1-RSRP reporting in Rel-15 TS 38.133, 9.5.4.1 can be reused.

For the SSB based CMR+IMR scenario, N = 8 specified for Rel-15 SSB based L1-RSRP reporting can be reused.

Measurement period for CSI-RS-based CMR+IMR scenarios

From [1], the following is agreed, but P and N are FFS:

- By following L1-RSRP measurement requirement and restricting CMR and IMR’s 1-to-1 mapping, measurement period requirements for FR1 and FR2 shall be defined as:

Table x: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

Table y: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

    - FFS how to define the value of P, and whether or not a single P value for both CMR and IMR.

    - FFS the scaling factor N under different conditions of “repetition” field of CMR and IMR.

Following the same discussion as for Measurement period for SSB-based CMR+IMR scenario, it is proposed to set P as max(PCSI-RS, PIMR), where PCSI-RS and PIMR are the P value that is separately computed for CSI-RS and IMR, respectively.

: For the CSI-RS based CMR+IMR scenario, set P = max(PCSI-RS, PIMR), where PCSI-RS and PIMR are the P value that is separately computed for CSI-RS and IMR, respectively.

L1-SINR is derived from measurements on CSI-RS resources allocated for CMR and IMR. It is assumed “repetition = on” and the same pattern for both CMR and IMR. Similar to the SSB based CMR+IMR scenario, N can be reused from Rel-15 CSI-RS based L1-RSRP reporting.

For the CSI-RS based CMR+IMR scenario, N specified for Rel-15 CSI-RS based L1-RSRP reporting can be reused.

Conclusion

The document has presented the simulation results of L1-SINR measurement accuracy and discussed L1-SINR reporting period. Based on our discussions and analysis, the following proposals can be made:

Number of samples for L1-SINR measurement:

- M = 3 if single shot measurement is not applicable for CMR-only, SSB+NZP-IMR, SSB+ZP-IMR, CSI-RS+NZP-IMR and CSI-RS+ZP-IMR.

Measurement period for SSB-based CMR+IMR scenarios:

For the SSB based CMR+IMR scenario, set P = max(PSSB, PIMR), where PSSB and PIMR are the P value that is separately computed for SSB and IMR, respectively.

For the SSB based CMR+IMR scenario, N = 8 specified for SSB based L1-RSRP reporting can be reused.

Measurement period for CSI-RS-based CMR+IMR scenarios:

For the CSI-RS based CMR+IMR scenario, set P = max(PCSI-RS, PIMR), where PCSI-RS and PIMR are the P value that is separately computed for CSI-RS and IMR, respectively.

For the CSI-RS based CMR+IMR scenario, N specified for CSI-RS based L1-RSRP reporting can be reused.

References

[1]   R4-2002182, WF on NR eMIMO RRM requirements, Samsung

[2]    R4-19158501, Way forward on NR eMIMO RRM, Samsung

[3]