3GPP TSG-RAN4  #94-e-Bis															     	R4-2004472

20th – 30th April 2020

Source: 	Qualcomm Incorporated

Title: 	REFSENS for NR V2X

Agenda item:	6.4.4.1

Document for:	Approval

# Introduction

In this paper we propose a value for the diversity gain for NR V2X to be used in the formula agreed to in the RAN#94-e[1]

# Discussion

In the last meeting it was agreed that the REFSENS for NR V2X should be as given below:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB/NRB) +( NFV2X+ IM) - Diversity gain

Where

-	kTB: Thermal noise level in units of dBm for a given bandwidth.

-	NF: Noise figure. A value of 13 dB is used for NR V2X requirements. 9~10dB NF will be considered for licensed bands.

-	IM: 2.5 dB is assumed.

-	Target SNR: [TBD] dB

- 	Diversity gain. A value of 2.5dB is suggested

Proposal 1: Use a value of 2.5dB for diversity gain

Since the allowed  LCRB can be [10, 15, 20, 25, 30, 40, 45, 50, 60, 70, 75, 80, 90, 100, 105, 110, 120, 130, 135, 140, 150, 160, 165, 170, 175, 180, 190, 195,200, 210] in NR V2X the RB allocation is limited by LCRB.  The value of LCRB that is used in the REFSENS calculation is the one that is closest to but less than NRB.

The REFSENSE for NR V2X for n38 and n47 can be defined at shown in table 1 where a NF=9dB is used for n38.

| NR Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 10MHz (dBm) | 20MHz (dBm) | 30MHz (dBm) | 40MHz (dBm) | Duplex Mode |
| n38 | 15 | [-96.2] | [-93.0] | [-91.2] | [-90.1] | TDD |
|  | 30 | [-96.8] | [-93.1] | [-91.4] | [-90.0] |  |
|  | 60 | [-96.4] | [-93.8] | [-92.3] | [-90.1] |  |
| n47 | 15 | [-92.2] | [-89.0] | [-87.2] | [-86.1] | TDD |
|  | 30 | [-92.8] | [-89.1] | [-87.4] | [-86.0] |  |
|  | 60 | [-92.4] | [-89.8] | [-88.3] | [-86.1] |  |

Table 1 – V2X REFSENSE for n47 and n38

Table 2 gives the LCRB values for the REFSENS calculations

| NR Operating band / SCS / Channel bandwidth / LCRB /Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 10MHz (dBm) | 20MHz (dBm) | 30MHz (dBm) | 40MHz (dBm) | Duplex Mode |
| n38 | 15 | 50 | 105 | 160 | 210 | TDD |
|  | 30 | 20 | 50 | 75 | 105 |  |
|  | 60 | 10 | 20 | 30 | 50 |  |
| n47 | 15 | 50 | 105 | 160 | 210 | TDD |
|  | 30 | 20 | 50 | 75 | 105 |  |
|  | 60 | 10 | 20 | 30 | 50 |  |

Table 2 - LCRB values for REFSENS calculations

The REFSENS numbers in table1 must be verified once simulations are completed for determining the V2X SNR.

Proposal 2: Simulations should be conducted to verify the V2X SNR.

To derive the target SNR, RAN4 has to first agree on the reference measurement channel to be considered in the simulation. The reference measurement channel in LTE V2X is shown below:

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  |  |  |  | 48 |  | 96 |
| Subcarriers per resource block |  |  |  |  | 12 |  | 12 |
| Packets per period |  |  |  |  | 1 |  | 1 |
| Modulation |  |  |  |  | QPSK |  | QPSK |
| Target Coding Rate |  |  |  |  | 1/3 |  | 1/3 |
| Transport Block Size |  |  |  |  | 3496 |  | 6968 |
| Transport block CRC | Bits |  |  |  | 24 |  | 24 |
| Number of Code Blocks per Sub-Frame |  |  |  |  | 1 |  | 2 |
| Maximum number of HARQ transmissions |  |  |  |  | 1 |  | 1 |
| Binary Channel Bits per subframe | Bits |  |  |  | 11520 |  | 23040 |
| Max. Throughput averaged over 1 period of 100ms | kbps |  |  |  | 34.96 |  | 69.68 |
| UE Category |  |  |  |  | ≥ 1 |  | ≥ 1 |
| Note 1: 2RBs allocated to SA transmission and 4 symbols allocated to RS. Note 2: Throughput (in kbps) will depend on SA period configuration. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

We suggest to use LTE reference measurement channel modulation order and code rate, i.e. Modulation=QPSK and code rate=1/3, as a starting point for simulation assumption discussion. We also suggest to use LCRB in table 2 and consider 30kHz SCS for the simulation to derive the target SNR.

Proposal 3: For target SNR simulations assumptions, SCS=30kHz (and corresponding LCRB), modulation=QPSK and code rate of 1/3 is proposed as a starting point of discussion.

# Conclusion

In this paper the REFSENS values for n38 an n47 are calculated and the following proposals are made.

Proposal 1: Use a value of 2.5dB for diversity gain

Proposal 2: Simulations should be conducted to verify the V2X SNR.

Proposal 3: For target SNR simulations assumptions, SCS=30kHz (and corresponding LCRB), modulation=QPSK and code rate of 1/3 is proposed as a starting point of discussion.

# Reference

- R4 – 2002883, “Email discussion summary for RAN4#94e_#14_5G_V2X_NRSL_UE_RX”, RAN4#94-e, Feb 2020.