3GPP TSG-RAN4  #94-e-Bis 																	R4-20004473

20th – 30th Apr 2020

Source: 	Qualcomm Incorporated

Title: 	Pi/2 BPSK DMRS

Agenda item:	6.11.1.1

Document for:	Approval

# Introduction

In RAN#93 a WF agreement was initiated and later agreed to on the RAN4 reflector [1] to perform MPR simulations on waveforms having Pi/2 BPSK data and Pi/2 BPSK DMRS data symbols for both FR1 and FR2. This paper discusses issues related to the Pi/2 BPSK DMRS feature and the results presented in [2].

# Discussion

In [2] the MPR results for Pi/2 BPSK simulations were presented. It is felt that the MPR relaxations in [2] are achievable using spectrum shaping guidelines in section 6.4.2 of TS38.101. This filtering is transparent to the network and the selection of the filtering coefficients is left to the user’s discretion. A UE vendor would ideally design the filtering to optimize their device performance. A given design may have a set of filter coefficients that are unique to it.   In our opinion a given design has to meet performance with only one set of coefficients and does not need to meet the same performance with all possible filter coefficient or with no filtering.

Proposal 1: A UE is not required to meet pi/2 BPSK performance requirements with all possible filter coefficient sets.

As Pi/2 BPSK DMRS is an optional feature which tries to maximize the UL power it is felt that it should be treated separately in the specification tables as it would have different requirements from the ZC DMRS which is already captured in the RF specifications. We welcome further optimization of MPR for ZC based DMRS; this evaluation can proceed in parallel under the TEI WI.

Proposal 2: Pi/2 BPSK DMRS can be treated as a separate specification in the MPR tables

Companies are further encouraged to submit simulations on this modulation scheme in order to reach a consensus on the possible MPR numbers.

# Conclusion

In this contribution issues related to Pi/2 BPSK were discussed and the following proposals are made.

Proposal 1: A UE is not required to meet pi/2 BPSK performance requirements with all possible filter coefficient sets.

Proposal 2: Pi/2 BPSK DMRS can be treated as a separate specification in the MPR tables

# Reference

- R4-1916209, “WF on MPR Assumptions for PI/2 DMRS Simulations for FR1 and FR2”, RAN4#93, Reno, USA, November 2019

- R4-20000470, “Pi/2 BPSK DMRS Investigation”, RAN4#94-e