3GPP TSG-WG RAN4 Meeting #94bis-e	R4-2004508

# Online, 20th – 24th April, 2020

Source: 		Nokia, Nokia, Shanghai Bell, BT plc

Title: 			TP to TR 37.716-21-11: CA_3_n7-n28

Agenda Item: 		8.7.2 [DC_R16_xBLTE_2BNR_yDL2UL-Core]

Document for:		Approval

# Introduction

This TP introduces the following EN-DC configurations for x bands LTE inter-band CA (2DL/1UL) and 2 NR band (2DL/1UL).

- DL_3A_n7A-n28A_UL_3A_n7A

- DL_3A_n7A-n28A_UL_3A_n28A

- DL_3C_n7A-n28A_UL_3A_n7A

- DL_3C_n7A-n28A_UL_3A_n28A

- DL_3C_n7A-n28A_UL_3C_n7A

- DL_3C_n7A-n28A_UL_3C_n28A

## TP to TR 37.716-21-21

## <Start of Changes>

## 5.3	summary of self-interference analysis

Table 5.3-1 summarizes the EN- DC band combinations with self-interference problems for 3DL/2UL EN-DC operation.

Table 5.3-1: Summary of Self-interference analysis for LTE 1 band & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration |  | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1_n1-n78 | DC_1A_n78A |  |  | 4th IMD into n1 |  | - The 4th IMD already covered in 2DL/2UL DC_1A_n78A |
| DC_1_n3-n78 | DC_1A_n3A |  | 2nd harmonic from n3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems already covered in DC_n3A-n78A. - 28.4 dB by 2nd IMD in rel-15 TR - 11.2 dB by 4th IMD in Rel-15 TR. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_1A_n78A |  | - | 2nd IMD into n3 | - | - 27.9 dB by 2nd IMD in rel-15 TR. |
| DC_1_n7-n78 | DC_1A_n7A |  | - | 4th & 5th IMDs into n78 | - | - 10.1 dB by 4th IMD problem. - 3.8 dB by 5th IMD problem That were already covered in DC_7_n1-n78 in rel-15 TR |
|  | DC_1A_n78A, DC_1A_n78(2A) |  | - | 4th IMD into B7 | - | -9.1dB by 4th IMD problem that was already covered in DC_1-7_n78 in rel-15 TR |
| DC_1_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I |  | 2nd harmonic from B1 into n78 | - | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
| DC_3_n1-n77 | DC_3A_n1A |  | 2nd harmonic from B3 into n77 | 2nd & 4th IMDs into n77 | Yes | - Harmonic problems will be covered in DC_3A_n77A. - 2nd & 4th IMDs problems was same as DC_3A_n1A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n77A |  | - | 2nd & 5th IMD into n1 | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. |
| DC_3_n1-n78 | DC_3A_n1A |  | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n78A |  | - | 5th IMD into n1 | - | - 3.5 dB by 5th IMD in rel-15 TR |
| DC_3_n3-n77 | DC_3A_n3A |  | 2nd harmonic into n77 | - | - | - This harmonic impact already covered in 2DL/2UL DC_3A-n77A |
|  | DC_3A_n77A |  | 2nd harmonic into n77 | - | - | - This harmonic impact already covered in 2DL/2UL DC_3A-n77A In rel-15 |
| DC_3_n3-n78 | DC_3A_n3A |  | 2nd harmonic into n78 | 2nd , 3rd ,4th & 5th IMDs into n3 | - | - This harmonic & IMD problems are already covered in 2DL/2UL DC_3A-n78A |
|  | DC_3A_n78A |  | 2nd harmonic into n78 | - | - | - This harmonic impact already covered in 2DL/2UL DC_3A-n78A In rel-15. |
| DC_3_n7-n78 | DC_3A_n7A |  | 2nd harmonic from 3 into n78 | 3rd IMD into n78 | - | - Harmonic problem already solved in DC_3A_n78A - 16.1dB by 3rd IMD problem that already covered in DC_7A_n3A-n78A in rel-15 TR. |
|  | DC_3A_n78A, DC_3A_n78(2A) |  | - | - | - | No issues |
| DC_3_n20-n78 | DC_3A_n20A |  | 2nd & 4th harmonics into n78 | 3rd & 5th IMDs into n78 | - | These harmonic issues are already covered in DC_3A_n78A and DC_20A_n78A - 16.1 dB by 3rd IMD in rel-15 TR - 4.5 dB by 5th IMD in rel-15 TR |
|  | DC_3A_n78A |  | - | - | - | No issues |
| DC_3_n28-n78 | DC_3A_n28A |  | 2nd harmonic into n78 5th harmonic into n78 | 5th IMD into n78 | - | - Harmonic problem already discussed and will be solved in DC_3A_n78A or DC_28A_n78A - 4.5dB by 5th IMD problem in rel-15 TR |
|  | DC_3A_n78A |  | 2nd harmonic into n78 | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 | - | - 2nd harmonic problem already discussed and will be solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already discussed and will be solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band |
| DC_3_n78-n257 DC_3-3_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I |  | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
| DC_5_n78-n257 | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
| DC_7_n1-n3 | DC_7A_n1A |  |  |  | Yes | No issues |
|  | DC_7A_n3A |  |  |  | - | No issues |
| DC_7_n1-n78 | DC_7A_n1A, DC_7C_n1A |  |  | 4th & 5th IMDs into n78 | - | - 10.1 dB by 4th IMD in rel-15 TR. - 3.8 dB by 5th IMD in rel-15 TR. |
|  | DC_7A_n78A, DC_7C_n78A |  |  | 4th IMD into n1 | - | - 9.0 dB by 4th IMD in rel-15 TR. |
| DC_7_n3-n78 | DC_7A_n3A |  | 2nd harmonic from n3 into n78 | 3rd IMD into n78 | - | - Harmonic problem already solved in CA_n3A-n78A - 16.1 dB by 3rd IMD in rel-15 TR |
|  | DC_7A_n78A |  | - | 3rd & 4th IMDs into n3 | - | - 15.6 dB by 3rd IMD in rel-15 TR - 9.2 dB by 4th IMD in rel-15 TR. |
| DC_7_n28-n78 | DC_7A_n28A |  | 5th harmonic into n78 | 2nd,4th IMDs into n78 | - | - 5th harmonic problem will be solved in DC_28A_n78A - 29.7 dB by 2nd IMD in rel-15 TR - 10.3 dB by 4th IMD problem in rel-15 TR |
|  | DC_7A_n78A |  | - | 2nd & 5th IMDs into B28 4th IMD into n78 | - | - 28.8 dB by 2nd IMD in rel-15 TR - 4.0 dB by 5th IMD problems in rel-15 TR - 4th IMD problem is no issue for TDD band |
| DC_7-7_n78-n257 | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_7A_n257A |  | - | - | - | No issues |
| DC_7_n78-n257 | DC_7A_n78A |  | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_7A_n257A |  | - | - | - | No issues |
| DC_8_n8-n78 | DC_8A_n78A |  | - | 4th IMD into n8 | - | The 4th IMD issue already covered in DC_8A-n78A |
| DC_8_n78-n257 | DC_8A_n78A |  | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n257A |  | 4th harmonic from B8 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
| DC_13_n5-n260 | DC_13A_n5A |  | - | - | - | No issue |
|  | DC_13A_n260A |  | - | - | - | No issue |
| DC_20_n1-n3 | DC_20A_n1A |  |  |  | - | No issues |
|  | DC_20A_n3A |  |  |  | - | No issues |
| DC_20_n1-n78 | DC_20A_n1A |  | 4th harmonic from B20 into n78 | 3rd IMD into n78 | - | - Harmonic problem already solved in DC_20A_n78A - 16.0 dB by 3rd IMD in rel-15 TR |
|  | DC_20A_n78A |  | - | 3rd IMD into n1 | - | - 15.3 dB by 3rd IMD in rel-15 TR |
| DC_20_n3-n78 | DC_20A_n3A |  | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 | 3rd & 5th IMDs into n78 | - | - 16.1 dB by 3rd IMD in rel-15 TR - 4.5 dB by 5th IMD in rel-15 TR |
|  | DC_20A_n78A |  | - | 3rd IMD into n3 | - | - 15.7 dB by 3rd IMD in rel-15 TR |
| DC_20_n20-n78 | DC_20A_n78A |  | - | 4th IMD into n20 | - | This IMD impact already covered in 2DL/2UL DC_20A-n78A |
| DC_28_n8-n78 | DC_28A_n8A |  | 4th harmonic into n78 5th harmonic into n78 | 4th & 5th IMDs into n78 | - | - 4th harmonic problem will be solved in DC_8A-n78A - 5th harmonic problem will be solved in DC_28A-n78A - 9.1 dB by 4th IMD in rel-15 TR - 3.7 dB by 5th IMD in rel-15 TR |
|  | DC_28A_n78A |  | 5th harmonic into n78 | 5th IMD into B28 5th IMD into n8 | - | - 5th harmonic problem will be solved in DC_28A-n78A - 5th IMD problem already discussed and will be solved in DC_28A-n78A - 4.3 dB by 5th IMD in rel-15 TR |
| DC_28_n8-n258 | DC_28A_n8A |  | 33th harmonics into n258 27th harmonics into n258 | - | - | - No harmonic problems by 27th & 33th |
|  | DC_28A_n258A |  | - | - | - | - No issue |
| DC_28_n28-n78 | DC_28A_n78A |  | - | 5th IMD into n28 | - | This IMD impact already covered in 2DL/2UL DC_28A-n78A |
| DC_66_n5-n260 | DC_66A_n5A |  | 22th harmonics into n260 | - | - | Up to 7th harmonic order will be not any impact to n260. |
|  | DC_66A_n260A |  | - | - | - | No issues |
| DC_66_n71-n260 DC_66_n71-n260(2A) |  | DC_66A_n71A | 22th harmonics into n260 | - | - | Up to 7th harmonic order will be not any impact to n257. |
|  |  | DC_66A_n260A | - | - | - | No issues |
| DC_66_n71-n261 DC_66_n71-n261(2A) |  | DC_66A_n71A | 16th harmonics into n261 | - | - | Up to 7th harmonic order will be not any impact to n261. |
|  |  | DC_66A_n261A | - | - | - | No issues |
| DC_1_n77-n78 |  | DC_1A_n77A |  | 4th IMD & 5th IMD | - | These MSD issues are same in DC_1A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_1A_n77A |
|  |  | DC_1A_n78A | 2nd harmonic | 4th IMD & 5th IMD |  | 2nd harmonic issue already specified in TS38.101-1 for DC_1A_n77A These MSD issues are same in DC_1A_n78A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_1A_n78A |
| DC_3_n77-n78 |  | DC_3A_n77A | 2nd harmonic | 4th IMD & 5th IMD |  | 2nd harmonic issue already specified in TS38.101-1 for DC_3A_n78A These MSD issues are same in DC_3A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_3A_n77A |
|  |  | DC_3A_n78A | 2nd harmonic | 4th IMD & 5th IMD |  | 2nd harmonic issue already specified in TS38.101-1 for DC_3A_n77A These MSD issues are same in DC_3A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_3A_n78A |
| DC_19_n77-n78 |  | DC_19A_n77A | 4th harmonic | 2nd IMD |  | 4th harmonic & 2nd IMD will be impact to n78 band. However, consider specific band for n77 in Japan, there was no IMD problem |
|  |  | DC_19A_n78A | 4th & 5th harmonic | 2nd IMD |  | 5th IMD already specified for DC_19A_n77A in TS38.101-3 Consider specific band for n78 in Japan, there was no IMD problem |
| DC_21_n77-n78 |  | DC_21A_n77A |  | 4th IMD & 5th IMD | - | These MSD issues are same in DC_21A_n77A. However, RAN4 consider the specific band for n77 in Japan, there was no IMD problem |
|  |  | DC_21A_n78A |  | 4th IMD |  | These MSD issue is same in DC_21A_n78A. However, RAN4 consider the specific band for n78 in Japan, there was no IMD problem |
| DC_1_n40-n78 |  | DC_1A_n40A |  | 4th IMD & 5th IMD | - | - 9.8dB by 4th IMD - 4.2dB by 5th IMD |
|  |  | DC_1A_n78A |  | 4th IMD | - | - 10.6dB by 4th IMD |
| DC_3_n1-n79 |  | DC_3A_n1A |  | 5th IMD | Yes | - 4.7dB by 5th IMD |
|  |  | DC_3A_n79A |  | 5th IMD | - | - 3.6dB by 5th IMD |
| DC_3_n40-n78 |  | DC_3A_n40A | 2nd harmonic | 5th IMD | - | Harmonic issue already covered in DC_3A_n78A - 4.8dB by 5th IMD |
|  |  | DC_3A_n78A |  | 5th IMD | - | - 4.4dB by 5th IMD |
| DC_66_n25-n41 |  | DC_66A_n25A | - | - | - | No issue |
|  |  | DC_66A_n41A |  | 4th IMD | - | - 11.0dB by 4th IMD |
| DC_1_n77-n257 |  | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  |  | DC_1A_n257A | 2nd harmonic from B1 into n77 | - | - | - Harmonic problem already covered in DC_1A_n77A |
| DC_1_n77-n258 |  | DC_1A_n77A | 6th, 7th & 8th Harmonics from n77 into n258 | - | - | - No harmonic problems by 6th, 7th & 8th order between FR1 and FR2 |
|  |  | DC_1A_n258A | 2nd harmonic from B1 into n77 | - | - | - Harmonic problem already covered in DC_1A_n77A |
| DC_1_n78-n258 |  | DC_1A_n78A | 7th & 8th Harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  |  | DC_1A_n258A | - | - | - | No issue |
| DC_1_n79-n258 |  | DC_1A_n79A | 5th & 6th Harmonics from n79 into n258 | - | - | - Harmonic problem will be solved in DC_n79A-n258A (5th order) |
|  |  | DC_1A_n258A | - | - | - | No issue |
| DC_3_n1-n257 |  | DC_3A_n1A | - | - | Yes | No issue |
|  |  | DC_3A_n257A | - | - | - | No issue |
| DC_3_n77-n258 |  | DC_3A_n77A | 6th, 7th & 8th harmonics from n77 into n258 | - | - | - No harmonic problems by 6th ~ 8th order between FR1 and FR2 |
|  |  | DC_3A_n258A | 2nd harmonic from B3 into B77 | - | - | - Harmonic problem already covered in DC_3A_n77A |
| DC_3_n78-n258 |  | DC_3A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  |  | DC_3A_n258A | 2nd harmonic from B3 into B78 | - | - | - Harmonic problem already covered in DC_3A_n78A |
| DC_3_n79-n258 |  | DC_3A_n79A | 5th & 6th Harmonics from n79 into n258 | - | - | - Harmonic problem will be solved in DC_n79A-n258A (5th order) |
|  |  | DC_3A_n258A | - | - | - | No issue |
| DC_8_n77-n257 |  | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  |  | DC_8A_n257A | 4th harmonic from B8 into n77 | - | - | - Harmonic problem already covered in DC_8A_n77A |
| DC_1_n5-n78 |  | DC_1A_n5A | - | 3rd IMD | - | - 18.1dB by 3rd IMD |
|  |  | DC_1A_n78A | - | 5th IMD | - | Already specified in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_3_n5-n78 |  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 3rd & 5th IMDs | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 16.1dB by 3rd IMD - FFS |
|  |  | DC_3A_n78A, DC_3C_n78A | - | - | - | No issue |
| DC_7_n5-n78 |  | DC_7A_n5A, DC_7C_n5A | - | 2nd & 4th IMDs | - | - 29.7dB by 3rd IMD - FFS |
|  |  | DC_7A_n78A, DC_7C_n78A | - | 2nd & 5th IMDs | - | Already specified these IMDs problems for DC_5-7_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_28_n5-n78 |  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 |
|  |  | DC_28A_n78A | - | 5th IMD | - | - 3.8dB by 5th IMD |
| DC_39_n41-n79 |  | DC_39A_n41A | 2nd harmonic from n41 into n79 | 2nd I& 5th MDs | - | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - 29.8dB by 2nd IMD - 4.4dB by 5th IMD |
|  |  | DC_39A_n79A | - | 2nd & 5th IMDs | - | - 30.2dB by 2nd IMD - 4.3dB by 5th IMD |
| DC_40_n41-n79 |  | DC_40A_n41A | 2nd harmonic from n40 into n79 2nd harmonic from n41 into n79 | 2nd & 4th IMDs | - | - FFS by 2nd harmonic from B40 - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - 30.5dB by 2nd IMD - No impact to china band since over 4800MHz is considered in NR |
|  |  | DC_40A_n79A | - | 2nd , 4th & 5th IMDs | - | Band 40 and n41 should be operate the TDD synchronous operation No MSD into n41 |
| DC_2_n7-n66 |  | DC_2A_n7A | - | - | - | No issue |
|  |  | DC_2A_n66A | - | - | - | No issue |
| DC_12_n7-n66 |  | DC_12A_n7A | 3rd harmonic from B12 into n66 | - | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 |
|  |  | DC_12A_n66A | - | - | - | No issue |
| DC_5_n7-n78 |  | DC_5A_n7A | - | 2nd & 4th IMDs | - | - These MSD issue follow DC_7_n5-n78 |
|  |  | DC_5A_n78A | - | 2nd IMD | Yes | - 2nd IMD problems was covered in DC_5A-7A_n78A. - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_7_n7-n78 |  | DC_7A_n7A | - | - | - | No issue |
|  |  | DC_7A_n78A | - | - | Yes | - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_12_n7-n78 |  | DC_12A_n7A | 5th harmonic from B12 into n78 | 4th IMD |  | - 5th harmonic issue will be covered in DC_12A_n78A - 8.4dB by 4th IMD |
|  |  | DC_12A_n78A | - | 2nd IMD | Yes | - 28.7dB by 2nd IMD - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_66_n7-n78 |  | DC_66A_n7A | 2nd harmonic from B66 into n78 | 3rd IMD | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 -reuse MSD of DC_3A_n7-n78 for 3rd IMD problem |
|  |  | DC_66A_n78A | - | - | Yes | - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_2_n66-n78 |  | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 | 2nd & 4th IMDs | - | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 29.4dB by 2nd IMD - 8.9 dB by 4th IMD |
|  |  | DC_2A_n78A | - | 4th IMD | - | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_12_n66-n78 |  | DC_12A_n66A | 2nd harmonic from n66 inton78 5th harmonic from B12 into n78 | 5th IMD |  | - 2nd harmonic is same as DC_66A_n78A and 5th harmonic will be covered in DC_12A_n78A - 4.1dB by 5th IMD |
|  |  | DC_12A_n78A | 3rd harmonic from B12 into n66 | 3rd IMD |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 16.5dB by 3rd IMD |
| DC_3-3_n1_n78 |  | DC_3A_n1A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs | - | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  |  | DC_3A_n78A | - | 5th IMD | Yes | - 3.5 dB by 5th IMD in rel-15 TR - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
| DC_7-7_n1_n78 |  | DC_7A_n1A |  | 4th & 5th IMDs | Yes | - 10.1 dB by 4th IMD in rel-15 TR. - 3.8 dB by 5th IMD in rel-15 TR. - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  |  | DC_7A_n78A |  | 4th IMD | - | - 9.0 dB by 4th IMD in rel-15 TR. |
| DC_2_n41-n260 |  | DC_2A_n41A | High order harmonic into n260 | - | - | - No harmonic problem |
| DC_2_n41-n261 |  | DC_2A_n41A | High order harmonic into n261 | - | - | - No harmonic problem |
| DC_66_n41_n260 |  | DC_66A_n41A | High order harmonic into n260 | - | - | - No harmonic problem |
| DC_66_n41_n261 |  | DC_66A_n41A | High order harmonic into n261 | - | - | - No harmonic problem |
| DC_2_n5-n258 |  | DC_2A_n5A | High order harmonic into n258 | - | - | - No harmonic problem |
|  |  | DC_2A_n258A | - | - | - | No issue |
| DC_2_n5-n260 |  | DC_2A_n5A | High order harmonic into n260 | - | - | - No harmonic problem |
|  |  | DC_2A_n260A | - | - | - | No issue |
| DC_2_n5-n261 |  | DC_2A_n5A | High order harmonic into n261 | - | - | - No harmonic problem |
|  |  | DC_2A_n261A | - | - | - | No issue |
| DC_2_n71-n258 |  | DC_2A_n71A | High order harmonic into n258 | - | - | - No harmonic problem |
|  |  | DC_2A_n258A | - | - | - | No issue |
| DC_2_n71-n260 |  | DC_2A_n71A | High order harmonic into n260 | - | - | - No harmonic problem |
|  |  | DC_2A_n260A | - | - | - | No issue |
| DC_2_n71-n261 |  | DC_2A_n71A | High order harmonic into n261 | - | - | - No harmonic problem |
|  |  | DC_2A_n261A | - | - | - | No issue |
| DC_66_n5-n258 |  | DC_66A_n5A | High order harmonic into n258 | - | - | - No harmonic problem |
|  |  | DC_66A_n258A | - | - | - | No issue |
| DC_66_n5-n261 |  | DC_66A_n5A | High order harmonic into n261 | - | - | - No harmonic problem |
|  |  | DC_66A_n261A | - | - | - | No issue |
| DC_66_n71-n258 |  | DC_66A_n71A | High order harmonic into n258 | - | - | - No harmonic problem |
|  |  | DC_66A_n258A | - | - | - | No issue |
| DC_3_n77-n257 |  | DC_3A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | - No harmonic problems by 6th ~ 8th order between FR1 and FR2 |
|  |  | DC_3A_n257A, DC_3A_n257D, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into B77 | - | - | - Harmonic problem already covered in DC_3A_n77A |
| DC_7_n66-n78 |  | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD |  | - Harmonic problem already covered in DC_66A_n78A - 16.0dB by 3rd IMD |
|  |  | DC_7A_n78A | - | 4th IMD | Yes | - 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_66_n66-n78 |  | DC_66A_n66A | 2nd harmonic from B66 into n78 | 2nd & 4th IMDs |  | - Harmonic problem already covered in DC_66A_n78A - FFS - FFS |
|  |  | DC_66A_n78A | - | 5th IMD |  | - Follow MSD result for DC_66A_n78A |
| DC_20_n1-n7 |  | DC_20A_n1A | - | - | - | No issue |
|  |  | DC_20A_n7A | - | - | - | No issue |
| DC_3_n1-n7 |  | DC_3A_n1A, DC_3C_n1A | - | - | - | No issue |
|  |  | DC_3A_n7A, DC_3C_n7A | - | - | - | No issue |
| DC_20_n1-n28 |  | DC_20A_n1A | - | - | - | No issue |
|  |  | DC_20A_n28A | 3rd harmonic from n28 into n1 | 5th IMD | - | - Harmonic problem already covered in DC_1A_n28A - No need 5th IMD problem due to restrlicted n28 freq. range in Europe |
| DC_3_n1-n28 |  | DC_3A_n1A, DC_3C_n1A | - | - | - | No issue |
|  |  | DC_3A_n28A, DC_3C_n28A | 3rd harmonic from n28 into n1 | 4th IMD | - | - Harmonic problem already covered in DC_1A_n28A - 4th IMD problem same as DC_1A-3A_28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_20_n7-n28 |  | DC_20A_n7A | - | 3rd & 5th IMDs | - | - 15.7dB by 3rd IMD - 2.8dB by 5th IMD |
|  |  | DC_20A_n28A | - | 5th IMD | - | - 5th IMD problem same as DC_7A-20A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_20_n7-n78 |  | DC_20A_n7A | 4th harmonic from B20 into n78 | 2nd & 4th IMDs | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 28.8dB by 2nd IMD - 8.5dB by 4th IMD |
|  |  | DC_20A_n78A | - | 2nd IMD | Yes | - 2nd IMD problem same as DC_7A-20A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n7-n28 |  | DC_3A_n7A, DC_3C_n7A |  | 2nd IMD | - | - 20.0dB by 2nd IMD |
|  |  | DC_3A_n28A, DC_3C_n28A | - | 3rd IMD | - | - 3rd IMD problem same as DC_3A-7A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_2_n41-n71 |  | DC_2A_n41A | - | 2nd & 5th IMDs | - | - 28.7dB by 2nd IMD - 3.8dB by 5th IMD |
|  |  | DC_2A_n71A | 4th harmonic from n71 into n41 | 2nd IMD | - | The harmonic problem will be treated CA_n41A_n71A - 29.2dB by 2nd IMD |
| DC_66_n41-n71 |  | DC_66A_n41A | - | - | - | No issue |
|  |  | DC_66A_n71A | 4th harmonic from n71 into n41 | - | - | The harmonic problem will be treated CA_n41A_n71A |
| DC_18_n3-n78 |  | DC_18A_n3A | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 | 3rd & 5th IMDs | - | These harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 15.2dB by 3rd IMD - 4.0dB by 5th IMD |
|  |  | DC_18A_n78A | - | 3rd IMD |  | - Only one Japan operation use B18 and the n78 freq. range is allocated 3600 to 3800 MHz. So No IMD problem |
| DC_28_n3-n78 |  | DC_28A_n3A | 2nd harmonic from n3 into n78 5th harmonic from B28 into n78 | 5th IMD | - | - 5th IMD problem same as DC_3A_n28A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  |  | DC_28A_n78A | 5th harmonic from B28 into n78 | 3rd IMD | - | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 3rd IMD already covered in DC_3A-28A_n78A |
| DC_8_n1-n78 |  | DC_8A_n1A | 4th harmonic from B8 into n78 | 3rd IMD | - | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 14.9dB by 3rd IMD |
|  |  | DC_8A_n78A | - | - | - | No issue |
| DC_3_n40-n41 |  | DC_3A_n40A | 3rd harmonic from B3 into n41 | - | - | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  |  | DC_3A_n41A | - | - | Yes | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n40-n79 |  | DC_3A_n40A | 2nd harmonic from n40 into n79 | 5th IMD | - | - The 2nd harmonic problem will be studied in CA_n40-n79. - 4.7dB by 5th IMD |
|  |  | DC_3A_n79A | - | 5th IMD | - | - 3.2dB by 5th IMD |
| DC_3_n41-n79 |  | DC_3A_n41A | 2nd harmonic from n41 into n79 | 2nd & 5th IMDs | Yes | - The 2nd harmonic problem will be studied in CA_n41-n79. - 30.8dB by 2nd IMD - 5.2dB by 5th IMD - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  |  | DC_3A_n79A | - | 2nd & 5th IMDs | - | - Already MSD analysis are covered in DC_3A-41A_n79A |
| DC_8_n40-n41 |  | DC_8A_n40A | 3rd harmonic from B8 into n41 | - | - | - Harmonic problem will be studied in DC_8A_n41A. - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  |  | DC_8A_n41A | - | 5th IMD | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_8_n40-n79 |  | DC_8A_n40A | 2nd harmonic from n40 into n79 5th harmonic from B8 into n79 | 4th IMD | - | - These harmonic problem will be treated lower order DC or CA band combos. - 10.7dB by 4th IMD |
|  |  | DC_8A_n79A | - | 4th IMD | - | - 9.2dB by 4th IMD |
| DC_8_n41-n79 |  | DC_8A_n41A | 2nd harmonic from n41 into n79 5th harmonic from B8 into n79 | 3rd IMD | - | - These harmonic problem will be treated lower order DC or CA band combos. - 16.3dB by 3rd IMD |
|  |  | DC_8A_n79A | 3rd harmonic from B8 into n41 | 3rd IMD | - | - 15.5dB by 3rd IMD |
| DC_39_n40-n41 |  | DC_39A_n40A | - | 3rd IMD | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  |  | DC_39A_n41A | - | - | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_39_n40-n79 |  | DC_39A_n40A | 2nd harmonic from n40 into n79 | 4th IMD | - | - The harmonic problem will be treated CA_n40-n79 band combos. - 5.8 dB by 4th IMD |
|  |  | DC_39A_n79A | - | - | - | No issue |
| DC_41_n78-n257 |  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | No harmonic impact to n257 |
|  |  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
| DC_42_n78-n257 |  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_18_n78-n257 |  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | No harmonic impact to n257 |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
| DC_28_n78-n257 |  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | No harmonic impact to n257 |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
| DC_1_n3-n257 |  | DC_1A_n3A | High order harmonic into n257 | - | -- | No harmonic impact to n257 |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - | - | Yes | Cross isolation issue already covered in Table 7.3.1A-0bA in TS36.101 |
| DC_18_n3-n257 |  | DC_18A_n3A | High order harmonics into n257 | - | - | No harmonic problem to n257 |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
| DC_28_n3-n257 |  | DC_28A_n3A | High order harmonics into n257 | - | - | No harmonic problem to n257 |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
| DC_1_n3-n28 |  | DC_1A_n3A | - | - | - | No issue |
|  |  | DC_1A_n28A | - | 5th IMD | Yes | - The 5th IMD problem same as DC_1A-3A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation already treated in TS36.101 |
| DC_8_n3-n28 |  | DC_8A_n3A | - | 2nd & 5th IMDs | - | - 30.4dB by 2nd IMD - 2.6dB by 5th IMD |
|  |  | DC_8A_n28A | 2nd harmonic from B8 into n3 | - | - | The 2nd harmonic problem will be covered in DC_8A_n3A |
| DC_11_n77-n257 |  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  |  | DC_11A_n257A | - | - | - | No issue |
| DC_2_n12-n258 |  | DC_2A_n12A | High order harmonic into n258 | - | - | No harmonic problem to n258 |
|  |  | DC_2A_n258A | - | - | - | No issue |
| DC_2_n12-n260 |  | DC_2A_n12A | High order harmonic into n260 | - | - | No harmonic problem to n260 |
|  |  | DC_2A_n260A | - | - | - | No issue |
| DC_2_n12-n261 |  | DC_2A_n12A | High order harmonic into n261 | - | - | No harmonic problem to n261 |
|  |  | DC_2A_n261A | - | - | - | No issue |
| DC_66_n12-n258 |  | DC_66A_n12A | High order harmonic into n258 | - | - | No harmonic problem to n258 |
|  |  | DC_66A_n258A | - | - | - | No issue |
| DC_66_n12-n260 |  | DC_66A_n12A | High order harmonic into n260 | - | - | No harmonic problem to n260 |
|  |  | DC_66A_n260A | - | - | - | No issue |
| DC_66_n12-n261 |  | DC_66A_n12A | High order harmonic into n261 | - | - | No harmonic problem to n261 |
|  |  | DC_66A_n261A | - | - | - | No issue |
| DC_7-7_n66-n78 |  | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD | - | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - 15.1dB by 3rd IMD |
|  |  | DC_7A_n78A | - | 4th IMD | Yes | - The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |

## <Next Changes>

## 6.X	DC_3_n7-n28

### 6.X.1	Operating bands for DC

Table 6.X.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n7-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n7A-n28A | DC_3A_n7A DC_3A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 90 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_3C_n28A | 3 | 15 | See CA_3C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.X.3	Co-existence studies

The harmonics and intermodulation products analysis for 2UL bands 3 and 7 with 3DL bands 3, 7 and 28 is presented in clause 6.11.1.2 in TR 36.879-13.

The harmonics and intermodulation products analysis for 2UL bands 3 and 28 with 3DL bands 3, 7 and 28 is presented in clause 6.21.1.2 in TR 36.715-00-02.

Based on the above studies,

-	2nd order IMD generated by dual uplink of Band 3 + Band n7 may fall into own Rx of band n28

-	3rd order IMD generated by dual uplink of Band 3 + Band n28 may fall into own Rx of band n7

### 6.X.4	∆TIB and ∆RIB values

For DC_3_n7-n28, the TIB,c and RIB values can be based on E-UTRA CA_3-7-28 given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n7-n28 | 3 | 0.5 |
|  | n7 | 0.5 |
|  | n28 | 0.3 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n7-n28 | 3 | 0 |
|  | n7 | 0 |
|  | n28 | 0 |

### 6.X.5	MSD

Table 6.X.5-1 lists the MSD for DC_3A_n7A-n28A which is reused from E-UTRA 3DL/2UL CA_3A_7A-28A in TS 36.101. The MSD values were previously derived in clause 6.11.1.4 in TR 36.879-13 and clause 6.21.1.4 in TR 36.715-00-02.

Table 6.X.5-1: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| DC_3A_n7A-n28A | DC_3A_n7A | 3 | 1747 | 5 | 25 | 1842 | 5 | N/A | FDD | N/A |
|  |  | n7 | 2543 | 5 | 25 | 2663 | 5 | N/A |  | N/A |
|  |  | n28 | 741 | 5 | 25 | 796.0 | 5 | 20.0 |  | IMD2 |
|  | CA_3A-n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | N/A |
|  |  | n7 | 2562 | 5 | 25 | 2682 | 5 | 17.0 |  | IMD3 |
|  |  | n28 | 743 | 5 | 25 | 798 | 5 | N/A |  | N/A |

## <End of Changes>