3GPP TSG-RAN WG4 Meeting #94bis-e															  R4-2004558

Electronic Meeting, April 20th – April 30th 2020

Source:		Ericsson

Title:			Proposal for use of reverberation chamber for receiver immunity testing

Agenda item:		4.8.1.2

Document for:	Agreement

# Introduction

Testing of the receiver immunity is also possible when a reverberation chamber is utilized.

Currently there is a proposal for introducing the reverberation chamber as possible method for testing receiver immunity. The latest draft of EN 301 489-50 v2.3.1_003 (final draft for approval), clause 7.2.2, with reference to IEC/EN 61000-4-21 proposes the following approach:

| ## 9.2 Radio frequency ## electromagnetic field (80 MHz to 6 000 MHz) 9.2 2 Test method Other than low RF power equipment Low RF power equipment, domestic indoor cellular system with maximum transmitted RF output power less than or equal to [30 dBm (1W)] EIRP. Alternative test method: | Test level: For the frequency range 80 MHz to 690 MHz, test level shall be 3 V/m For the frequency range 690 MHz to 6 000 MHz test level shall be 10 V/m Test level: For the frequency range 80 MHz to 6 000 MHz, test level shall be 3 V/m Use of reverberation chamber test method according to EN 61000-4-21[29], clause 6.1 and Annex D is allowed. |
| --- | --- |

# Proposal

Proposal 1: Agree on the introduction of reverberation chamber as an alternative test method for receiver immunity across the BS EMC specifications, in a similar approach as in the EN standard mentioned above.