3GPP TSG-RAN WG4 Meeting #94bis-e															  R4-2004563

Electronic Meeting, April 20th – April 30th 2020

Source:		Ericsson

Title:			Proposal for EMC reduction of test configurations

Agenda item:		4.8.1.2

Document for:	Discussion

# Introduction

Current testing of EMC requirements as specified in TS 37.113 [1] (MSR BS) and TS 37.114 [2] (AAS BS) is increasingly complex and considers many test configurations, due to various possible RAT combinations. In R4-2004562 [3], we provide elements on the need for a reduction in the test configurations.

In this contribution we want to present an initial proposal on how to achieve the reduction of the test configurations.

# Discussion

In the contribution [3], we analyse the elements that might lead to a reduction in the test configurations for EMC. In order to provide more elements to the discussion, this contribution presents an initial proposal of how to reach this goal.

## According to TS 37.113 [2], a MSR BS Base station is “characterized by the ability of its receiver and transmitter to process two or more carriers in common active RF components simultaneously in a declared RF bandwidth, where at least one carrier is of a different RAT than the other carrier(s).” Based on this definition, one characteristic of MSR is the use of common active RF components to process two or more carriers simultaneously.

Observation 1: One characteristic of MSR is the use of common active RF component to process two or more carriers simultaneously.

Currently the conformance specification for MSR (37.141[4]) defines 19 possible CS, as presented in the figures below:

This amount of Capability sets require either to use long-duration testing times or high amount of resources to perform the EMC testing according to the number of RATs for MSR.

Based on the previous comments, in Figure 1, we are presenting a proposal if the radio supports GSM. If the simplification is applied, we will require to use CS 17, CS 18. However, it will be necessary to propose two new capability sets to cover the cases 1 (IoT-SA,WCDMA, NR), and Case 2 (GSM, WCDMA,NR).

Figure 1. Proposal when radio supports GSM

In Figure 2, we present the proposal for a radio not supporting GSM. Following the principles of our proposal, we can keep CS 17 and define a new CS (for WCDMA, IoT and NR).

Figure 2. Proposal when radio do not support GSM

Our proposal considers that using NB-IoT test results could cover GSM. LTE cover WCDMA and based on these possibilities, we can reduce EMC MSR test scope to maximum three RATs combination. IoT can replace GSM because IoT have similar bandwidth as GSM. In addition, both are narrow band signal, which is the key impact factor for filter to suppress the spurious emission.

Regarding LTE, the test for this technology can replace WCDMA considering the same reasons mentioned for IoT/GSM, with the only difference that LTE/WCDMA are wideband signals. With more testing experience and results an NR inclusion in the simplification might be considered in the future.

# Conclusion

In this contribution a proposal on EMC Test Configuration reduction is presented. Based on our observations, we propose:

Proposal 1: To agree in the reduction of the CS used for MSR considering the following aspects: NB-IoT test results could cover GSM and LTE ones can cover WCDMA.

# References

3GPP TS 37.113. “NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)”

3GPP TS 37.114. “Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)”

R4-2004562. On the need for EMC test reduction proposal. Ericsson.

3GPP TS 37.141. NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing.