3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2004613

Online, 20th April – 30th April 2020

Source:	Ericsson

Title:	TP for TR 37.716-31-11: Addition of EN-DC configurations containing bands 1-3-28_n40.

Agenda item:	8.5.2

Document for:	Approval

# Background

This contribution provides text proposal to TR 37.716-31-11 for addition of DC_1A-3A-28A_n40A as defined in [1].

# Text Proposal

##### ---Start of changes---

## 5.1.x	DC_1A-3A-28A_n40A

#### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-28_n40 | CA_1-3-28 | n40 | No |

#### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1:  Inter-band EN-DC configurations of 3 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_28A_n40A | CA_1A-3A-28A | n40A |

#### 5.1.x.3	∆TIB and ∆RIB values

For DC_1-3-28_n40 the TIB,c and RIB,c values are derived from CA_1-3-28-40 and are given in the tables below.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n40 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 28 | 0.6 |
|  | n40 | 0.5 |

Table 5.1.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n40 | 1 | 0 |
|  | 3 | 0 |
|  | 28 | 0.2 |
|  | n40 | 0 |

### 5.1.x.4	REFSENS requirements

No further MSD are needed to be specified.

##### ---End of changes---

# Reference

[1]	RP-200102, “Revised WID on Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, Ericsson