3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2004631

Online, 20th April – 30th April 2020

Source:	Ericsson, US Cellular

Title:	TP for TR 37.716-31-11: Addition of EN-DC configurations containing bands DC_2-12-48_n71.

Agenda item:	8.5.2

Document for:	Approval

# Background

This contribution provides text proposal to TR 37.716-31-11 for addition of DC_2A-12A-48A_n71A as defined in [1].

# Text Proposal

##### ---Start of changes---

## 5.1.x	DC_2A-12A-48A_n71A

#### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-12-48_n71 | CA_2-12-48 | n71 | No |

#### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1:  Inter-band EN-DC configurations of 3 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-12A-48A_n71A2 | DC_2A_n71A DC_12A_n71A1 DC_48A_n71A | CA_2A-12A-48A | n71A |
| NOTE 1: Only single switched UL is supported NOTE 2: This combination is applicable only for non-form-factor constrained device use cases (e.g. customer premise or vehicle-mounted devices) |  |  |  |

#### 5.1.x.3	∆TIB and ∆RIB values

For DC_2-12-48_n71 the TIB,c and RIB,c values are derived from DC_12_n71 and DC_2_n48 and are given in the tables below.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12-48_n71 | 2 | 0.6 |
|  | 12 | 0.5 |
|  | 48 | 0.8 |
|  | n71 | 0.5 |

Table 5.1.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12-48_n71 | 2 | 0.2 |
|  | 12 | 0 |
|  | 48 | 0.5 |
|  | n71 | 0 |

### 5.1.x.4	REFSENS requirements

No further MSD are needed to be specified.

##### ---End of changes---

# Reference

[1]	RP-192446, “Revised WID on Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, Ericsson